Carbon-13C Dioxide, Enriched Carbon-13C Dioxide SDS

Publisher: Neonest AB

Source: https://resds.com/document/carbon-13c-dioxide-enriched-carbon-13c-dioxide-sds-5b1834f8f5

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Carbon-13C dioxide,   13 CO 2

Molecular weight:   45.00

Chemical formula:   13 CO 2

Synonyms:   None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Compressed Gas   H280

Label elements:

Hazard pictograms:

Signal Words:   Warning.

Hazard Statements:

H280 Contains gas under pressure; may explode if heated.

Page 2

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 2/9

Precautionary Statements:

P410 + P403 Protect from sunlight. Store in a well-ventilated place.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Concentration

% by weight

Classification

EC1272/2008 EC - No.

Carbon-13C dioxide

1111-72-4

>99.9%   Press. Gas (Comp.)   H280

204-696-9

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

General Advice:   Consult a physician. Show this safety data sheet to the doctor in attendance. Move out of dangerous

area.

Inhalation:   If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

Skin Contact:   Wash off with soap and plenty of water. Consult a physician.

Eye Contact:   Flush eyes with water as a precaution.

Ingestion:   Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed:

None known.

Indication of any immediate medical attention and special treatment needed:   None known.

Section 5: Fire - Fighting Measures

Extinguisher media:

Extinguishing Media:   Water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special Hazards:   Carbon oxides.

Advice for Firefighters:   Wear Self-Contained Breathing Apparatus for firefighting if necessary.

Further Information:   Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel

to safe areas.

Environmental precautions:   Do not let product enter drains.

Methods and materials for containment and cleaning up:

Clean up promptly by sweeping or vacuum. For disposal see section 13.

Page 3

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 3/9

Section 7: Handling and Storage

Precautions for safe handling:

Handling:   Contains gas under pressure; may explode if heated.

Conditions for safe storage, including any incompatibilities:

Keep container lightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:

Component   CAS No. Value   Control Parameters   Basis

Carbon-13C dioxide   1111-72-4   TWA   5000 ppm   USA. ACGIH Threshold

Limit Values (TLV)

Remarks:   Asphyxia

STEL   30,000 ppm   USA, ACGIH

Threshold Limit Values

Remarks:   Asphyxia

TWA   5,000 ppm   USA. NIOSH-

9,000 mg/m3   Recommended

Exposure Limits

Remarks:   Normal constituent of air (about 300 ppm)

ST   30,000 ppm   USA. NIOSH-

54,000 mg/m3   Recommended Exposure

Limits

Remarks:   Normal constituent of air (about 300 ppm)

TWA   5000 ppm   USA. Occupational

9,000 mg/m3   Exposure Limits (OSHA)   –

Table Z-1 Limits for Air

Contaminants

Remarks:   The value in mg/m3 is approximate.

TWA   10,000 ppm   USA. OSHA   –   Table Z-1

18,000 mg/m3   Limits for Air Contaminants

1910.1000

Remarks:   Exposures under 10,000 ppm to be cited as de minimus.

STEL   30,000 ppm   USA. OSHA   –   Table Z-1

54,000 mg/m3   Limits for Air Contaminants

1910.1000

Exposure controls:

Appropriate engineering controls:   Handle in accordance with good industrial hygiene and safety practice. Wash hands

before breaks and at the end of workday.

Individual protection measures, such as personal protective equipment:

Page 4

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 4/9

Eye/Face:   Use equipment for eye protection tested and approved under appropriate government standards such as

NIOSH (US) or EN 166(EU).

Skin:   Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without

touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in

accordance with applicable laws and good laboratory practices. Wash and dry hands.

Full Contact:   Material: Fluorinated rubber. Minimum layer thickness: 0.7 mm. Breakthrough time: 480 min

Splash Contact: Material:   Fluorinated rubber. Minimum layer thickness: 0.7 mm. Breakthrough time: 480 min

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the

supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an industrial

hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It should not be

construed as offering an approval for any specific use scenario.

Body Protection:   Impervious clothing. The type of protective equipment must be selected according to the

concentration and amount of the dangerous substance at the specific workplace.

Respiratory Protection:   Where risk assessment shows air-purifying respirators are appropriate, use a full-face

respirator with multipurpose combination (US) or type AXBEK (EN 14387) respirator cartridges as a backup to

engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use

respirators and components tested and approved under appropriate government standards such as NIOSH (US) or

CEN (EU).

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Gas.

Colour:   Colourless.

Odour:   No data available.

Odour threshold:   No data available.

pH (concentration):   No data available.

Melting point/range (° C):   No data available.

Boiling point/range (° C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   Not applicable.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour pressure (20   ° C):   43,878.6 mmHg

Vapour density:   1.55

Relative densi ty (25 ° C):   No data available

Water solubility (g/L ) at 20 ° C:   No data available.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Page 5

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 5/9

Section 10: Stability and Reactivity

Reactivity:   No data available.

Stability:   Stable under recommended storage conditions.

Possibility of Hazardous Reactions:   No data available.

Conditions to avoid:   No data available.

Incompatible materials:   No data available.

Hazardous Decomposition Products / Other Decomposition Products:   No data available.

Section 11: Toxicological Information

Information on toxicological effects:

Not classified based on available information.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results   of   PBT&   vPvB   assessment:   PBT/vPvB   assessment   not   available,   as   chemical   safety   assessment   not

required/not conducted.

Other adverse effects:   No data available.

Page 6

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 6/9

Section 13: Disposal Considerations

Product:   Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed professional

waste disposal service to dispose of this material.

Contaminated Packaging:   Dispose of as unused product.

Section 14: Transport Information

UN number:   1013

UN proper shipping name:   CARBON DIOXIDE.

Transport hazard class:   Class 2.2

Transport hazard label:

Marine pollutant:   No.

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Page 7

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 7/9

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Additional Information:

SARA 302 Components:   No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components:   This material does not contain any chemical components with known CAS numbers that

exceed the threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards:   Sudden Release of Pressure Hazard.

Massachusetts Right to Know Components:   Carbon-13C dioxide/CAS No. 1111-72-4/Revision Date 1993-04-24

Pennsylvania Right to Know Components:   Carbon-13C dioxide/CAS No. 1111-72-4/Revision Date 1993-04-24

New Jersey Right to Know Components:   Carbon-13C dioxide/CAS No. 1111-72-4/Revision Date 1993-04-24

California Prop. 65 Components:   This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

Page 8

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 8/9

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

Page 9

Safety Data Sheet for Carbon-13C

Dioxide, Enriched Carbon-13C Dioxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 10-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Carbon-13C dioxide,   13 CO 2 ,   Enriched Carbon-13C dioxide   https://www.buyisotope.com   Page 9/9

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H280 Contains gas under pressure; may explode if heated.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.