Cerium (Ce) Standard Solution SRM 3110 SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/cerium-ce-standard-solution-nist-srm-3110-en-2ad1c523a4

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

08 December 2016

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   3110

SRM Name: Cerium (Ce) Standard Solution

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended for use as a primary calibration standard for the quantitative

determination of cerium. A unit of SRM 3110 consists of five 10 mL sealed borosilicate glass ampoules of an

acidified aqueous solution prepared gravimetrically to contain a known mass fraction of cerium. The solution

contains nitric acid at a volume fraction of approximately 16 %.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                        +1-703-527-3887   (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Classification

Physical Hazard:    Not classified.

Health Hazard:      Skin Corrosion/Irritation            Category 1B

Serious Eye Damage/Eye Irritation   Category 1

Label Elements

Symbol

Signal Word

DANGER

Hazard Statement(s)

H314               Causes severe skin burns and eye damage.

Precautionary Statement(s)

P260            Do not breathe fumes, mists, vapors, or spray.

P264             Wash hands thoroughly after handling.

P280              Wear protective gloves, protective clothing, and eye protection.

P301+P330+P331      If swallowed: Rinse mouth. Do NOT induce vomiting.

P303+P361+P353      If on skin (or hair): Remove immediately all contaminated clothing.  Rinse skin with

water.

P304+P340              If inhaled: Remove person to fresh air and keep comfortable for breathing.

P305+P351+P338      If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if

present and easy to do. Continue rinsing.

P310                Immediately call a doctor.

P363             Wash contaminated clothing before reuse.

SRM 3110                                                                             Page 1 of 7

Page 2

P405                 Store locked up.

P501                Dispose of container and contents according to local regulations.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Cerium in nitric acid solution

Other Designations:

Nitric acid (aqua fortis; hydrogen nitrate; azotic acid; engraver’s acid)

Cerium nitrate [Cerous nitrate; cerium (III) nitrate; nitric acid, cerium (III) salt]

NOTE: Cerium in nitric acid solution forms a solvated cerium nitrate salt.

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the NIST Certificate

of Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)               (%)

Nitric acid                           7697-37-2         231-714-2               <20

Cerium nitrate                       10108-73-3         233-297-2                      2.3

Non-Hazardous Component(s)

Water                               7732-18-5         231-791-2               >77

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation: If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and

shoes. Get immediate medical attention. Thoroughly clean and dry contaminated clothing before reuse. Destroy

contaminated shoes.

Eye Contact: Immediately flush eyes, including under the eyelids with copious amounts of water for at least 15

minutes. Seek immediate medical attention.

Ingestion: Contact a poison control center immediately for instructions. Do not induce vomiting. Give water to

rinse out mouth. Never give liquids to a person with reduced awareness or becoming unconscious.  If vomiting

occurs, keep head lower than hips to prevent aspiration.  If not breathing, give artificial respiration by qualified

personnel. Seek immediate medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Acid burns to skin, eyes, and lungs.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek immediate medical attention.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing media appropriate to the surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: Thermal decomposition will produce oxides of nitrogen and cerium.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

SRM 3110                                                                             Page 2 of 7

Page 3

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 3            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures:  Immediately contact emergency

personnel. Keep unnecessary personnel away. Use suitable protective equipment; see Section 8, “Exposure Controls

and Personal Protection”.

Methods and Materials for Containment and Clean up: Do not touch spilled material. Notify safety personnel

of spills. Absorb with sand or other non-combustible material. Collect spilled material in appropriate container for

disposal. Isolate hazard area and deny entry.

7. HANDLING AND STORAGE

Safe Handling Precautions: See Section 8, “Exposure Controls and Personal Protection”.

Storage:  Store and handling in accordance with all current regulations and standards.  Keep separated from

incompatible substances (see Section 10, “Stability and Reactivity”).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits

Component: Nitric acid:

NIOSH (REL): 5 mg/m3 (2 ppm) TWA

10 mg/m3 (4 ppm) STEL

65 mg/m3 (25 ppm) IDLH

ACGIH (TLV): 5 mg/m3 (2 ppm) TWA

10 mg/m3 (4 ppm) STEL

OSHA (PEL):  5 mg/m3 (2 ppm) TWA

Component: Cerium nitrate

No occupational exposure limits established.

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:    If workplace conditions warrant a respirator, a respiratory protection program that

meets OSHA 29CFR 1910.134 must be followed. Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eyewash station should be

readily available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task

being performed and the risks involved and should be approved by a specialist before handling this product.

Chemical-resistant gloves should be worn at all times when handling chemicals.

SRM 3110                                                                             Page 3 of 7

Page 4

9. PHYSICAL AND CHEMICAL PROPERTIES

NOTE: The physical and chemical data provided are for the pure components.

Descriptive Properties:                                Nitric acid               Cerium nitrate

(<20 % of this SRM)             (2.3 % of this SRM)

Appearance (physical state, color, etc.):       colorless to yellow              solid

liquid

Molecular Formula                  HNO3                      Ce(NO3)3

Molar Mass (g/mol)                         63.01                        326.13

Odor                                                  irritating odor                 not available

Odor threshold                               not available                  not available

pH                                     1 (1 M)                       not available

Evaporation rate                             not available                  not applicable

Melting point/freezing point               –42 ºC (–43 ºF)                not available

Relative Density as specific gravity           1.5027 at 25 ºC                not available

(water = 1)

Vapor Pressure                              47.9 at 20 ºC                  not available

Vapor Density (air = 1)                       3.2                            not applicable

Viscosity (cP)                                not available                  not available

Solubility(ies)                                 miscible with water and        soluble in water, alcohol,

ether                       and acetone

Partition coefficient (n-octanol/water)        not available                  not available

Particle Size                                  not applicable                 not available

Thermal Stability Properties

Autoignition Temperature                    not applicable                 not applicable

Thermal Decomposition                      not applicable                 not available

Initial boiling point and boiling range       83 ºC (181 ºF)                 not available

Explosive Limits, LEL (Volume %)           not applicable                 not applicable

Explosive Limits, UEL (Volume %)          not applicable                 not applicable

Flash Point                                   not applicable                 not applicable

Flammability (solid, gas)                     not applicable                 not applicable

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:   X    Stable               Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Contact with combustible or incompatible materials.

Incompatible Materials:  Acids, combustible materials, halo carbons, amines, bases, oxidizing materials, metals,

halogens, metal salts, metal oxides, reducing agents, peroxides, metal carbide, cyanides.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will form oxides of nitrogen and cerium.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

SRM 3110                                                                             Page 4 of 7

Page 5

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics:  Burning pain; severe skin

corrosion and eye damage.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation: Inhalation of nitric acid can damage the mucous membranes and upper respiratory tract. Short term

exposure may cause irritation and inflammation of the upper respiratory tract, coughing, choking, sore throat,

shortness of breath, headache, dizziness, and nausea. Long term exposure to acid fumes may cause damage to

teeth, bronchial irritation, chronic cough, bronchial pneumonia, and gastrointestinal disturbances. Rare earth salts

can cause itching, sensitivity to heat, and lung damage.

Skin Contact:  Nitric acid can cause severe skin burns. Severity of the damage depends on the concentration

and duration of exposure.  Effects of acid burns may be delayed. Rare earth salts can cause irritation and hair

loss.

Eye Contact:  Nitric acid can cause severe eye irritation, corneal burns, permanent eye damage, or blindness.

Severity of the damage depends on the concentration and duration of exposure.  Rare earth salts can cause

irritation and eye damage.

Ingestion:  If ingested, nitric acid can cause severe burns and damage to the gastrointestinal tract. Rare earth

salts can cause tearing, diarrhea, bluish skin color, and liver damage.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified.

Nitric acid: Rat, Inhalation LC50: 130 mg/m3 (4 h)

Cerium nitrate: Rat, Oral LD50: 3154 mg/kg

Skin Corrosion/Irritation: This SRM contains >1 % of nitric acid and it is classified as Category 1B.

Serious Eye damage/Eye irritation: This SRM contains >1 % nitric acid and it is classified as Category 1.

Respiratory Sensitization: No data available.

Skin Sensitization: No data available.

Germ Cell Mutagenicity: No data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen               Yes       X   No

Nitric acid and cerium nitrate are not listed by NTP, IARC or OSHA as carcinogens/potential carcinogens.

Reproductive Toxicity: Not classified.

Nitric acid: Rat, Oral TDLo: 21 150 mg/kg (pregnant 1 d to 21 d)

Nitric acid: Rat, Oral TDLo: 2345 mg/kg (pregnant 18 d)

Cerium nitrate: Rat, Intratesticular TDLo: 26 092 µg/kg (1 d)

Specific Target Organ Toxicity, Single Exposure: No data available.

Specific Target Organ Toxicity, Repeated Exposure: No data available.

Aspiration Hazard: No data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data

Nitric acid: Fish, mosquitofish (Gambusia affinis) LC50: 72 mg/L (96 h)

Cerium nitrate: Invertebrate, water flea, (Daphnia magna) EC50: >100 mg/L (48 h)

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

SRM 3110                                                                             Page 5 of 7

Page 6

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations. Nitric acid

subject to disposal regulations: U.S. EPA 40 CFR 262, Hazardous Waste Numbers: D001, D002.

No data available for cerium nitrate.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: UN1760, Corrosive liquid, n.o.s. (contains nitric acid), Hazard Class 8, Packing Group II.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4):  Nitric acid, 1000 lbs. (454 kg) final RQ

SARA Title III Section 302 (40 CFR 355.30):  Nitric acid, 1000 lbs. (454 kg) TPQ

SARA Title III Section 304 (40 CFR 355.40):  Nitric acid, 1000 lbs. (454 kg) EPCRA RQ

SARA Title III Section 313 (40 CFR 372.65):  1 % de minimis concentration for nitric acid

OSHA Process Safety (29 CFR 1910.119):     Regulated for nitric acid at higher concentrations

500 lbs. TQ (≥94.5 % by weight).

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:       Yes

CHRONIC HEALTH:     Yes

FIRE:               No

REACTIVE:           No

PRESSURE:           No

State Regulations: Not listed under California Proposition 65.

U.S. TSCA Inventory: Nitric acid and cerium nitrate are listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations: WHMIS Information is not provided for this material.

SRM 3110                                                                             Page 6 of 7

Page 7

16. OTHER INFORMATION

Issue Date: 08 December 2016

Sources:   ChemADVISOR, Inc., SDS Nitric Acid, 09 December 2015.

ChemADVISOR, Inc., SDS Cerous Nitrate, 09 December 2015.

Hazardous Substances Data Bank (HSDB), National Library of Medicine's TOXNET system, Nitric

Acid CAS No. 7697-37-2; available at http://toxnet.nlm.nih.gov (accessed Dec 2016).

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NRC          Nuclear Regulatory Commission

Hygienists

ALI          Annual Limit on Intake                     NTP           National Toxicology Program

CAS         Chemical Abstracts Service                 OSHA         Occupational Safety and Health Administration

CERCLA     Comprehensive Environmental Response,         PEL           Permissible Exposure Limit

Compensation, and Liability Act

CFR         Code of Federal Regulations                RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                 REL         Recommended Exposure Limit

EC50          Effective Concentration, 50 %             RM           Reference Material

EINECS      European Inventory of Existing Commercial       RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

LC50          Lethal Concentration, 50 %                   STEL          Short Term Exposure Limit

LD50          Lethal Dose, 50 %                        STOT          Specific Target Organ Toxicity

LEL         Lower Explosive Limit                    TLm          Threshold Limit, median

SDS           Safety Data Sheet                        TLV          Threshold Limit Value

NFPA         National Fire Protection Association            TPQ          Threshold Planning Quantity

NIOSH        National Institute for Occupational Safety and Health  TSCA         Toxic Substances Control Act

NIST          National Institute of Standards and Technology     TWA        Time Weighted Average

n.o.s.         Not Otherwise Specified                   UEL         Upper Explosive Limit

WHMIS      Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 3110                                                                             Page 7 of 7