Cerium (III) Bromide Hydrate 1405 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iii-bromide-hydrate-prochem-en-3c02a76b28

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Cerium (III) Bromide Hydrate

PRODUCT NUMBER:  1405

CAS NUMBER:       396654-07-2

SYNONYMS:         Cerous bromide hydrate, Cerium tribromide hydrate

MANUFACTURER:

Prochem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                    TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315                 Causes skin irritation.

H319                 Causes serious eye irritation.

H335               May cause respiratory irritation.

Precautionary Phrases

P261                    Avoid breathing dust/fume/gas/mist/vapors/spray.

P280                 Wear protective gloves/protective clothing/face protection/eye protection.

P305+P351+P338         IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy

to do. Continue rinsing.

P304+P340                IF INHALED: Remove person to fresh air and keep comfortable for breathing.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 0

EYE CONTACT: May be a mild irritant to the eyes.

SKIN CONTACT: May cause slight irritation of the skin.

INHALATION: Inhalation of dust can lead to irritation of the respiratory tract.

INGESTION: No information available on the physiological effects of ingestion.

CERIUM (III) BROMIDE HYDRATE                                                           PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: CeBr3! XH2O

Molecular Weight: 379.85 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Cerium (III) Bromide Hydrate                                             396654-07-2                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with plenty of water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen bromide and cerium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Avoid contact

with skin and eyes. Protect from humidity and water.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from oxidizing agents, water, and

moisture. Store under dry inert gas.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

CERIUM (III) BROMIDE HYDRATE                                                           PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White solid

ODOR: None

MOLECULAR WEIGHT: 379.85 (anhy.)

BOILING POINT: No data available

MELTING POINT: No data available

SPECIFIC GRAVITY: No data available

SOLUBILITY: Very soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: None

INCOMPATIBILITY: Oxidizing agents, water, and moisture

DECOMPOSITION PRODUCTS: Hydrogen bromide and cerium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: No data available

CARCINOGENIC EFFECTS: No data available

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good

SECTION 15  REGULATORY INFORMATION

TSCA: Listed

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 04/15

SDS DEPT.

CERIUM (III) BROMIDE HYDRATE                                                           PAGE 3