Cerium(III) chloride anhydrous ALFAA21112 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/cerium-iii-chloride-anhydrous-sds-avocado-research-chemicals-ltd-part-of-thermo-fisher-en-6325d8cf2a

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 8             SAFETY DATA SHEET

Creation Date 19-Aug-2011

Revision Date 23-Jul-2026

Version 5

ALFAA21112

Cerium(III) chloride anhydrous

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:                  三氯化铈(III)三氯化铈(III)三氯化铈(III)三氯化铈(III)

Product Description:                 Cerium(III) chloride anhydrous

Cat No. :                        21112

Synonyms                       Cerium trichloride.

CAS No                           7790-86-5

Molecular Formula             Ce Cl3

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Powder Solid                                Light grey                              Odorless

Emergency Overview

May be harmful if swallowed. Causes severe skin burns and eye damage. Very toxic to aquatic life with long lasting effects.

Hygroscopic.

Classification of the substance or mixture

Acute Oral Toxicity                                                                Category 5

Skin Corrosion/Irritation                                          C Category 1

Serious Eye Damage/Eye Irritation                                                  Category 1

Acute aquatic toxicity                                                              Category 1

Chronic aquatic toxicity                                                            Category 1

Label Elements

______________________________________________________________________________________________

Page 1

Page 2

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 2 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

Signal Word                    Danger

Hazard Statements

H303 - May be harmful if swallowed

H314 - Causes severe skin burns and eye damage

H410 - Very toxic to aquatic life with long lasting effects

Precautionary Statements

Prevention

P264 - Wash face, hands and any exposed skin thoroughly after handling

P270 - Do not eat, drink or smoke when using this product

P280 - Wear protective gloves/protective clothing/eye protection/face protection

Response

P303 + P361 + P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

P301 + P330 + P331 - IF SWALLOWED: rinse mouth. Do NOT induce vomiting

P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

P310 - Immediately call a POISON CENTER or doctor

P362 + P364 - Take off contaminated clothing and wash it before reuse

Storage

P403 - Store in a well-ventilated place

Disposal

P501 - Dispose of contents/ container to an approved waste disposal plant

Physical and Chemical Hazards

Hygroscopic.

Health Hazards

May be harmful if swallowed. Corrosive. Causes skin and eye burns.

Environmental hazards

Very toxic to aquatic life with long lasting effects. Will likely be mobile in the environment due to its water solubility. The product is

water soluble, and may spread in water systems.

This product does not contain any known or suspected endocrine disruptors.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Component                              CAS No                Weight %

Cerium chloride (CeCl3)                                    7790-86-5                  >95

SECTION 4. FIRST AID MEASURES

General Advice

Immediate medical attention is required. Show this safety data sheet to the doctor in attendance.

Eye Contact

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required.

Keep eye wide open while rinsing.

Skin Contact

Wash off immediately with soap and plenty of water while removing all contaminated clothes and shoes. Call a physician

immediately.

Inhalation

Remove to fresh air. Call a physician or poison control center immediately. Do not use mouth-to-mouth method if victim ingested or

inhaled the substance; give artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper

respiratory medical device. If not breathing, give artificial respiration.

Ingestion

______________________________________________________________________________________________

Page 2

Page 3

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 3 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

Immediate medical attention is required. Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Drink

plenty of water.

Most important symptoms and effects

Causes burns by all exposure routes. Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible

perforation of stomach or esophagus should be investigated: Ingestion causes severe swelling, severe damage to the delicate

tissue and danger of perforation

Self-Protection of the First Aider

Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and prevent spread of

contamination.

Notes to Physician

Treat symptomatically.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Extinguishing media which must not be used for safety reasons

No information available.

Specific Hazards Arising from the Chemical

The product causes burns of eyes, skin and mucous membranes. Do not allow run-off from fire-fighting to enter drains or water

courses.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear. Thermal decomposition can lead to release of irritating gases and vapors.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Evacuate personnel to safe areas. Use personal protective equipment as required. Avoid contact with skin, eyes or clothing.

Environmental Precautions

Do not flush into surface water or sanitary sewer system. Do not allow material to contaminate ground water system. Prevent

product from entering drains. Local authorities should be advised if significant spillages cannot be contained.

Methods for Containment and Clean Up

Avoid dust formation. Sweep up and shovel into suitable containers for disposal.

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

Do not breathe dust. Do not ingest. If swallowed then seek immediate medical assistance. Do not get in eyes, on skin, or on

clothing. Use only under a chemical fume hood. Wear personal protective equipment/face protection.

Storage

Keep containers tightly closed in a dry, cool and well-ventilated place. Corrosives area. Store under an inert atmosphere.

Specific Use(s)

Use in laboratories

______________________________________________________________________________________________

Page 3

Page 4

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 4 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Monitoring methods

BS EN 14042:2003 Title Identifier: Workplace atmospheres. Guide for the application and use of procedures for the assessment of

exposure to chemical and biological agents. MDHS14/3 General methods for sampling and gravimetric analysis of respirable and

inhalable dust

Exposure Controls

Engineering Measures

Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations and safety showers are close to the

workstation location. Wherever possible, engineering control measures such as the isolation or enclosure of the process, the

introduction of process or equipment changes to minimise release or contact, and the use of properly designed ventilation systems,

should be adopted to control hazardous materials at source.

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Natural rubber     See manufacturers               -          EN 374             (minimum requirement)

Nitrile rubber       recommendations

Neoprene

PVC

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use    Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: Particulates filter conforming to EN 143

Small scale/Laboratory use     Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

limits are exceeded or if irritation or other symptoms are experienced.

Recommended half mask:- Particle filtering: EN149:2001

When RPE is used a face piece Fit Test should be conducted

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls   Prevent product from entering drains. Do not allow material to contaminate ground water

system. Local authorities should be advised if significant spillages cannot be contained.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

______________________________________________________________________________________________

Page 4

Page 5

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 5 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

Appearance                           Light grey

Physical State                   Powder Solid

Odor                              Odorless

Odor Threshold               No data available

pH                         No information available

Melting Point/Range               848 °C  / 1558.4 °F

Softening Point                No data available

Boiling Point/Range               1727 °C  / 3140.6 °F       @ 760 mmHg

Flash Point                   No information available           Method - No information available

Evaporation Rate                  Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Vapor Pressure                No information available

Vapor Density                     Not applicable                         Solid

Specific Gravity  / Density        No data available

Bulk Density                  No data available

Water Solubility                     Soluble

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Autoignition Temperature          Not applicable

Decomposition Temperature      No data available

Viscosity                         Not applicable                         Solid

Explosive Properties            No information available

Oxidizing Properties            No information available

Molecular Formula             Ce Cl3

Molecular Weight                  246.48

SECTION 10. STABILITY AND REACTIVITY

Stability                            Hygroscopic.

Hazardous Reactions            None under normal processing.

Hazardous Polymerization         Hazardous polymerization does not occur.

Conditions to Avoid                 Incompatible products. Excess heat. Exposure to moist air or water.

Materials to avoid                  Bases. Strong oxidizing agents. Strong acids.

Hazardous Decomposition Products Thermal decomposition can lead to release of irritating gases and vapors. Hydrogen

chloride gas.

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

(a) acute toxicity;

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Cerium chloride (CeCl3)               2800 mg/kg (Rat)

(b) skin corrosion/irritation;        Category 1 C

(c) serious eye damage/irritation;   Category 1

(d) respiratory or skin sensitization;

______________________________________________________________________________________________

Page 5

Page 6

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 6 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

Respiratory                  Based on available data, the classification criteria are not met

Skin                        Based on available data, the classification criteria are not met

(e) germ cell mutagenicity;        Based on available data, the classification criteria are not met

(f) carcinogenicity;               Based on available data, the classification criteria are not met

There are no known carcinogenic chemicals in this product

(g) reproductive toxicity;          Based on available data, the classification criteria are not met

(h) STOT-single exposure;         Based on available data, the classification criteria are not met

(i) STOT-repeated exposure;       Based on available data, the classification criteria are not met

Target Organs               None known.

(j) aspiration hazard;               Not applicable

Solid

Other Adverse Effects            The toxicological properties have not been fully investigated.

Symptoms  / effects,both acute and Product is a corrosive material. Use of gastric lavage or emesis is contraindicated.

delayed                             Possible perforation of stomach or esophagus should be investigated: Ingestion causes

severe swelling, severe damage to the delicate tissue and danger of perforation

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects                 Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic

environment. The product contains following substances which are hazardous for the

environment.

Component                Freshwater Fish         Water Flea        Freshwater Algae         Microtox

Cerium chloride (CeCl3)          LC50: 0.13 mg/L/96h    EC50: 6.9 mg/L/48h

(Oncorhynchus Mykiss)

Persistence and Degradability

Persistence                     Soluble in water, Persistence is unlikely, based on information available.

Degradability                  Not relevant for inorganic substances.

Degradation in sewage          Contains substances known to be hazardous to the environment or not degradable in waste

treatment plant                 water treatment plants.

Bioaccumulative Potential          Bioaccumulation is unlikely

Bioconcentration factor (BCF)                                             89

Mobility in soil                  The product is water soluble, and may spread in water systems Will likely be mobile in the

environment due to its water solubility Highly mobile in soils

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

______________________________________________________________________________________________

Page 6

Page 7

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 7 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Should not be released into the environment. Waste is classified as hazardous. Dispose of

Products                                  in accordance with the European Directives on waste and hazardous waste. Dispose of in

accordance with local regulations.

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point.

Other Information              Do not flush to sewer. Large amounts will affect pH and harm aquatic organisms. Waste

codes should be assigned by the user based on the application for which the product was

used. Do not empty into drains. Do not let this chemical enter the environment.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport

UN-No                       UN1759

Proper Shipping Name              Corrosive solid, n.o.s.

Technical Shipping Name      Cerium chloride (CeCl3)

Hazard Class                    8

Packing Group                                     II

IMDG/IMO

UN-No                       UN1759

Proper Shipping Name              Corrosive solid, n.o.s.

Technical Shipping Name      Cerium chloride (CeCl3)

Hazard Class                    8

Packing Group                                     II

IATA

UN-No                       UN1759

Proper Shipping Name              Corrosive solid, n.o.s.

Technical Shipping Name      Cerium chloride (CeCl3)

Hazard Class                    8

Packing Group                                     II

Special Precautions for User      No special precautions required

SECTION 15. REGULATORY INFORMATION

International Inventories

X = listed, China (IECSC), Europe (EINECS/ELINCS/NLP), U.S.A. (TSCA), Canada (DSL/NDSL), Philippines (PICCS), Japan (ENCS), Japan

(ISHL), Australia (AICS), Korea (KECL).

Component        The        List of    IECSC   EINECS   TSCA   DSL   PICCS  ENCS    ISHL    AICS    KECL

Inventory of dangerous

Hazardous  goods GB

Chemicals   12268 -

(2015      2012

Edition)

Cerium chloride             -                 -        X      232-227-8    X      X            -      X      X      X     KE-05416

(CeCl3)

National Regulations

______________________________________________________________________________________________

Page 7

Page 8

ALFAA21112               SAFETY DATA SHEET                     Revision Date 23-Jul-2026Page 8 / 8

Cerium(III) chloride anhydrous

______________________________________________________________________________________________

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

Creation Date                     19-Aug-2011

Revision Date                      23-Jul-2026

Revision Summary                Not applicable.

Training Advice

Chemical incident response training.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

89                                            VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of Safety Data Sheet

______________________________________________________________________________________________

Page 8