Cerium (III) Chloride, Hydrate 1415 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iii-chloride-hydrate-prochem-en-5a142d35d2

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Cerium (III) Chloride, Hydrate

PRODUCT NUMBER:  1415

CAS NUMBER:        19423-76-8

SYNONYMS:         Cerium trichloride hydrate

MANUFACTURER:

Prochem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                    TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P305+P351+P338     IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P304+P340            IF INHALED: Remove person to fresh air and keep comfortable for breathing.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                           Reactivity Hazard: 1

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 1

EYE CONTACT: Causes severe irritation to the eyes.

SKIN CONTACT: Causes irritation of the skin.

INHALATION: Inhalation of dust can lead to irritation of the respiratory tract.

INGESTION: No data available.

CERIUM (III) CHLORIDE, HYDRATE                                                          PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: CeCl3! XH2O

Molecular Weight: 246.47 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Cerium (III) Chloride, Hydrate                                               19423-76-8                                           100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Do not induce vomiting. Seek medical assistance immediately.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen chloride and cerium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store under dry inert gas. Hygroscopic.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White solid

ODOR: None

MOLECULAR WEIGHT: 246.47 (anhy.)

CERIUM (III) CHLORIDE, HYDRATE                                                          PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

BOILING POINT: No data available

MELTING POINT: 212° C

SPECIFIC GRAVITY: 3.92 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Incompatibles

INCOMPATIBILITY: Strong acids, strong bases, and strong reducing agents.

DECOMPOSITION PRODUCTS: Hydrogen chloride and cerium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good

SECTION 15  REGULATORY INFORMATION

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc. makes

no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the product based

on the information.

DATE PREPARED: 02/15

SDS DEPT.

CERIUM (III) CHLORIDE, HYDRATE                                                          PAGE 3