Cerium (III) Chloride Solution SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iii-chloride-solution-sds-prochem-en-1933868bfe

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Cerium (III) Chloride Solution

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word    Warning

Hazard Statements

H303             May be harmful if swallowed

H315               Causes skin irritation

H319               Causes serious eye irritation

H335             May cause respiratory irritation.

Precautionary Phrases

P264             Wash hands thoroughly after handling.

P270             Do not eat, drink, or smoke when using this product.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P301 + P312           IF SWALLOWED: call a POISON CENTER or doctor/physician IF you feel unwell.

P302 + P352           IF ON SKIN: wash with plenty of soap and water.

P305 + P351 + P338   IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 2            Flammability: 0                  Reactivity:0

NFPA RATING:

Health: 2              Fire: 0                           Reactivity Hazard: 0

CERIUM (III) CHLORIDE SOLUTION                                                                 PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

POTENTIAL HEALTH EFFECTS:  Inhalation

EYE CONTACT: Causes severe irritation to eyes.

SKIN CONTACT: Causes irritation of the skin.

INHALATION: Inhalation of vapor/mist can lead to irritation of the respiratory tract.

INGESTION: No data

CHRONIC HEALTH EFFECTS: None known

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Molecular Weight: No data

CHEMICAL NAME                                        CAS#                                %

Cerium (III) Chloride hexahydrate                                         18618-55-8                                              66

Water                                                                 7732-18-5                                               34

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water (lukewarm if possible) for at least 15 minutes.

Assure flushing under eyelids. A victim may need assistance in keeping their eyelids open.  If redness or irritation persists, get

medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation of redness persists.

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in breathing,

coughing, wheezing, or pain. In such cases seek immediate medical assistance.

INGESTION: Do not induce vomiting. Seek medical assistance immediately.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

EXTINGUISHING MEDIUM: Not applicable

SPECIAL FIRE FIGHTING PROCEDURES:   If this product is involved in a fire, fire fighters should be equipped with a NIOSH

approved positive pressure self-contained breathing apparatus and full protective clothing.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Toxic fumes of hydrogen chloride gas is emited.

UNUSUAL FIRE OR EXPLOSION HAZARDS: None known

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECUATIONS: Evacuate area if necessary. Wear full protective suit, gloves, and respirator. Ventilate area.

ENVIRONMENTAL PRECAUTIONS: Do not allow material to be released to the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Isolate spill. Soak up with inert material (sand, vermiculite,

etc.). Dispose of in accordance with all waste disposal regulations.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Be sure to have appropriate exhaust and ventilation.

CONDITIONS FOR SAFE STORAGE: Keep in a tightly sealed container. Store in a cool, dry well-ventilated are.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses w/side shields, or safety goggles when handling a

chemical substance in the laboratory.

SKIN PROTECTION: Wear appropriate chemical resistant gloves.

VENTILATION: Local exhaust

CERIUM (III) CHLORIDE SOLUTION                                                                 PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

RESPIRATOR:   If ventilation is not available, a respirator should be worn. The use of respirators requires a Respirator Protection

Program to be in compliance with 29CFR 1910.34

ADDITIONAL PROTECTION: Use in a controlled environment. Wear adequate protection to avoid contact with skin, eyes,

clothing, and shoes. Eyewash and shower capable of sustained flushing should be readily available. Do not eat, smoke, or

drink in area.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Yellow liquid

MELTING POINT: No data

BOILING POINT: No data

MOLECULAR  WEIGHT: No data

VAPOR PRESSURE: Not applicable

VAPOR DENSITY (AIR = 1): No data

% VOLATILE BY VOLUME: Non volatile

SOLUBILITY IN WATER: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: No data

INCOMPATIBILITY: Oxidizing agents, acids, and bases

DECOMPOSITION PRODUCTS: Toxic fumes of hydrogen chloride gas.

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No data

MUTAGENIC EFFECTS: No data

TETRATOGENIC EFFECTS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local state and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: No data

SARA (TITLE 313): None

RCRA: No data

CERIUM (III) CHLORIDE SOLUTION                                                                PAGE 3

Page 4

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 10/13

CERIUM (III) CHLORIDE SOLUTION                                                                PAGE 4