Cerium (III) fluoride 1419 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iii-fluoride-prochem-en-e6f8269999

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Cerium (III) fluoride

PRODUCT NUMBER:  1419

CAS NUMBER:        7758-88-5

SYNONYMS:         Cerous fluoride

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL 61109                                      TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H312 + H332         Harmful in contact with skin or if inhaled

H319               Causes serious eye irritation

H335             May cause respiratory irritation

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapours/spray

P280              Wear protective gloves/protective clothing/eye protection/face protection

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

HMIS CLASSIFICATION:

Health: 1            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 1              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: May cause irritation of the eyes.

SKIN CONTACT: May cause mild irritation and redness.

INHALATION: May cause irritation to the respiratory tract. Symptoms may include coughing and shortness of breath.

CERIUM (III) FLUORIDE                                                                         PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

INGESTION: Toxicity reduced to the extent that this compound is relatively insoluble. In large amounts may cause vomiting,

abdominal pain, and diarrhea.

ACUTE HEALTH EFFECTS: Ingestion of large amounts may cause fluoride poisoning. Symptoms include nausea, vomiting,

diarrhea, weakness, coma, and cardiovascular collapse.

CHRONIC HEALTH EFFECTS: Prolonged exposure to hydrolysable fluorine compounds can cause deterioration of bone and tooth

structure.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: CeF3

Molecular Weight: 197.12

CHEMICAL NAME                   CAS#             %               ACGIH (TWA)                     OSHA (PEL)

Cerium (III) fluoride                        7758-88-5                    100                   2.5 mg/m 3 (as F)                            2.5 mg/m 3 (as F)

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove the victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty

in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable or combustible.

AUTO IGNITION TEMPERATURE: None

EXPLOSION LIMITS: None

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen fluoride and cerium oxide.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Use personal protective equipment. Avoid dust formation. Ensure adequate ventilation. Evacuate

personnel to safe areas. Avoid breathing dust.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with powdered sodium

bicarbonate, lime, or calcium carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide

appropriate exhaust ventilation.

CONDITIONS FOR SAFE STORAGE: Keep container tightly closed in a dry and well-ventilated place. Moisture sensitive. Handle and

store under inert gas. Keep in a dry place.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

CERIUM (III) FLUORIDE                                                                         PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

ADDITIONAL PROTECTION: Protection to avoid contact with skin, eyes, and clothing. Safety shower and eyewash in area. Use in a

controlled environment. Do not drink, eat, or smoke in area.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White to off-white powder

MOLECULAR WEIGHT: 197.12

BOILING POINT: 2300° C (approx.)

MELTING POINT:1460° C

SPECIFIC GRAVITY: 6.16 gm/ml

VAPOR PRESSURE: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Hygroscopic

HAZARDOUS POLYMERIZATION: No hazardous polymerization

CONDITIONS TO AVOID: Contact with moisture

INCOMPATIBILITY: Active metals and oxidizing agents.

DECOMPOSITION PRODUCTS: Hydrogen fluoride and cerium oxide.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat - > 5,000 mg/kg

CARCINOGENIC EFFECTS: Components of this product present at levels greater than or equal to 0.1% is identified as a confirmed

carcinogen by IARC, ACGIH, NTP, and OSHA.

MUTAGENIC EFFECTS: No data available

TETRATOGENIC EFFECTS: No data available

RTECS DATA: FK6125000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 313 COMPONENTS: Title compound not listed

CERIUM (III) FLUORIDE                                                                         PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

OSHA: Toxic by inhalation. Harmful by skin absorption.  Irritant.

SARA 311/312 Hazards: Acute health hazard

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right To Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Cerium (III) fluoride

NEW JERSEY RIGHT TO KNOW COMPONENTS: Cerium (III) fluoride

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to the State of California to cause

cancer, birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 04/14

CERIUM (III) FLUORIDE                                                                         PAGE 4