Cerium (III) Nitrate Hexahydrate 1423-1 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iii-nitrate-hexahydrate-prochem-en-44e1119950

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Cerium (III) Nitrate Hexahydrate

PRODUCT NUMBER:  1423-1

CAS NUMBER:        10294-41-4

SYNONYMS:         Cerous nitrate hexahydrate, nitric acid, cerium (3+) salt

MANUFACTURER:

Prochem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                    TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire, oxidizer.

H318              Causes serious eye damage.

Precautionary Phrases

P210              Keep away from heat.

P220                Keep/Store away from clothing/combustible materials.

P221               Take any precaution to avoid mixing with combustibles.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P305+P351+P338     IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P310                 Immediately call a POISON CENTER or doctor/physician.

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction.

HMIS CLASSIFICATION:

Health: 2            Chronic: *               Fire: 0                     Reactivity Hazard: 2

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 2       Special: OX

CERIUM (III) NITRATE HEXAHYDRATE                                                        PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes slight to mild irritation of the eyes.

SKIN CONTACT: Causes slight to mild irritation of the skin.

INHALATION:  Irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: Ingestion may lead to dizziness, abdominal cramps, vomiting, bloody diarrhea, weakness, and convulsions.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Ce(NO3)3 ! 6H2O

Molecular Weight: 434.22

CHEMICAL NAME                                           CAS#                               %

Cerium (III) Nitrate Hexahydrate                                            10294-41-4                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Contact with combustible material may cause fire.

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistant foam. This substance is an oxidizer and its heat of reaction with reducing agents or combustibles may cause ignition.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides and cerium oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust. Keep away from heat and sources of ignition.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store under inert gas. Air sensitive. Hygroscopic.

Store away from flammable substances, reducing agents, metal powders, water, and moisture. Do not store with organic materials.

CERIUM (III) NITRATE HEXAHYDRATE                                                        PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White to yellow solid

ODOR: None

MOLECULAR WEIGHT: 434.22

BOILING POINT: No data available

MELTING POINT: 65° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Vary soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Hygroscopic. Air sensitive.

INCOMPATIBILITY: Strong reducing agents and strong acids.

DECOMPOSITION PRODUCTS: Nitrogen oxides and cerium oxides.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat – 4,200 mg/kg.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: FK6300000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN1477                                        Marine Pollutant: No

Nitrates, inorganic, n.o.s.

(Cerium (III) Nitrate Hexahydrate)

Class 5.1

PG II

CERIUM (III) NITRATE HEXAHYDRATE                                                        PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not listed

SARA 311/312: Reactivity Hazard, Acute Health Hazard, Chronic Health Hazard

SARA (TITLE 313): Not listed

PENNSYLVANIA RIGHT TO KNOW: Cerium (III) nitrate hexahydrate

NEW JERSEY RIGHT TO KNOW: Cerium (III) nitrate hexahydrate

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc. makes

no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the product based

on the information.

DATE PREPARED: 12/14

CERIUM (III) NITRATE HEXAHYDRATE                                                        PAGE 4