Cerium (IV) Ammonium Nitrate 1404 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iv-ammonium-nitrate-sds-prochem-en-9d9171952e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Cerium (IV) Ammonium Nitrate

PRODUCT NUMBER:  1404

CAS NUMBER:        16774-21-3

SYNONYMS:        Ammonium cerium (IV) nitrate, Ammonium hexanitratocerate (IV), Ceric ammonium nitrate,

diammonium cerium nitrate

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire, oxidizer.

H302                Harmful if swallowed.

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P210              Keep away from heat.

P220                 Keep/Store away from clothing/combustible materials.

P221               Take any precaution to avoid mixing with combustibles.

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P270             Do not eat, drink, or smoke when using this product.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P301+P312            IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF IN HALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

CERIUM (IV) AMMONIUM NITRATE                                                          PAGE 1

Page 2

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction.

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 2

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 2       Special: OX

EYE CONTACT: Causes serious eye irritation.

SKIN CONTACT: Causes skin irritation.

INHALATION: May cause respiratory irritation.

INGESTION: Harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: (NH4)2Ce(NO3)6

Molecular Weight: 548.22

CHEMICAL NAME                                           CAS#                               %

Cerium (IV) Ammonium Nitrate                                             16774-21-3                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistant foam. This substance is an oxidizer and its heat of reaction with reducing agents or combustibles may cause ignition.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides, ammonia, and cerium oxide fume.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust. Keep away from heat and sources of ignition.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store under inert gas. Store away from flammable

substances, reducing agents, strong bases, metal powders. Do not store with organic materials and acids.

CERIUM (IV) AMMONIUM NITRATE                                                          PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Orang red solid

ODOR: Pungent

MOLECULAR WEIGHT: 548.22

BOILING POINT: No data available

MELTING POINT: 107° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Avoid moisture

INCOMPATIBILITY: Strong reducing agents, organic materials, and powdered metals.

DECOMPOSITION PRODUCTS: Nitrogen oxides, ammonia, and cerium oxide fume.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat - > 300 – 2,000 mg/kg (OECD Test Guideline 401)

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: CQ9800000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN1477                                                Marine Pollutant: No

Nitrates, inorganic, n.o.s.

(Cerium (IV) Ammonium Nitrate)

Class 5.1

PG II

CERIUM (IV) AMMONIUM NITRATE                                                          PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not listed

SARA 311/312: Reactivity Hazard, Acute Health Hazard

SARA (TITLE 313): Not listed

PENNSYLVANIA RIGHT TO KNOW: Cerium (IV) Ammonium Nitrate

NEW JERSEY RIGHT TO KNOW: Cerium (IV) Ammonium Nitrate

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 12/14

CERIUM (IV) AMMONIUM NITRATE                                                          PAGE 4