Cerium(IV) sulfate, 0.1N Standardized Solution ALFAA35650 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/cerium-iv-sulfate-0-1n-standardized-solution-sds-avocado-research-chemicals-ltd-part-of-thermo-fisher-en-c15a7ee3b0

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 9             SAFETY DATA SHEET

Creation Date 16-Apr-2010

Revision Date 03-Aug-2026

Version 7

ALFAA35650

Cerium(IV) sulfate, 0.1N Standardized Solution

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:                 硫酸铈(IV)0.1N标准溶液硫酸铈(IV)0.1N标准溶液硫酸铈(IV)0.1N标准溶液硫酸铈(IV)0.1N标准溶液

Product Description:               Cerium(IV) sulfate, 0.1N Standardized Solution

Cat No. :                        35650

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Liquid                                 Yellow                    No information available

Emergency Overview

Causes severe skin burns and eye damage. May be corrosive to metals. Toxic to aquatic life with long lasting effects.

Classification of the substance or mixture

Substances/mixtures corrosive to metal                                              Category 1

Skin Corrosion/Irritation                                                           Category 1 B

Serious Eye Damage/Eye Irritation                                                  Category 1

Chronic aquatic toxicity                                                            Category 2

Label Elements

Signal Word                    Danger

Hazard Statements

H290 - May be corrosive to metals

______________________________________________________________________________________________

Page 1

Page 2

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 2 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

H314 - Causes severe skin burns and eye damage

H411 - Toxic to aquatic life with long lasting effects

Precautionary Statements

Prevention

P234 - Keep only in original packaging

P264 - Wash face, hands and any exposed skin thoroughly after handling

P271 - Use only outdoors or in a well-ventilated area

P280 - Wear protective gloves/protective clothing/eye protection/face protection

Response

P301 + P330 + P331 - IF SWALLOWED: rinse mouth. Do NOT induce vomiting

P303 + P361 + P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

P310 - Immediately call a POISON CENTER or doctor

P363 - Wash contaminated clothing before reuse

P390 - Absorb spillage to prevent material damage

Storage

P402 - Store in a dry place

P403 + P233 - Store in a well-ventilated place. Keep container tightly closed

P406 - Store in corrosion resistant polypropylene container with a resistant inliner

Disposal

P501 - Dispose of contents/ container to an approved waste disposal plant

Physical and Chemical Hazards

May be corrosive to metals.

Health Hazards

Corrosive. Causes skin and eye burns. Causes serious eye damage.

Environmental hazards

Toxic to aquatic life with long lasting effects. Will likely be mobile in the environment due to its water solubility. The product is

water soluble, and may spread in water systems.

Other Hazards

This product does not contain any known or suspected endocrine disruptors. Toxic to terrestrial vertebrates.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Component                              CAS No                Weight %

Water                                            7732-18-5                  87

Sulfuric acid                                          7664-93-9                  10

Sulfuric acid, cerium(4+) salt (2:1)                               13590-82-4                  3

SECTION 4. FIRST AID MEASURES

General Advice

Show this safety data sheet to the doctor in attendance. Immediate medical attention is required.

Eye Contact

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required.

Skin Contact

Wash off immediately with plenty of water for at least 15 minutes. Remove and wash contaminated clothing and gloves, including

the inside, before re-use. Call a physician immediately.

Inhalation

If not breathing, give artificial respiration. Remove from exposure, lie down. Do not use mouth-to-mouth method if victim ingested or

inhaled the substance; give artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper

respiratory medical device. Call a physician immediately.

______________________________________________________________________________________________

Page 2

Page 3

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 3 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

Ingestion

Do NOT induce vomiting. Clean mouth with water. Never give anything by mouth to an unconscious person. Call a physician

immediately.

Most important symptoms and effects

Causes burns by all exposure routes. Ingestion causes severe swelling, severe damage to the delicate tissue and danger of

perforation: Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible perforation of stomach or

esophagus should be investigated

Self-Protection of the First Aider

Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and prevent spread of

contamination.

Notes to Physician

Treat symptomatically.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Substance is nonflammable; use agent most appropriate to extinguish surrounding fire. CO 2, dry chemical, dry sand,

alcohol-resistant foam.

Extinguishing media which must not be used for safety reasons

No information available.

Specific Hazards Arising from the Chemical

Thermal decomposition can lead to release of irritating gases and vapors. The product causes burns of eyes, skin and mucous

membranes.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear. Thermal decomposition can lead to release of irritating gases and vapors.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Use personal protective equipment as required. Ensure adequate ventilation. Evacuate personnel to safe areas. Keep people away

from and upwind of spill/leak.

Environmental Precautions

Avoid release to the environment. Collect spillage. Do not flush into surface water or sanitary sewer system. See Section 12 for

additional Ecological Information.

Methods for Containment and Clean Up

Soak up with inert absorbent material. Keep in suitable, closed containers for disposal.

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

Wear personal protective equipment/face protection. Do not get in eyes, on skin, or on clothing. Use only under a chemical fume

hood. Do not breathe mist/vapors/spray. Do not ingest. If swallowed then seek immediate medical assistance.

Storage

Keep containers tightly closed in a dry, cool and well-ventilated place. Corrosives area.

______________________________________________________________________________________________

Page 3

Page 4

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 4 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

Specific Use(s)

Use in laboratories

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Component                China              ACGIH TLV           OSHA PEL              NIOSH

Sulfuric acid           TWA: 1 mg/m3          TWA: 0.2 mg/m3        (Vacated) TWA: 1 mg/m3        IDLH: 15 mg/m3

STEL: 2 mg/m3                               TWA: 1 mg/m3        REL = 1 mg/m3 (TWA)

Component            Hong Kong                Thailand            European Union       The United Kingdom

Sulfuric acid           TWA: 1 mg/m3           TWA: 1 mg/m3        TWA: 0.05 mg/m3 (8h)    STEL: 0.15 mg/m3 15 min

STEL: 3 mg/m3                                                 TWA: 0.05 mg/m3 8 hr

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH: NIOSH - National Institute for Occupational Safety and Health

Exposure Controls

Engineering Measures

Use only under a chemical fume hood. Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations

and safety showers are close to the workstation location. Wherever possible, engineering control measures such as the isolation

or enclosure of the process, the introduction of process or equipment changes to minimise release or contact, and the use of

properly designed ventilation systems, should be adopted to control hazardous materials at source.

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Butyl rubber      See manufacturers               -          EN 374             (minimum requirement)

recommendations

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use    Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: Particulates filter conforming to EN 143 or Acid gases filter

Type E Yellow conforming to EN14387

Small scale/Laboratory use     Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

limits are exceeded or if irritation or other symptoms are experienced.

Recommended half mask:- Valve filtering: EN405; or; Half mask: EN140; plus filter, EN

141

When RPE is used a face piece Fit Test should be conducted

______________________________________________________________________________________________

Page 4

Page 5

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 5 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls   Prevent product from entering drains. Do not allow material to contaminate ground water

system.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance                        Yellow

Physical State                        Liquid

Odor                       No information available

Odor Threshold               No data available

pH

Melting Point/Range            No data available

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                       Not applicable                   Method - No information available

Evaporation Rate                > 1 (ether = 1)

Flammability (solid,gas)            Not applicable                         Liquid

Explosion Limits               No data available

Vapor Pressure                No data available

Vapor Density                       0.7 (Water = 1.0)                         (Air = 1.0)

Specific Gravity  / Density           1.1

Bulk Density                      Not applicable                         Liquid

Water Solubility                     Soluble

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Autoignition Temperature        No data available

Decomposition Temperature      No data available

Viscosity                    No data available

Explosive Properties            No information available

Oxidizing Properties            No information available

SECTION 10. STABILITY AND REACTIVITY

Stability                             Stable under normal conditions.

Hazardous Reactions            None under normal processing.

Hazardous Polymerization         Hazardous polymerization does not occur.

Conditions to Avoid                 Incompatible products. Excess heat.

Materials to avoid                  Strong oxidizing agents.

Hazardous Decomposition Products Carbon monoxide (CO). Carbon dioxide (CO2). Sulfur oxides.

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

(a) acute toxicity;

Toxicology data for the components

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

______________________________________________________________________________________________

Page 5

Page 6

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 6 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

Water                                               -                                            -                                            -

Sulfuric acid                   2140 mg/kg  ( Rat )                                LC50 = 0.375 mg/L  ( Rat ) 4 h

(b) skin corrosion/irritation;       No data available

(c) serious eye damage/irritation;   No data available

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;             No data available

The table below indicates whether each agency has listed any ingredient as a carcinogen

Component               EU               UK               Germany               IARC

Sulfuric acid                                                                                    Group 1

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

Target Organs               None known.

(j) aspiration hazard;            No data available

Symptoms  / effects,both acute and Ingestion causes severe swelling, severe damage to the delicate tissue and danger of

delayed                                perforation: Product is a corrosive material. Use of gastric lavage or emesis is

contraindicated. Possible perforation of stomach or esophagus should be investigated

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects                  Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic

environment. The product contains following substances which are hazardous for the

environment.

Component                Freshwater Fish         Water Flea        Freshwater Algae         Microtox

Sulfuric acid              LC50: > 500 mg/L, 96h   EC50: 29 mg/L/24h                    -                                -

static (Brachydanio

rerio)

Persistence and Degradability

Persistence                     Soluble in water, Persistence is unlikely, based on information available.

Degradation in sewage          Contains substances known to be hazardous to the environment or not degradable in waste

treatment plant                 water treatment plants.

Bioaccumulative Potential          Bioaccumulation is unlikely

______________________________________________________________________________________________

Page 6

Page 7

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 7 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

Mobility in soil                  The product is water soluble, and may spread in water systems Will likely be mobile in the

environment due to its water solubility Highly mobile in soils

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Waste is classified as hazardous. Dispose of in accordance with the European Directives

Products                        on waste and hazardous waste. Dispose of in accordance with local regulations.

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point.

Other Information              Do not flush to sewer. Waste codes should be assigned by the user based on the

application for which the product was used. Do not empty into drains. Large amounts will

affect pH and harm aquatic organisms. Do not let this chemical enter the environment.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport

UN-No                       UN3264

Proper Shipping Name              Corrosive liquid, acidic, inorganic, n.o.s.

Technical Shipping Name       Cerium(IV) sulfate, Sulfuric acid

Hazard Class                    8

Packing Group                                     II

IMDG/IMO

UN-No                       UN3264

Proper Shipping Name              Corrosive liquid, acidic, inorganic, n.o.s.

Technical Shipping Name       Cerium(IV) sulfate, Sulfuric acid

Hazard Class                    8

Packing Group                                     II

IATA

UN-No                       UN3264

Proper Shipping Name              Corrosive liquid, acidic, inorganic, n.o.s.

Technical Shipping Name       Cerium(IV) sulfate, Sulfuric acid

Hazard Class                    8

Packing Group                                     II

Special Precautions for User      No special precautions required

SECTION 15. REGULATORY INFORMATION

International Inventories

X = listed, China (IECSC), Europe (EINECS/ELINCS/NLP), U.S.A. (TSCA), Canada (DSL/NDSL), Philippines (PICCS), Japan (ENCS), Japan

(ISHL), Australia (AICS), Korea (KECL).

Component        The        List of    IECSC   EINECS   TSCA   DSL   PICCS  ENCS    ISHL    AICS    KECL

Inventory of dangerous

Hazardous  goods GB

Chemicals   12268 -

______________________________________________________________________________________________

Page 7

Page 8

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 8 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

(2015      2012

Edition)

Water                    -                 -        X      231-791-2    X      X      X      X             X     KE-35400

Sulfuric acid        X        X       X      231-639-5    X      X      X      X      X      X     KE-32570

Sulfuric acid,               -                 -        X      237-029-5    X      X      X      X      X      X     KE-05394

cerium(4+) salt (2:1)

National Regulations

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

Creation Date                      16-Apr-2010

Revision Date                     03-Aug-2026

Revision Summary            SDS sections updated, 2.

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Use of personal protective equipment, covering appropriate selection, compatibility, breakthrough thresholds, care, maintenance, fit

and standards.

First aid for chemical exposure, including the use of eye wash and safety showers.

Chemical incident response training.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Physical hazards              On basis of test data

Health Hazards                       Calculation method

Environmental hazards              Calculation method

Disclaimer

______________________________________________________________________________________________

Page 8

Page 9

ALFAA35650               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 9 / 9

Cerium(IV) sulfate, 0.1N Standardized Solution

______________________________________________________________________________________________

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of Safety Data Sheet

______________________________________________________________________________________________

Page 9