Cerium (IV) Sulfate Hydrate 1425-5 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/cerium-iv-sulfate-hydrate-prochem-en-5a3b095d89

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Cerium (IV) Sulfate Hydrate

PRODUCT NUMBER:  1425-5

CAS NUMBER:        95838-16-7

SYNONYMS:          Ceric sulfate hydrate

MANUFACTURER:

ProChem, Inc.                                         PHONE:       815-398-1788

826 Roosevelt Road                                     TOLL FREE:    800-795-8788

Rockford, IL  61109

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word       D

Danger

Hazard Statements

H303               May be harmful if swallowed

H314                 Causes severe skin burns and eye damage.

H335               May cause respiratory irritation

H410                   Very toxic to aquatic life with long lasting effects.

Precautionary Phrases

P260               Do not breathe dusts or mists.

P264                Wash skin thoroughly after handling.

P273                    Avoid release to the environment.

P301+P330+P331         IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P303+P361+P353+P310   IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower.

P304+P340+P310         IF INHALED: Remove person to fresh air and keep comfortable for breathing. Immediately call a POISON

CETNER/doctor.

P305+P351+P338+P310   IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy

to do. Continue rinsing. Immediately call a POISON CENTER/doctor.

P363                Wash contaminated clothing before reuse.

P501                   Dispose of contents/container to an approved waste disposal plant.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

CERIUM (IV) SULFATE TETRAHYDRATE                                                      PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes serious eye damage.

SKIN CONTACT: Causes severe burns to skin.

INHALATION:  Irritating to the mucous membranes and upper respiratory tract.

INGESTION: Immediate medical attention is required. Do not induce vomiting. Drink plenty of water.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Ce(SO4)!XH2O

Molecular Weight: 332.24 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Cerium (IV) Sulfate Hydrate                                                95838-16-7                                                    99.9

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Immediately wash affected area thoroughly with soap and water. Remove contaminated clothes if

necessary. Seek medical assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Do not induce vomiting. Give 2-4 cups of milk or water provided person is conscious and alert. Seek medical

assistance immediately.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use Carbon dioxide fire extinguisher. (Powder or Foam) as Water is ineffective. Any dry chemical, dry

sand measures that are appropriate to local circumstances and the surrounding environment.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Sulfur oxides and cerium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all required personal protective equipment when using this material. Ensure adequate ventilation

and use ventilation hood. Avoid the formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

CERIUM (IV) SULFATE TETRAHYDRATE                                                      PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

RESPIRATOR: Use an approved respirator.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White to off white solid

ODOR: None

MOLECULAR WEIGHT: 332.24 (anhy.)

BOILING POINT: No data available

MELTING POINT: 200° C (dec.).

SPECIFIC GRAVITY: 3.24.

VAPOR DENSITY: No data available

SOLUBILITY IN WATER: Soluble in small amounts. Hydrolyzes in large amounts of water.

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable. Moisture sensitive.

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: No data available

INCOMPATIBILITY: No data available.

DECOMPOSITION PRODUCTS: Sulfur oxides and cerium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

TOXICITY TO DAPHNIA AND OTHER AQUATIC INVERTEBRATES: Static test EC50 – Daphnia magna (water flea) – 1.9 mg/l – 48h

TOXICITY TO ALGAE: Static test ErC50 – Pseudokirchneriella subcapitata (green algea) – 0.658 mg/l – 72h. Static test NOEC-

Pseudokirchneriella subcapitata (green algae) – 0.555 mg/l – 72h.

Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

This product contains substances which are hazardous for the environment and will cause long-term effects.

Do not allow material to contaminate ground water system.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN3260.

Corrosive solid, acidic, inorganic, n.o.s.

Class 8

PG II

CERIUM (IV) SULFATE TETRAHYDRATE                                                      PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

TSCA – Listed – Toxic Substance Control Act, (40 CFR Part 710)

SARA 302: Not listed

SARA (TITLE 313): Not listed -

SARA 311/312: Acute Health Hazard

MASSACHUSETTS RIGHT TO KNOW: No components are subject to the Massachusetts Right to Know Act.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREVIOUSLY REVISED: 03/22

REVISED DATE: 08/26

REVISED SUMMARY: removed fax number. Updated section 2, section 15. All other details in this document about this product

remained unchanged. Updates made by EHS 08/10/206

EHS/SDS DEPT.

CERIUM (IV) SULFATE TETRAHYDRATE                                                      PAGE 4