Cerium oxide (CeO2) / diffraction intensity set SRM 674b SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/cerium-oxide-containing-x-ray-powder-diffraction-intensity-set-nist-srm-674b-sds-en-c8f213b4da

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

10 October 2017

SAFETY DATA SHEET PACKET

Product Identifier

SRM Number:   674b

SRM Name:    X-Ray Powder Diffraction Intensity Set (Quantitative Powder Diffraction Standard)

SRM Description:

A unit of SRM 674b consists of four bottles containing approximately 10 g of each oxide powder (ZnO,

TiO2, Cr2O3, and CeO2), bottled in an argon atmosphere. Attached are the individual Safety Data Sheets

for the materials in this Standard Reference Material (SRM).

SRM 674b Parts:

X-Ray Powder Diffraction Intensity Set ZnO (Zinc Oxide)

X-Ray Powder Diffraction Intensity Set TiO2 (Titanium Dioxide)

X-Ray Powder Diffraction Intensity Set Cr2O3 (Chromium Oxide)

X-Ray Powder Diffraction Intensity Set CeO2 (Cerium Oxide)

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                       +1-703-527-3887    (International)

Website: http://www.nist.gov/srm

Page 2

Date of Issue:

10 October 2017

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   674b

SRM Name:    X-Ray Powder Diffraction Intensity Set (Quantitative Powder Diffraction Standard)

SRM Part:      X-Ray Powder Diffraction Intensity Set ZnO (Zinc Oxide)

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended primarily for use as internal standards for quantitative X-ray

diffraction analysis.  This A unit of SRM 674b consists of four bottles containing approximately 10 g of each

oxide powder (ZnO, TiO2, Cr2O3, and CeO2), bottled in an argon atmosphere.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                       +1-703-527-3887    (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Note: This SDS is for ZnO; see additional SDSs for the classification for TiO2, Cr2O3, and CeO2.

Classification

Physical Hazard:       Not classified.

Health Hazard:        Not classified.

Label Elements

Symbol: No symbol/No pictogram

Signal Word: No signal word

Hazard Statement(s): Not applicable.

Precautionary Statement(s): Not applicable.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Zinc Oxide

Other Designations: Zinc white; Chinese white; flowers of zinc; zinc monoxide; zinc gelatin; white zinc; permanent

white; philosophers wool; zincoid; snow white; ZnO

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the NIST Certificate

of Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)                (%)

Zinc Oxide                         1314-13-2          215-222-5                 100

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 1 of 5

ZnO (Zinc Oxide)

Page 3

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:  If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes. Thoroughly clean and dry contaminated

clothing before reuse.

Eye Contact: Flush eyes with water for at least 15 minutes.  If necessary, seek medical attention.

Ingestion:  If a large amount is swallowed, get medical attention.

Most Important Symptoms/Effects, Acute and Delayed: May cause irritation.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek medical attention if needed.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Regular dry chemical, carbon dioxide, water, regular foam.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: None listed.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 0            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures: Use suitable protective equipment;

see Section 8, “Exposure Controls and Personal Protection”.

Methods and Materials for Containment and Clean up:  Collect spilled material in appropriate container for

disposal. Avoid generating dust. Clean up residue with a high-efficiency particulate filter vacuum.

7. HANDLING AND STORAGE

Safe Handling Precautions: Minimize dust generation. See Section 8, “Exposure Controls and Personal Protection”.

Storage:  It is recommended that the unused portion of the powder be stored in a tightly capped container such as the

original bottle or in a manner to protect against humidity. Store and handling in accordance with all current regulations

and standards. Keep separated from incompatible substances (See Section 10, “Stability and Reactivity”).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits:

ACGIH (TLV):    2 mg/m3 (TWA, respirable particulates)

10 mg/m3 (STEL, respirable particulates)

NIOSH (REL):    5 mg/m3 (TWA, dust and fume)

10 mg/m3 (STEL, fume)

15 mg/m3 (Ceiling, dust)

500 mg/m3 (IDLH)

OSHA (PEL):     5 mg/m3 (TWA, fume)

15 mg/m3 (TWA, total dust)

5 mg/m3 (TWA, respirable fraction)

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 2 of 5

ZnO (Zinc Oxide)

Page 4

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:   If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eye wash station should be readily

available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task being

performed and the risks involved and should be approved by a specialist before handling this product. Chemical-

resistant gloves should be worn at all times when handling chemicals.

9. PHYSICAL AND CHEMICAL PROPERTIES

Descriptive Properties:

Appearance                                  white or gray, powder

(physical state, color, etc.):

Molecular Formula:                   ZnO

Molar Mass (g/mol):                          81.37

Odor:                                          odorless

Odor threshold:                                not available

pH (solution):                                 6.95 to 7.37

Evaporation rate:                              not applicable

Melting point/freezing point (ºC):            1975 (3587 ºF)

Relative Density (g/mL):                        5.6

Vapor Pressure (mmHg):                       0.0 at 21 °C

Vapor Density (air = 1):                   >1

Viscosity (cP):                                 not applicable

Solubility(ies):                                water soluble (1.6 ppm at 29 °C);

soluble in dilute acetic acid, mineral acids,

ammonia, ammonium chloride solutions,

ammonium salt solutions, fixed alkali

hydroxide solutions, strong alkali

Partition coefficient (n-octanol/water):         not available

Particle Size:                                   not available

Thermal Stability Properties:

Autoignition Temperature (ºC):                not applicable

Thermal Decomposition (ºC):                  not available

Initial boiling point and boiling range (ºC):     not applicable

Explosive Limits, LEL (Volume %):           not applicable

Explosive Limits, UEL (Volume %):           not applicable

Flash Point (ºC):                               not applicable

Flammability (solid, gas):                      not available

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:     X       Stable              Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Avoid generating dust.

Incompatible Materials: Halo carbons, combustible materials, metals, acids, oxidizing materials.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 3 of 5

ZnO (Zinc Oxide)

Page 5

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will produce zinc, oxides of zinc.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation             Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Exposure may cause irritation.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation: Acute: irritation; inhaled fumes may cause metal fume fever, difficulty breathing; chronic: nausea

and liver damage.

Skin Contact:  Acute: low potential for skin irritation, although it can alter skin pigmentation; chronic: skin

disorders.

Eye Contact: Acute: mechanical irritation, redness, and pain; chronic: no information available.

Ingestion: Acute: nausea, diarrhea, constipation; chronic: no information available.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified.

Rat, Oral LD50: >5000 mg/kg

Mouse, Inhalation LC50: 2500 mg/m3

Skin Corrosion/Irritation: Not classified.

Rabbit, Skin (mild): 500 mg (24 h)

Serious Eye damage/Eye irritation: Not classified.

Rabbit, Eyes (mild): 500 mg (24 h)

Respiratory Sensitization: Not classified; no data available.

Skin Sensitization: Not classified; no data available.

Germ Cell Mutagenicity: Not classified; no data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen               Yes       X   No

Zinc oxide is not listed by IARC, NTP or OSHA as a carcinogen.

Reproductive Toxicity: Not classified.

Rat, Oral TDLo: 6846 mg/kg (pregnant 1 d to 22 d)

Specific Target Organ Toxicity, Single Exposure: Not classified; no data available.

Specific Target Organ Toxicity, Repeated Exposure: Not classified; no data available.

Aspiration Hazard: Not classified; no data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: Not regulated by DOT or IATA.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 4 of 5

ZnO (Zinc Oxide)

Page 6

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4): Not regulated.

SARA Title III Section 302 (40 CFR 355.30): Not regulated.

SARA Title III Section 304 (40 CFR 355.40): Not regulated.

SARA Title III Section 313 (40 CFR 372.65): 1.0 % de minimis concentration (related to Zinc compounds).

OSHA Process Safety (29 CFR 1910.119): Not regulated.

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:        No.

CHRONIC HEALTH:     No.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations:

California Proposition 65: Not listed.

U.S. TSCA Inventory: Listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations:

WHMIS Information: Not provided for this material.

16. OTHER INFORMATION

Issue Date: 10 October 2017

Sources:    ChemAdvisor, Inc., MSDS Zinc Oxide, 09 December 2015.

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NIOSH        National Institute for Occupational Safety and

Hygienists                                                        Health

ALI          Annual Limit on Intake                       NIST          National Institute of Standards and Technology

CAS         Chemical Abstracts Service                NRC          Nuclear Regulatory Commission

CEN         European Committee for Standardization         NTP           National Toxicology Program

CERCLA     Comprehensive Environmental Response,        OSHA         Occupational Safety and Health Administration

Compensation, and Liability Act

CFR         Code of Federal Regulations                  PEL           Permissible Exposure Limit

CPSU        Coal Mine Dust Personal Sample Unit          RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                REL         Recommended Exposure Limit

EC50          Effective Concentration, 50 %             RM           Reference Material

EINECS      European Inventory of Existing Commercial      RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

ISO            International Organization for Standardization      STEL          Short Term Exposure Limit

LC50          Lethal Concentration, 50 %                   TDLo         Toxic Dose Low

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NFPA         National Fire Protection Association          TWA        Time Weighted Average

MSHA       Mine Safety and Health Administration          UEL         Upper Explosive Limit

WHMIS      Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 5 of 5

ZnO (Zinc Oxide)

Page 7

Date of Issue:

10 October 2017

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   674b

SRM Name:    X-Ray Powder Diffraction Intensity Set (Quantitative Powder Diffraction Standard)

SRM Part:      X-Ray Powder Diffraction Intensity Set TiO2 (Titanium Dioxide)

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended primarily for use as internal standards for quantitative X-ray

diffraction analysis. A unit of SRM 674b consists of four bottles containing approximately 10 g of each oxide

powder (ZnO, TiO2, Cr2O3, and CeO2), bottled in an argon atmosphere.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                       +1-703-527-3887    (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Note: This SDS is for TiO2; see additional SDSs for the classification for ZnO, Cr2O3, and CeO2.

Classification

Physical Hazard:       Not classified.

Health Hazard:          Carcinogenicity   Category 2

Label Elements

Symbol

Signal Word

WARNING

Hazard Statement(s):

H351           Suspected of causing cancer (inhalation).

Precautionary Statement(s):

P201           Obtain special instructions before use.

P202        Do not handle until all safety precautions have been read and understood.

P281          Use personal protective equipment as required.

P308+P313        If exposed or concerned: Get medical attention.

P405            Store locked up.

P501           Dispose of contents and container in accordance with local regulations.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 1 of 6

TiO2 (Titanium Dioxide)

Page 8

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Titanium Dioxide

Other Designations: Titanium oxide; titanium dioxide: TiO2; titanium peroxide; titanic oxide; brookite; rutile

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the NIST Certificate

of Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)                (%)

Titanium Dioxide                   13463-67-7          236-675-5                 100

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:  If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes. Thoroughly clean and dry contaminated

clothing before reuse.

Eye Contact: Flush eyes with water for at least 15 minutes.  If necessary, seek medical attention.

Ingestion:  If a large amount is swallowed, get medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Irritation, cancer inhalation hazard.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek medical attention if needed.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing agents appropriate for surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: None listed.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 1            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures: Use suitable protective equipment;

see Section 8, “Exposure Controls and Personal Protection”.

Methods and Materials for Containment and Clean up:  Collect spilled material in appropriate container for

disposal. Avoid generating dust.

7. HANDLING AND STORAGE

Safe Handling Precautions: Minimize dust generation. See Section 8, “Exposure Controls and Personal Protection”.

Storage:  It is recommended that the unused portion of the powder be stored in a tightly capped container such as the

original bottle or in a manner to protect against humidity. Store and handling in accordance with all current regulations

and standards. Keep separated from incompatible substances (See Section 10, “Stability and Reactivity”).

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 2 of 6

TiO2 (Titanium Dioxide)

Page 9

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits:

ACGIH (TLV):  10 mg/m3 (TWA)

5000 mg/m3 (IDLH)

NIOSH (REL):  No occupational exposure limits available.

OSHA (PEL):    15 mg/m3 (TWA, total dust)

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:   If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eye wash station should be readily

available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task being

performed and the risks involved and should be approved by a specialist before handling this product. Chemical-

resistant gloves should be worn at all times when handling chemicals.

9. PHYSICAL AND CHEMICAL PROPERTIES

Descriptive Properties:

Appearance                                        colorless, white or black, powder

(physical state, color, etc.):

Molecular Formula:                        TiO2

Molar Mass (g/mol):                          79.88

Odor:                                          odorless

Odor threshold:                                not available

pH (solution):                                    neutral (10 % suspension)

Evaporation rate:                              not applicable

Melting point/freezing point (ºC):            1825 to 1850 (3317 ºF to 3362 ºF)

Relative Density                              3.84 to 4.26

as Specific Gravity (water = 1):

Vapor Pressure (mmHg):                      not applicable

Vapor Density (air = 1):                        not applicable

Viscosity (cP):                                 not applicable

Solubility(ies):                                  soluble in hot concentrated sulfuric acid,

hydrofluoric acid, alkali;

insoluble in water, hydrochloric acid, nitric

acid, dilute sulfuric acid.

Partition coefficient (n-octanol/water):         not available

Particle Size:                                   not available

Thermal Stability Properties:

Autoignition Temperature (ºC):                not applicable

Thermal Decomposition (ºC):                  not available

Initial boiling point and boiling range (ºC):    2500 to 3000 (4532 ºF to 5432 ºF)

Explosive Limits, LEL (Volume %):           not applicable

Explosive Limits, UEL (Volume %):           not applicable

Flash Point (ºC):                               not applicable

Flammability (solid, gas):                      not available

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 3 of 6

TiO2 (Titanium Dioxide)

Page 10

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:     X       Stable              Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Avoid generating dust.

Incompatible Materials: Metals.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will produce oxides of titanium.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin                    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Exposure may cause irritation;

inhalation may cause cancer.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation: Acute: irritation, cough; chronic: same as acute, difficulty breathing, may cause cancer.

Skin Contact: Acute: no information available; chronic: irritation.

Eye Contact: No information available.

Ingestion: No information available.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified.

Rat, Oral LD50: >10 000 mg/kg

Skin Corrosion/Irritation: Not classified.

Human, Skin intermittent (mild): 300 ug (3 d)

Serious Eye damage/Eye irritation: Not classified; no data available.

Respiratory Sensitization: Not classified; no data available.

Skin Sensitization: Not classified; no data available.

Germ Cell Mutagenicity: Not classified; no data available.

Carcinogenicity: Category 2.

Listed as a Carcinogen/Potential Carcinogen       X   Yes            No

Titanium dioxide is listed by IARC as Group 2B (possibly carcinogenic to humans). Titanium dioxide is

not listed by NTP or OSHA as a carcinogen.

Reproductive Toxicity: Not classified; no data available.

Specific Target Organ Toxicity, Single Exposure: Not classified; no data available.

Specific Target Organ Toxicity, Repeated Exposure: Not classified; no data available.

Aspiration Hazard: Not classified; no data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 4 of 6

TiO2 (Titanium Dioxide)

Page 11

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: Not regulated by DOT or IATA.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4): Not regulated.

SARA Title III Section 302 (40 CFR 355.30): Not regulated.

SARA Title III Section 304 (40 CFR 355.40): Not regulated.

SARA Title III Section 313 (40 CFR 372.65): Not regulated.

OSHA Process Safety (29 CFR 1910.119): Not regulated.

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:        No.

CHRONIC HEALTH:      Yes.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations:

California Proposition 65: WARNING! This product contains a chemical (titanium dioxide) known to the state of

California to cause cancer.

U.S. TSCA Inventory: Listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations:

WHMIS Information: Not provided for this material.

16. OTHER INFORMATION

Issue Date: 10 October 2017

Sources:    ChemAdvisor, Inc., MSDS Titanium Dioxide, 09 December 2015.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 5 of 6

TiO2 (Titanium Dioxide)

Page 12

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NIOSH        National Institute for Occupational Safety and

Hygienists                                                        Health

ALI          Annual Limit on Intake                       NIST          National Institute of Standards and Technology

CAS         Chemical Abstracts Service                NRC          Nuclear Regulatory Commission

CEN         European Committee for Standardization         NTP           National Toxicology Program

CERCLA     Comprehensive Environmental Response,        OSHA         Occupational Safety and Health Administration

Compensation, and Liability Act

CFR         Code of Federal Regulations                  PEL           Permissible Exposure Limit

CPSU        Coal Mine Dust Personal Sample Unit          RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                REL         Recommended Exposure Limit

EC50          Effective Concentration, 50 %             RM           Reference Material

EINECS      European Inventory of Existing Commercial      RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

ISO            International Organization for Standardization      STEL          Short Term Exposure Limit

LC50          Lethal Concentration, 50 %                   TDLo         Toxic Dose Low

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NFPA         National Fire Protection Association          TWA        Time Weighted Average

MSHA       Mine Safety and Health Administration          UEL         Upper Explosive Limit

WHMIS      Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 6 of 6

TiO2 (Titanium Dioxide)

Page 13

Date of Issue:

10 October 2017

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   674b

SRM Name:    X-Ray Powder Diffraction Intensity Set (Quantitative Powder Diffraction Standard)

SRM Part:      X-Ray Powder Diffraction Intensity Set Cr2O3 (Chromium Oxide)

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended primarily for use as internal standards for quantitative X-ray

diffraction analysis. A unit of SRM 674b consists of four bottles containing approximately 10 g of each oxide

powder (ZnO, TiO2, Cr2O3, and CeO2), bottled in an argon atmosphere.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                       +1-703-527-3887    (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Note: This SDS is for Cr2O3; see additional SDSs for the classification for ZnO, TiO2, and CeO2.

Classification

Physical Hazard:       Not classified.

Health Hazard:        Not classified.

Label Elements

Symbol: No symbol/No pictogram

Signal Word: No signal word

Hazard Statement(s): Not applicable.

Precautionary Statement(s): Not applicable.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Chromium oxide

Other Designations: Chromium (III) oxide; chromic oxide; dichromium trioxide; chrome oxide; anadomis green;

casalis green oxide pigment; chromia; chromium sesquioxide; green rouge; Cr2O3

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the NIST Certificate

of Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)                (%)

Chromium Oxide                    1308-38-9          215-160-9                 100

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 1 of 5

Cr2O3 (Chromium Oxide)

Page 14

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:  If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes. Thoroughly clean and dry contaminated

clothing before reuse.

Eye Contact: Flush eyes with water for at least 15 minutes.  If necessary, seek medical attention.

Ingestion:  If a large amount is swallowed, get medical attention.

Most Important Symptoms/Effects, Acute and Delayed: May cause irritation and allergic reactions.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek medical attention if needed.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  Dust/air mixtures may ignite or explode.  See Section 9,

“Physical and Chemical Properties” for flammability properties.

Extinguishing Media:

Suitable: Use extinguishing agents appropriate for surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: None listed.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 1            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures: Use suitable protective equipment;

see Section 8, “Exposure Controls and Personal Protection”.

Methods and Materials for Containment and Clean up:  Collect spilled material in appropriate container for

disposal. Avoid generating dust.

7. HANDLING AND STORAGE

Safe Handling Precautions: Minimize dust generation. See Section 8, “Exposure Controls and Personal Protection”.

Storage:  It is recommended that the unused portion of the powder be stored in a tightly capped container such as the

original bottle or in a manner to protect against humidity. Store and handling in accordance with all current regulations

and standards. Keep separated from incompatible substances (See Section 10, “Stability and Reactivity”).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits: [as Cr, related to Chromium(III) compounds]

ACGIH (TLV):   No occupational exposure limits established.

NIOSH (REL):     0.5 mg/m3 (TWA)

25 mg/m3 (IDLH)

OSHA (PEL):      0.5 mg/m3 (TWA)

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 2 of 5

Cr2O3 (Chromium Oxide)

Page 15

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:   If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eye wash station should be readily

available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task being

performed and the risks involved and should be approved by a specialist before handling this product. Chemical-

resistant gloves should be worn at all times when handling chemicals.

9. PHYSICAL AND CHEMICAL PROPERTIES

Descriptive Properties:

Appearance                                    green powder

(physical state, color, etc.):

Molecular Formula:                           Cr2O3

Molar Mass (g/mol):                           151.99

Odor:                                             odorless

Odor threshold:                                  not available

pH (solution):                                     7.5 at 5 %

Evaporation rate:                                not applicable

Melting point/freezing point (ºC):              2435 (4415 ºF)

Relative Density as Specific Gravity (water = 1):   5.21

Vapor Pressure (mmHg):                        not available

Vapor Density (air = 1):                          not available

Viscosity (cP):                                    not applicable

Solubility(ies):                                     insoluble in water, acids, alkali, alcohol, acetone

Partition coefficient (n-octanol/water):           not available

Particle Size:                                     not available

Thermal Stability Properties:

Autoignition Temperature (ºC):                  not applicable

Thermal Decomposition (ºC):                    not available

Initial boiling point and boiling range (ºC):      4000 (7232 ºF)

Explosive Limits, LEL (Volume %):              not applicable

Explosive Limits, UEL (Volume %):             not applicable

Flash Point (ºC):                                 not applicable

Flammability (solid, gas):                        not available

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:     X       Stable              Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Avoid generating dust.

Incompatible Materials: Halogens, metals, combustible materials, oxidizing materials, metal carbide.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will produce oxides of chromium.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 3 of 5

Cr2O3 (Chromium Oxide)

Page 16

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin                    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Exposure may cause irritation

and allergic reactions.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation:  Acute: irritation, allergic reactions, chest pain; chronic: same as acute, lung damage, kidney

damage, and liver damage.

Skin Contact: Acute and chronic: irritation and allergic reactions.

Eye Contact: Acute: irritation; chronic: no information available.

Ingestion: Acute: nausea, diarrhea, vomiting; chronic: no information available.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified; no data available.

Skin Corrosion/Irritation: Not classified; no data available.

Serious Eye damage/Eye irritation: Not classified; no data available.

Respiratory Sensitization: Not classified; no data available.

Skin Sensitization: Not classified; no data available.

Germ Cell Mutagenicity: Not classified; no data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen              Yes       X   No

Chromium (III) compounds are listed by IARC as Group 3 (not classifiable). Chromium oxide is not listed

by NTP or OSHA as a carcinogen.

Reproductive Toxicity: Not classified; no data available.

Specific Target Organ Toxicity, Single Exposure: Not classified; no data available.

Specific Target Organ Toxicity, Repeated Exposure: Not classified; no data available.

Aspiration Hazard: Not classified; no data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: Not regulated by DOT or IATA.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4): Not regulated.

SARA Title III Section 302 (40 CFR 355.30): Not regulated.

SARA Title III Section 304 (40 CFR 355.40): Not regulated.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 4 of 5

Cr2O3 (Chromium Oxide)

Page 17

SARA Title III Section 313 (40 CFR 372.65):    1.0 % de minimis concentration; Chemical Category N090,

(related to Chromium(III) compounds).

OSHA Process Safety (29 CFR 1910.119): Not regulated.

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:        No.

CHRONIC HEALTH:     No.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations:

California Proposition 65: Not listed.

U.S. TSCA Inventory: Listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations:

WHMIS Information: Not provided for this material.

16. OTHER INFORMATION

Issue Date: 10 October 2017

Sources:    ChemAdvisor, Inc., MSDS Chromium(III) Oxide, 09 December 2015.

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NIOSH        National Institute for Occupational Safety and

Hygienists                                                        Health

ALI          Annual Limit on Intake                       NIST          National Institute of Standards and Technology

CAS         Chemical Abstracts Service                NRC          Nuclear Regulatory Commission

CEN         European Committee for Standardization         NTP           National Toxicology Program

CERCLA     Comprehensive Environmental Response,        OSHA         Occupational Safety and Health Administration

Compensation, and Liability Act

CFR         Code of Federal Regulations                  PEL           Permissible Exposure Limit

CPSU        Coal Mine Dust Personal Sample Unit          RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                REL         Recommended Exposure Limit

EC50          Effective Concentration, 50 %             RM           Reference Material

EINECS      European Inventory of Existing Commercial      RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

ISO            International Organization for Standardization      STEL          Short Term Exposure Limit

LC50          Lethal Concentration, 50 %                   TDLo         Toxic Dose Low

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NFPA         National Fire Protection Association          TWA        Time Weighted Average

MSHA       Mine Safety and Health Administration          UEL         Upper Explosive Limit

WHMIS      Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 5 of 5

Cr2O3 (Chromium Oxide)

Page 18

Date of Issue:

10 October 2017

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   674b

SRM Name:    X-Ray Powder Diffraction Intensity Set (Quantitative Powder Diffraction Standard)

SRM Part:      X-Ray Powder Diffraction Intensity Set CeO2 (Cerium Oxide)

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended primarily for use as internal standards for quantitative X-ray

diffraction analysis. A unit of SRM 674b consists of four bottles containing approximately 10 g of each oxide

powder (ZnO, TiO2, Cr2O3, and CeO2), bottled in an argon atmosphere.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                       +1-703-527-3887    (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Note: This SDS is for CeO2; see additional SDSs for the classification for ZnO, TiO2, and Cr2O3.

Classification

Physical Hazard:       Not classified.

Health Hazard:         Acute Toxicity, Oral        Category 4

Acute Toxicity, Inhalation   Category 4

Label Elements

Symbol

Signal Word

WARNING

Hazard Statement(s):

H302         Harmful if swallowed.

H332         Harmful if inhaled.

Precautionary Statement(s):

P261         Avoid breathing dust.

P264        Wash hands thoroughly.

P270        Do not eat, drink or smoke when using this product.

P271         Use in a well-ventilated area.

P301+P312      If swallowed: Call a doctor if you feel unwell.

P330          Rinse mouth.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 1 of 6

CeO2 (Cerium Oxide)

Page 19

P304+P340      If inhaled: Remove person to fresh air and keep comfortable for breathing.

P312              If inhaled: Call a doctor if you feel unwell.

P501          Dispose of contents and container in accordance with local regulations.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Cerium Oxide

Other Designations: Cerium (IV) oxide; cerium dioxide; ceria; cerium oxide; CeO2; ceric oxide

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the NIST Certificate

of Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)                (%)

Cerium Oxide                       1306-38-3          215-150-4                 100

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:  If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes. Thoroughly clean and dry contaminated

clothing before reuse.

Eye Contact: Flush eyes with water for at least 15 minutes.  If necessary, seek medical attention.

Ingestion:  If a large amount is swallowed, get medical attention.

Most Important Symptoms/Effects, Acute and Delayed: May cause irritation, nausea, headache, lung damage.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek medical attention if needed.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing agents appropriate for surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: None listed.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 1            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures: Use suitable protective equipment;

see Section 8, “Exposure Controls and Personal Protection”.

Methods and Materials for Containment and Clean up:  Collect spilled material in appropriate container for

disposal. Avoid generating dust. Clean up residue with a high-efficiency particulate filter vacuum.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 2 of 6

CeO2 (Cerium Oxide)

Page 20

7. HANDLING AND STORAGE

Safe Handling Precautions: Minimize dust generation. See Section 8, “Exposure Controls and Personal Protection”.

Storage:  It is recommended that the unused portion of the powder be stored in a tightly capped container such as the

original bottle or in a manner to protect against humidity. Store and handling in accordance with all current regulations

and standards. Keep separated from incompatible substances (See Section 10, “Stability and Reactivity”).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits: This material is a particulate matter and adequate inhalation/respiratory protection should be used

to minimize exposure. No occupational exposure limits have been established for cerium oxide. The exposure limits

for Particulates Not Otherwise Regulated are applicable.

OSHA (PEL): 15 mg/m3 (TWA, total particulates not otherwise regulated)

5 mg/m3 (TWA, respirable particulates not otherwise regulated)

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:   If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eye wash station should be readily

available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task being

performed and the risks involved and should be approved by a specialist before handling this product. Chemical-

resistant gloves should be worn at all times when handling chemicals.

9. PHYSICAL AND CHEMICAL PROPERTIES

Descriptive Properties:

Appearance                                  white or yellow, powder

(physical state, color, etc.):

Molecular Formula:                      CeO2

Molar Mass (g/mol):                         172.12

Odor:                                          not available

Odor threshold:                                not available

pH (solution):                                  not available

Evaporation rate:                              not applicable

Melting point/freezing point (ºC):            2600 (4712 ºF)

Relative Density                              7.132 at 23 °C

as specific gravity (water = 1):

Vapor Pressure (mmHg):                      not available

Vapor Density (air = 1):                        not available

Viscosity (cP):                                 not available

Solubility(ies):                                  water: insoluble;

soluble in sulfuric acid, nitric acid;

insoluble in dilute acids

Partition coefficient (n-octanol/water):         not available

Particle Size:                                   not available

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 3 of 6

CeO2 (Cerium Oxide)

Page 21

Thermal Stability Properties:

Autoignition Temperature (ºC):                not applicable

Thermal Decomposition (ºC):                  not available

Initial boiling point and boiling range (ºC):     not applicable

Explosive Limits, LEL (Volume %):           not applicable

Explosive Limits, UEL (Volume %):           not applicable

Flash Point (ºC):                               not applicable

Flammability (solid, gas):                      not available

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:     X       Stable              Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Avoid generating dust.

Incompatible Materials: No data available.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will produce oxides of cerium.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Exposure may cause irritation,

nausea, headache, lung damage.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation: Acute: nausea, headache, lung damage; chronic: itching, sensitivity to heat, and lung damage.

Skin Contact: Acute: hair loss; chronic: no information available.

Eye Contact: Acute: irritation and eye damage; chronic: no information available.

Ingestion: Acute and chronic: no information available.

Numerical Measures of Toxicity:

Acute Toxicity: Category 4, Oral; Category 4, Inhalation.

Rat, Oral LD50: >1000 mg/kg

Rat, Inhalation LC50: >2.01 mg/L (4 h)

Rat, Dermal LD50: >2000 mg/kg

Skin Corrosion/Irritation: Not classified.

Serious Eye damage/Eye irritation: Not classified.

Respiratory Sensitization: Not classified; no data available.

Skin Sensitization: Not classified; no data available.

Germ Cell Mutagenicity: Not classified; no data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen              Yes       X   No

Cerium oxide is not listed by IARC, NTP or OSHA as a carcinogen.

Reproductive Toxicity: Not classified; no data available.

Specific Target Organ Toxicity, Single Exposure: Not classified; no data available.

Specific Target Organ Toxicity, Repeated Exposure: Not classified; no data available.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 4 of 6

CeO2 (Cerium Oxide)

Page 22

Aspiration Hazard: Not classified; no data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: Not regulated by DOT or IATA.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4): Not regulated.

SARA Title III Section 302 (40 CFR 355.30): Not regulated.

SARA Title III Section 304 (40 CFR 355.40): Not regulated.

SARA Title III Section 313 (40 CFR 372.65): Not regulated.

OSHA Process Safety (29 CFR 1910.119): Not regulated.

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:         Yes.

CHRONIC HEALTH:     No.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations:

California Proposition 65: Not listed.

U.S. TSCA Inventory: Listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations:

WHMIS Information: Not provided for this material.

16. OTHER INFORMATION

Issue Date: 10 October 2017

Sources:    ChemAdvisor, Inc., MSDS Ceric Oxide, 09 December 2015.

CDC; NIOSH; NIOSH Pocket Guide to Chemical Hazards; Department of Health and Human Services

(DHHS), Centers for Disease Control and Prevention (CDC), National Institute for Safety and Health;

Particulates      Not      Otherwise       Regulated,      4 May 2011;       available        at

http://www.cdc.gov/niosh/npg/npgd0480.html (accessed Oct 2017).

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 5 of 6

CeO2 (Cerium Oxide)

Page 23

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NIOSH        National Institute for Occupational Safety and

Hygienists                                                        Health

ALI          Annual Limit on Intake                       NIST          National Institute of Standards and Technology

CAS         Chemical Abstracts Service                NRC          Nuclear Regulatory Commission

CEN         European Committee for Standardization         NTP           National Toxicology Program

CERCLA     Comprehensive Environmental Response,        OSHA         Occupational Safety and Health Administration

Compensation, and Liability Act

CFR         Code of Federal Regulations                  PEL           Permissible Exposure Limit

CPSU        Coal Mine Dust Personal Sample Unit          RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                REL         Recommended Exposure Limit

EC50          Effective Concentration, 50 %             RM           Reference Material

EINECS      European Inventory of Existing Commercial      RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

ISO            International Organization for Standardization      STEL          Short Term Exposure Limit

LC50          Lethal Concentration, 50 %                   TDLo         Toxic Dose Low

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NFPA         National Fire Protection Association          TWA        Time Weighted Average

MSHA       Mine Safety and Health Administration          UEL         Upper Explosive Limit

WHMIS      Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 674b                        Part: X-Ray Powder Diffraction Intensity Set                      Page 6 of 6

CeO2 (Cerium Oxide)