Copper-65 SDS

Publisher: Neonest AB

Source: https://resds.com/document/copper-65-isotope-sds-cu-65-65cu-copper-ii-oxide-enriched-copper-ii-oxide-20d9981eb2

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Copper (II) Oxide, Enriched Copper (II) Oxide.

Molecular weight:   79.545

Chemical formula:   CuO

Synonyms:   None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Aquatic Acute (Category 1)   H400

Aquatic Chronic (Category 1)   H410

Label elements:

Hazard pictograms:

Signal Words:   Warning.

Page 2

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 2/9

Hazard Statements:

H400 Very toxic to aquatic life.

H410 Very toxic to aquatic life with long lasting effects.

Precautionary Statements:

P273 Avoid release to the environment.

P391 Collect spillage.

P501 Dispose of contents/container in accordance with local and national regulations.

Other hazards:   None known.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Copper oxide

1317-38-0

1344-70-3   79.545   >99%   Aquatic Acute 1   H400

Aquatic Chronic 1H410

215-269-1

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

In case of inhalation:   Remove person to fresh air, keep warm and quiet, give oxygen if breathing is difficult, and seek

medical attention if symptoms persist.

In case of skin contact:   Remove any contaminated clothing, brush material off skin, flush with running water, and

wash carefully with soap and water. Seek medical attention if symptoms persist.

In case of eye contact:   Flush eyes with lukewarm water, lifting upper and lower eyelids, for at least 15 minutes. Seek

medical attention if symptoms persist.

In case of ingestion:   Give 1-2 glasses of milk or water and induce vomiting; seek immediate medical attention. Never

induce vomiting or give anything by mouth to an unconscious person.

Most important symptoms and effects, both acute and delayed:

Inhalation:   May be harmful if inhaled; may cause respiratory tract irritation.

Eyes:   May cause eye irritation.

Skin contact:   May be harmful if absorbed through skin; may cause skin irritation.

Ingestion:   Toxic if swallowed.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Use suitable extinguishing media for surrounding material and type of fire.

Page 3

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 3/9

Unsuitable extinguishing media:   None known.

Special hazards arising from the mixture:   May ignite on contact with dichloromethylsilane, hydrogen sulphide or

hydrogen trisulfide. May react violently with hydrazine, PN2H, titanium or zirconium. May have a violent exothermic

reaction with boron when heated. May explode when heated with powdered alu mini m   ani ini m   er   orate

rogen or   t a i   an   ri e   a   e   o e at   C with cesium acetylene carbide.

Advice for fire-fighters:   Wear self-contained breathing apparatus and protective clothing for firefighting.

Further information:   Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas.

Ensure adequate ventilation. Avoid breathing dust.

Environmental precautions:   Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up:   Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable,

closed containers for disposal.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Avoid contact with skin and eyes. Avoid formation of dust or aerosols. Provide appropriate

exhaust ventilation at places where dust is formed. Implement engineering and work practice controls to reduce and

maintain concentration of exposure at low levels. Do not blow dust off clothing or skin with compressed air. Maintain

eyewash capable of sustained flushing, safety drench shower and facilities for washing.

Hygiene measures:   Do not eat, drink or smoke when using this product. Use good housekeeping and sanitation

practices. Do not use tobacco or food in work area.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers:   Store in cool, dry, well-ventilated area. Keep tightly sealed when not

in use.

Section 8: Exposure Control s/ Personal Protection

Control parameters:

Occupational exposure limits:   Contains no substances with occupational exposure limit values.

Exposure controls:

Appropriate engineering controls:   Safety shower and eye bath, mechanical exhaust required.

Page 4

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 4/9

Individual protection measures, such as personal protective equipment:

Eye/face protection:   Use safety glasses with side-shields conforming to EN166. Use equipment for eye protection

tested and approved under appropriate government standards such as NIOSH (US) or EN166 (EU).

Hand protection:   Use chemical resistant gloves. Examples of preferred glove barrier materials include: Butyl rubber,

Neoprene, Nitrile/butadiene rubber, Polyethylene, Ethyl vinyl alcohol laminate, polyvinyl alcohol, Polyvinyl chloride.

Body protection:   Wear protective clothing as appropriate.

Respiratory protection:   For nuisance exposures, use type P95 (US) or type P1 (EU EN 143) particle respirator. For

higher-level protection, use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges. Use respirators

and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   Black.

Odour:   Odourless.

Odour threshold:   No data available.

Molecular Weight:   79.545

pH (concentration):   No data available.

Melting point/range (° C):   1326   °C

Boiling point/range (° C):   2000   °C

Freezing point   (°C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour pressure ( 20 ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   6.4 g/cm 3

Water solubility (g/L ) at 20 ° C:   Insoluble.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Sect ion 10: Stability and Reactivity

Reactivity:   No dangerous reaction known under conditions of normal use.

Chemical stability:   Stable under normal conditions.

Possibility of hazardous reactions:   No dangerous reaction known under conditions of normal use.

Conditions to avoid:   None known .

Incompatible   materials:   Cesium   acetylene   carbide,   hydrazine,   PN2H,   titanium,   zirconium,   chloromethylsilane,

hydrogen sulphide, hydrogen trisulfide, powdered aluminium, aniline perchlorate, hydrogen magnesium, phthalic

anhydride or boron.

Page 5

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 5/9

Hazardous decomposition products:   None known.

Section 11: Toxicological Information

Information on toxicological effects:

Acute Toxicity:

Oral LD 50 : Rat, 470 mg/kg

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

May cause dermatitis.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:

Toxicity to fish: LC 50 ,   Oncorhynchus mykiss , 25.4 mg/l, 96 hours.

Toxicity to daphnia and other aquatic invertebrates: EC 50   Daphnia magna,   0.011 - 0.039 mg/l, 48 hours.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   Not relevant.

Other adverse effects:   An environmental hazard cannot be excluded in the event of unprofessional handling or

disposal. Very toxic to aquatic life, with long-lasting effects.

Section 13: Disposal Considerations

Waste treatment methods:   Contact a licensed professional waste disposal service to dispose of this material. Dissolve

or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and

scrubber. Offer surplus and non-recyclable solutions to a licensed disposal company.

Section 14: Transport Information

Page 6

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 6/9

IMDG:

Proper Shipping Name:   ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Copper oxide).

Hazard Class : 9

UN Number:   3077

Packing Group:   III

Hazard Labels:

IATA:

Proper Shipping Name:   ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Copper oxide).

Hazard Class : 9

UN Number:   3077

Packing Group:   III

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Page 7

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 7/9

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

OSHA Hazards:   Toxic by ingestion.

SARA 302 Components:   No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components:   The following components are subject to reporting levels established by SARA Title III, Section

313: Copper Oxide, CAS No. 1317-38-0, Revision Date 2007-07-01.

SARA 311/312 Hazards:   Acute Health Hazard.

Massachusetts Right To Know Components:   No components are subject to the Massachusetts Right to Know Act.

Pennsylvania Right To Know Components:   Copper Oxide, CAS No. 1317-38-0, Revision Date 2007-07-01.

New Jersey Right To Know Components:   Copper Oxide, CAS No. 1317-38-0, Revision Date 2007-07-01.

California Prop. 65 Components:   This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

National regulations:   Follow national regulation for work with chemical agents.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

Page 8

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 8/9

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

Page 9

Safety Data Sheet for Copper (II) Oxide,

Enriched Copper (II) Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 12-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Copper (II) Oxide, Enriched Copper (II) Oxide   https://www.buyisotope.com   Page 9/9

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefä r   ngsk assen (German : Water Hazar   C asses)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H400 Very toxic to aquatic life.

H410 Very toxic to aquatic life with long lasting effects.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.