Depleted Zinc Acetate Dihydrate - Depleted in Zn-64 Isotope SDS

Publisher: Neonest AB

Source: https://resds.com/document/depleted-zinc-acetate-dihydrate-depleted-in-zn-64-isotope-sds-7b04bc3580

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Zinc Acetate Dihydrate (DZA).

Molecular weight:   219.51

Chemical formula:   (CH 3 COO) 2   Zn · 2H 2 O or C 4 H 6 O 4 Zn · 2H 2 O   or   C 4 H 10 O 6 Zn

Synonyms:   None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Acute Toxicity, Oral (Category 4)   H302

Eye Damage (Category 1)   H318

Aquatic Chronic (Category 2)   H411

Label elements:

Hazard pictograms:

Signal Words:   Danger.

Page 2

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 2/9

Hazard Statements:

H302 Harmful if swallowed.

H318 Causes serious eye damage.

H411 Toxic to aquatic life with long lasting effects.

Precautionary Statements:

P270 Do not eat, drink or smoke when using this product.

P273 Avoid release to the environment.

P280 Wear eye protection/face protection.

P301 + P330 + P331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and

easy to do. Continue rinsing.

P310 Immediately call a POISON CENTER/doctor.

P391 Collect spillage.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Concentration

% by weight

Classification

EC1272/2008 EC - No.

Zinc di(acetate)

597-045-6

>99.9%

Acute Tox. 4   H302

Eye Dam. 1   H318

Aquatic Chronic 2H411 209-170-2

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

Eyes:   Immediately wash out thoroughly with water or saline solution. Seek medical attention.

Skin:   Remove contaminated clothing. Wash skin thoroughly with soap and water. Seek medical attention.

Inhalation:   Remove from exposure and place in fresh air. If not breathing, give artificial respiration. If breathing is

difficult, give oxygen. Seek medical attention.

Ingestion:   Do not induce vomiting. If confined to the mouth area give large quantities of water as a mouthwash,

ensure the water is not swallowed. If substance has been swallowed, give 500ml of water to dilute in stomach. Seek

medical attention immediately.

Most important symptoms and effects, both acute and delayed:

None known.

I ndication of any immediate medical attention and special treatment needed:   Treat symptomatically.

Page 3

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 3/9

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Use extinguishing media appropriate to the surrounding fire conditions (water, foam,

CO 2 , or dry chemical).

Special hazards arising from the mixture:   Emits toxic fumes under fire conditions.

Advice for fire-fighters:   Wear Self-Contained Breathing Apparatus and protective clothing to prevent contact with skin

and eyes.

Section 6: Accidental   Release Measures

Personal precautions, protective equipment and emergency procedures:

Wear appropriate protective clothing, to avoid contact with skin and eyes, and inhalation of dust.

Environmental precautions:   Do not let this material enter the environment.

Methods and materials for containment and cleaning up:

Clear up spillage, wash affected area with large quantities of water.

Section 7: Handling and Storage

Precautions for safe handling:

Handling:   Do not breathe or ingest dust. Avoid any contact with skin and eyes. Occupational exposure limits for dust

should be adhered to, this to include the use of local extraction systems.

Conditions for safe storage, including any incompatibilities:

Keep cool and dry, in tightly closed container.

Section 8:   Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:   Handle in accordance with good industrial hygiene and safety practice.

Exposure controls:

Appropriate engineering controls:   Ensure adequate ventilation. In case of insufficient ventilation, wear suitable

respiratory equipment. Ensure that eyewash stations and safety showers are close to the workstation location.

Individual protection measures, such as personal protective equipment:

Respiratory protection:   Respirator must be worn if exposed to dust. Respirator with a dust filter.

Hand protection:   Glove material: PVC or rubber gloves.

Eye protection:   Tightly fitting safety goggles or face-shield.

Skin and body protection:   Work clothing.

Page 4

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 4/9

Section 9: Physical and   Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   White.

Odour:   Odourless.

Odour threshold:   No data available.

pH (concentration):   6.0 - 8.0 at 50 g/l

Melting point/range (° C):   237   °C

Boiling point/range (° C):   No data available .

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   Non-flammable.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour pressure (20   ° C):   No data available .

Vapour density:   No data available.

Relative densi ty (25 ° C):   1.840   g/cm 3

Water solubility (g/L ) at 20 ° C:   434.78 g/L

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Stability:   Stable under normal storage and temperature conditions.

Conditions to avoid:   No data available.

Incompatible materials:   Oxidizing agents.

Hazardous decomposition products:   Hazardous decomposition products formed under fire conditions: Carbon oxides,

zinc/zinc oxides.

Thermal decomposition:   >237 °C

Section 11: Toxicological Information

Information on toxicological effects:

Harmful if swallowed.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Causes serious eye damage.

Respiratory or skin sensitisation:

Not classified based on available information.

Page 5

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 5/9

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   Toxic to aquatic life with long lasting effects.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   No data available.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Product:   Disposal should be in accordance with local or state or national legislation. Do not dispose of with oxidisers.

Contaminated packaging:   Disposal must be made according to official regulations. Dispose of contaminated packaging

in the same manner as the product.

Section 14: Transport Information

UN number:   3077

UN proper shipping name:   ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Zinc acetate)

Transport hazard class:   Class 9

Transport hazard labels:

Packing group:   III

Marine pollutant:   Yes.

Page 6

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 6/9

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Page 7

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 7/9

Directive 94/33/EC on the protection of young people at work

Not listed.

Additional Information:

SARA 302 Components:   No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components:   The following components are subject to reporting levels established by SARA Title III, Section

313: Zinc di(acetate) / CAS No. 5970-45-6 / Revision Date 1993-04-24.

SARA 311/312 Hazards:   Acute Health Hazard.

Massachusetts Right to Know Components:   Zinc di(acetate) / CAS No. 5970-45-6 / Revision Date 1993-04-24.

Pennsylvania Right to Know Components:   Zinc di(acetate) / CAS No. 5970-45-6 / Revision Date 1993-04-24.

New Jersey Right to Know Components:   Zinc di(acetate) / CAS No. 5970-45-6 / Revision Date 1993-04-24.

California Prop. 65 Components:   This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

Page 8

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 8/9

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H302 Harmful if swallowed.

H318 Causes serious eye damage.

H411 Toxic to aquatic life with long lasting effects.

Revision information:

None.

Page 9

Safety Data Sheet for Depleted Zinc Acetate

Dihydrate - Depleted in Zn-64 Isotope

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 03-Jun-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Depleted Zinc Acetate Dihydrate, Depleted in Zn-64 Isotope   www.buyisotopes.com   Page 9/9

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.