Dysprosium (Dy) Standard Solution SRM 3115a SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/dysprosium-dy-standard-solution-nist-srm-3115a-en-57330aabad

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

05 July 2018

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   3115a

SRM Name:     Dysprosium (Dy) Standard Solution

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended for use as a primary calibration standard for the quantitative

determination of dysprosium. A unit of SRM 3115a consists of five 10 mL sealed borosilicate glass ampoules of

an acidified aqueous solution prepared gravimetrically to contain a known mass fraction of dysprosium.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                        +1-703-527-3887   (International)

Website: https://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Classification

Physical Hazard:    Not applicable.

Health Hazard:      Skin Corrosion/Irritation            Category 1B

Serious Eye Damage/Irritation       Category 1

Label Elements

Symbol

Signal Word

DANGER

Hazard Statement(s)

H314               Causes severe skin burns and eye damage.

Precautionary Statement(s)

P260            Do not breathe fumes, mists, vapors, or spray.

P264             Wash hands thoroughly after handling.

P280              Wear protective gloves, protective clothing, and eye protection.

P301+P330+P331      If swallowed: Rinse mouth. Do NOT induce vomiting.

P303+P361+P353      If on skin (or hair): Remove immediately all contaminated clothing.  Rinse skin with

water.

P304+P340              If inhaled: Remove person to fresh air and keep comfortable for breathing.

P305+P351+P338      If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if

present and easy to do. Continue rinsing.

P310                Immediately call a doctor.

P363             Wash contaminated clothing before reuse.

P405                 Store locked up.

P501                Dispose of contents and container according to local regulations.

SRM 3115a                                                                            Page 1 of 7

Page 2

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Dysprosium in nitric acid solution

Other Designations:

Nitric acid (aqua fortis; hydrogen nitrate; azotic acid; engraver’s acid)

Dysprosium nitrate [Dysprosium trinitrate; Nitric acid, dysprosium (3+) salt (3:1)]

NOTE: Dysprosium in nitric acid solution forms a solvated dysprosium nitrate salt. The health and physical hazard

information provided in this SDS are for nitric acid and dysprosium nitrate, a rare earth salt. No physical or chemical

data are listed for this solution. The actual effects of the solution may differ from the individual components.

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the NIST Certificate

of Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)               (%)

Nitric acid                           7697-37-2         231-714-2                 14

Dysprosium nitrate                   10143-38-1         233-410-5                      2.1

Non-Hazardous Component(s)

Water                               7732-18-5         231-791-2               >77

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation: If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and

shoes. Get immediate medical attention. Thoroughly clean and dry contaminated clothing before reuse. Destroy

contaminated shoes.

Eye Contact: Immediately flush eyes, including under the eyelids with copious amounts of water for at least

15 minutes. Seek immediate medical attention.

Ingestion: Contact a poison control center immediately for instructions. Do not induce vomiting. Give water to

rinse out mouth. Never give liquids to a person with reduced awareness or becoming unconscious.  If vomiting

occurs, keep head lower than hips to prevent aspiration.  If not breathing, give artificial respiration by qualified

personnel. Seek immediate medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Acid burns to skin, eyes, and lungs.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek immediate medical attention.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing media appropriate to the surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: Miscellaneous decomposition products.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 3            Fire = 0          Reactivity = 0

SRM 3115a                                                                            Page 2 of 7

Page 3

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures:  Immediately contact emergency

personnel. Keep unnecessary personnel away. Use suitable protective equipment; see Section 8, “Exposure Controls

and Personal Protection”.

Methods and Materials for Containment and Clean up: Do not touch spilled material. Notify safety personnel

of spills. Absorb with sand or other non-combustible material. Collect spilled material in appropriate container for

disposal. Isolate hazard area and deny entry.

7. HANDLING AND STORAGE

Safe Handling Precautions: See Section 8, “Exposure Controls and Personal Protection”. Handle glass ampoules

with care.

Storage:  Store and handle in accordance with  all current regulations and standards.  Keep separated from

incompatible substances (see Section 10, “Stability and Reactivity”).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits

Component(s)     OSHA (PEL)         ACGIH (TLV)            NIOSH (REL)

5 mg/m3 (2 ppm) TWA                    5 mg/m3 (2 ppm)         5 mg/m3 (2 ppm) TWA   Nitric acid                                                            10 mg/m3 (4 ppm) STEL                 TWA            10 mg/m3 (4 ppm) STEL                                                                      65 mg/m3 (25 ppm) IDLH

Dysprosium nitrate                    No occupational limits established.

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:    If workplace conditions warrant a respirator, a respiratory protection program that

meets OSHA 29CFR 1910.134 must be followed. Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eyewash station should be

readily available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task

being performed and the risks involved and should be approved by a specialist before handling this product.

Chemical-resistant gloves should be worn at all times when handling chemicals.

SRM 3115a                                                                            Page 3 of 7

Page 4

9. PHYSICAL AND CHEMICAL PROPERTIES

NOTE: The physical and chemical data provided are for the pure components.

Descriptive Properties                               Nitric acid             Dysprosium nitrate

(14 % of this SRM)            (2.1 % of this SRM)

Appearance (physical state, color,            colorless to yellow          white to pale yellow

etc.)                                             liquid                          crystals

Molecular Formula                 HNO3                   Dy(NO3)3

Molar Mass (g/mol)                        63.01                      348.51

Odor                                                irritating odor               not available

Odor threshold (mg/m3)                     not available                not available

pH                                    1 (1 M)                     not available

Evaporation rate                            not available                not applicable

Melting point/freezing point              –42 ºC (–43 ºF)              not available

Relative Density as specific gravity          1.5027 at 25 ºC              not available

(water = 1)

Vapor Pressure (mmHg)                    47.9 at 20 ºC                not available

Vapor Density (air = 1)                      3.2                         not available

Viscosity                                    not available                not available

Solubility(ies)                                miscible with water          miscible in water

and ether

Partition coefficient                         not available                not available

(n-octanol/water)

Thermal Stability Properties

Autoignition Temperature                  not applicable               not available

Thermal Decomposition                     not applicable               not available

Initial boiling point and boiling range      83 ºC (181 ºF)               not available

Explosive Limits, LEL (Volume %)          not applicable               not available

Explosive Limits, UEL (Volume %)         not applicable               not available

Flash Point                                  not applicable               not available

Flammability (solid, gas)                    not applicable               not available

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:   X    Stable               Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Contact with combustible or incompatible materials.

Incompatible Materials:  Acids, combustible materials, halo carbons, amines, bases, oxidizing materials, metals,

halogens, metal salts, metal oxides, reducing agents, peroxides, metal carbide, cyanides.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition:  Thermal decomposition will produce oxides of nitrogen, dysprosium, and other

miscellaneous decomposition products.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

SRM 3115a                                                                            Page 4 of 7

Page 5

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Acid burns to skin, eyes, and

lungs.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation: Inhalation of acid fumes can damage the mucous membranes and upper respiratory tract. Short term

exposure may cause irritation and inflammation of the upper respiratory tract, coughing, choking, sore throat,

shortness of breath, headache, dizziness, and nausea. Long term exposure to acid fumes may cause damage to

teeth, bronchial irritation, chronic cough, bronchial pneumonia, and gastrointestinal disturbances.

Skin Contact:  Nitric acid can cause severe skin burns. Severity of the damage depends on the concentration

and duration of exposure. Effects of acid burns may be delayed.

Eye Contact:  Nitric acid can cause severe eye irritation, corneal burns, permanent eye damage, or blindness.

Severity of the damage depends on the concentration and duration of exposure.

Ingestion:  If ingested, nitric acid can cause severe burns and damage to the gastrointestinal tract.

Numerical Measures of Toxicity

Acute Toxicity: Not classified.

Nitric acid: Rat, Inhalation LC50: 130 mg/m3 (4 h)

Dysprosium nitrate: Rat, Oral LD50: 2386 mg/kg

Skin Corrosion/Irritation: This SRM contains nitric acid and it is classified as Category 1B.

Serious Eye Damage/Irritation: This SRM contains nitric acid and it is classified as Category 1.

Respiratory Sensitization: No data available; not classified.

Skin Sensitization: No data available; not classified.

Germ Cell Mutagenicity: No data available; not classified.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen           Yes       X   No

Nitric acid and dysprosium nitrate are not listed by NTP, IARC or OSHA as carcinogens/potential carcinogens.

Reproductive Toxicity: Not classified.

Nitric acid: Rat, Oral TDLo: 21 150 mg/kg (pregnant 1 d to 21 d)

Rat, Oral TDLo: 2345 mg/kg (pregnant 18 d)

Specific Target Organ Toxicity, Single Exposure: No data available; not classified.

Specific Target Organ Toxicity, Repeated Exposure: No data available; not classified.

Aspiration Hazard: No data available; not classified.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data

Nitric acid: Fish, hooknose or pogge (Agonus cataphractus) LC50: 100 mg/L to 330 mg/L (48 h)

Dysprosium nitrate: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations. Subject to

disposal regulations: U.S. EPA 40 CFR 262, Hazardous Waste Numbers: nitric acid (D001, D002).

SRM 3115a                                                                            Page 5 of 7

Page 6

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: UN1760, Corrosive liquid, n.o.s. (contains nitric acid), Hazard Class 8, Packing Group II.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4):  Nitric acid, 1000 lbs. (454 kg) final RQ

SARA Title III Section 302 (40 CFR 355.30):  Nitric acid, 1000 lbs. (454 kg) TPQ

SARA Title III Section 304 (40 CFR 355.40):  Nitric acid, 1000 lbs. (454 kg) EPCRA RQ

SARA Title III Section 313 (40 CFR 372.65):  1 % de minimis concentration for nitric acid.

OSHA Process Safety (29 CFR 1910.119):     Regulated for nitric acid at higher concentrations

500 lbs. TQ (≥94.5 % by weight).

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:        Yes.

CHRONIC HEALTH:     No.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations: Not listed under California Proposition 65.

U.S. TSCA Inventory: Nitric acid is listed. Dysprosium nitrate is not listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations: WHMIS Information is not provided for this material.

SRM 3115a                                                                            Page 6 of 7

Page 7

16. OTHER INFORMATION

Issue Date: 05 July 2018

Sources:   ChemAdvisor, Inc., SDS Nitric Acid, 09 December 2015.

ChemAdvisor, Inc., SDS Dysprosium Oxide, 23 June 2016.

CDC; NIOSH; NIOSH Pocket Guide to Chemical Hazards; Department of Health and Human

Services (DHHS), Centers for Disease Control and Prevention (CDC), National Institute for Safety and

Health;  Nitric Acid,  11 April 2016;  available  at  https://www.cdc.gov/niosh/npg/npgd0447.html

(accessed Jun 2018).

Hazardous Substances Data Bank (HSDB), National Library of Medicine's TOXNET system, Nitric

Acid CAS No. 7697-37-2; available at https://toxnet.nlm.nih.gov (accessed Jun 2018).

ChemIDplus Advanced, National Library of Medicine's TOXNET system, Dysprosium  Nitrate

CAS No. 10143-38-1; available at https://chem.sis.nlm.nih.gov/chemidplus/ (accessed Jun 2018).

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NRC          Nuclear Regulatory Commission

Hygienists

ALI          Annual Limit on Intake                     NTP           National Toxicology Program

CAS         Chemical Abstracts Service                 OSHA         Occupational Safety and Health Administration

CERCLA     Comprehensive Environmental Response,         PEL           Permissible Exposure Limit

Compensation, and Liability Act

CFR         Code of Federal Regulations                RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                 REL         Recommended Exposure Limit

EC50          Effective Concentration, 50 %             RM           Reference Material

EINECS      European Inventory of Existing Commercial       RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

LC50          Lethal Concentration, 50 %                   STEL          Short Term Exposure Limit

LD50          Lethal Dose, 50 %                        STOT          Specific Target Organ Toxicity

LEL         Lower Explosive Limit                    TLm          Threshold Limit, median

MSDS         Material Safety Data Sheet                  TLV          Threshold Limit Value

NFPA         National Fire Protection Association            TPQ          Threshold Planning Quantity

NIOSH        National Institute for Occupational Safety and Health  TSCA         Toxic Substances Control Act

NIST          National Institute of Standards and Technology     TWA        Time Weighted Average

n.o.s.         Not Otherwise Specified                   UEL         Upper Explosive Limit

WHMIS      Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

https://www.nist.gov/srm.

SRM 3115a                                                                            Page 7 of 7