Dysprosium (III) Chloride Hexahydrate 1640 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/dysprosium-iii-chloride-hexahydrate-prochem-en-cb00cd4115

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Dysprosium (III) Chloride Hexahydrate

PRODUCT NUMBER:  1640

CAS NUMBER:        15059-52-6

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315                 Causes skin irritation.

H319                 Causes serious eye irritation.

H335               May cause respiratory irritation.

Precautionary Phrases

P261                    Avoid breathing dust/fume/gas/mist/vapors/spray.

P264                Wash skin thoroughly after handling.

P271                 Use only outdoors or in a well-ventilated area.

P280                 Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352                IF ON SKIN: Wash with plenty of soap and water.

P304+P340                IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338         IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P362                  Take off contaminated clothing and wash before reuse.

P403+P233               Store in a well-ventilated place. Keep container tightly closed.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 1

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 1

DYSPROSIUM (III) CHLORIDE HEXAHYDRATE                                                   PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: May be a mild irritation to the eyes.

SKIN CONTACT: May cause slight irritation of the skin.

INHALATION: Inhalation of dust may lead to irritation of the respiratory tract.

INGESTION: May be irritating if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: DyCl3 6H2O

Molecular Weight: 376.95

CHEMICAL NAME                                           CAS#                               %

Dysprosium (III) Chloride Hexahydrate                                     15059-52-6                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen chloride gas and dysprosium.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation. Avoid breathing dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Spill can be mixed with vermiculite or sodium carbonate

powder and swept up and placed in suitable container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Avoid contact

with skin and eyes. Avoid formation of dusts and aerosols.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Hygroscopic. Store in a tightly sealed container.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

DYSPROSIUM (III) CHLORIDE HEXAHYDRATE                                                   PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Light yellow solid

ODOR: None

MOLECULAR WEIGHT: 376.96

BOILING POINT: No data available

MELTING POINT: No data available

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Avoid moisture

INCOMPATIBILITY: Active metals

DECOMPOSITION PRODUCTS: Hydrogen chloride gas and dysprosium

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 311/312: Acute Health Hazard

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

DYSPROSIUM (III) CHLORIDE HEXAHYDRATE                                                   PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

NEW JERSEY RIGHT TO KNOW: Dysprosium (III) Chloride Hexahydrate

PENNSYLVANIA RIGHT TO KNOW: Dysprosium (III) Chloride Hexahydrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/15

SDS DEPT.

DYSPROSIUM (III) CHLORIDE HEXAHYDRATE                                                   PAGE 4