Erbium Chloride hexahydrate 1693 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/erbium-chloride-hexahydrate-prochem-en-cac9f3691e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Erbium Chloride hexahydrate

PRODUCT NUMBER:  1693

CAS NUMBER:        10025-75-9

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                     TOLL FREE:    800-795-8788

Rockford, IL  61109

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P233              Keep container tightly closed

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling

P280              Wear protective gloves / eye protection / face protection

P302+P352            IF ON SKIN: Wash with plenty of soap and water

P332+P313                   If skin irritation occurs: Get medical advice / attention.

P362               Take off contaminated clothing and wash before reuse.

P305+P351+P338     IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 2            Flammability: 0                  Reactivity: 0

NFPA RATING:

Health: 2              Fire: 0                           Reactivity Hazard: 0

ERBIUM CHLORIDE HEXAHYDRATE                                                                PAGE 1

Page 2

EYE CONTACT: May cause eye irritation.

SKIN CONTACT: May cause skin irritation.

INHALATION: May cause respiratory tract irritation.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: ErCl3 ! 6H2O

Molecular Weight: 381.71

CHEMICAL NAME                                           CAS#                               %

Erbium Chloride hexahydrate                                             10025-75-9                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove the victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty

in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: No data available

EXPLOSION LIMITS: No data available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen chloride and erbium oxide

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Use personal protective equipment. Avoid dust formation. Ensure adequate ventilation. Avoid breathing

dust.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Vacuum or sweep up material and place into a suitable

disposal container.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Always wear appropriate approved personal protection. Avoid contact with skin and eyes.

Ensure adequate ventilation. Do not expose to moisture. Material is hygroscopic.

CONDITIONS FOR SAFE STORAGE: Store material in a tightly sealed container. Store in a cool, dry place.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical-resistant gloves. Immediately change contaminated clothing. Wash hands and face

after working with substances.

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in OSHA compliance with 29 CFR 19-10.134.

ERBIUM CHLORIDE HEXAHYDRATE                                                                PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Pink solid

MOLECULAR WEIGHT: 381.71

BOILING POINT: No data available

MELTING POINT: 774° C

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

SPECIFIC GRAVITY: No data available

VAPOR PRESSURE: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable under recommended storage conditions.

HAZARDOUS POLYMERIZATION: Has not been reported

CONDITIONS TO AVOID: Exposure to moist air or water. Material is hygroscopic.

INCOMPATIBILITY: Strong oxidizing agents and strong acids

DECOMPOSITION PRODUCTS: Hydrogen chloride and erbium oxide.

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% are identified as a

confirmed carcinogen by IARC, ACGIH, NTP, and OSHA.

MUTAGENIC EFFECTS: No data available

TETRATOGENIC EFFECTS: No data available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 313 COMPONENTS: This material does not contain any chemical components with known CAS numbers that exceed the

threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

OSHA: Irritant

SARA 302 COMMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 311/312 HAZARDS: Acute Health Hazard

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right To Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Erbium (III) chloride hexahydrate

NEW JERSEY RIGHT TO KNOW COMPONENTS: Erbium (III) chloride hexahydrate

ERBIUM CHLORIDE HEXAHYDRATE                                                                PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION. (Cont.)

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals know to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREVIOUSLY REPARED: 04/14

DATE REVISED: 08/26

REVISED SUMMARY: Removed fax number, updated section 2 Precautionary Phrases. All other details within this document about this

product remained unchanged. EHS/08-31/26

EHS/SDS DEPT.

ERBIUM CHLORIDE HEXAHYDRATE                                                                PAGE 4