Erbium chloride hydrate 1693 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/erbium-chloride-hydrate-sds-prochem-en-fdb5212069

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Erbium chloride hydrate

PRODUCT NUMBER:  1693

CAS NUMBER:        19423-85-9

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapours/spray.

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 1            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 1              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: May cause eye irritation.

SKIN CONTACT: May cause skin irritation.

ERBIUM CHLORIDE HYDRATE                                                                     PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

INHALATION: May cause respiratory tract irritation.

INGESTION: The toxicological properties of this substance have not been fully investigated.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: ErCl3  XH2O

Molecular Weight: 273.62 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Erbium chloride hydrate                                                  19423-85-9                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove the victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty

in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen Chloride

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear protective equipment. Ensure adequate ventilation. Avoid breathing dust.

ENVIRONMENTAL PRECAUTIONS: Do not allow material to be released to the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Vacuum or sweep up material and place into a suitable

disposal container.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate protective equipment. Ensure good ventilation. Do not expose to

moisture. Material is hygroscopic.

CONDITIONS FOR SAFE STORAGE: Store material in a tightly sealed container in a cool and dry place.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when

handling a chemical substance in the laboratory.

SKIN PROTECTION: Chemical-resistant gloves.

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available, a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

ERBIUM CHLORIDE HYDRATE                                                                    PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Pink Solid

MOLECULAR WEIGHT: 273.62 (anhy.)

BOILING POINT: Not available

MELTING POINT: 774° C

SPECIFIC GRAVITY: No data available

VAPOR PRESSURE: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable under recommended storage conditions

HAZARDOUS POLYMERIZATION: Has not been reported

CONDITIONS TO AVOID: Exposure to moist air or water. Material is hygroscopic.

INCOMPATIBILITY: Strong oxidizing agents and strong acids

DECOMPOSITION PRODUCTS: Hydrogen Chloride and erbium oxide.

SECTION 11 TOXICOLOGICAL DATA

RTECS DATA: No information available.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified as a confirmed

carcinogen by IARC, ACGIH, NTP, and OSHA.

MUTAGENIC EFFECTS: No data available

TETRATOGENIC EFFECTS: No data available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 313 COMPONENTS: No chemicals are reportable under Section 313.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 04/14

ERBIUM CHLORIDE HYDRATE                                                                    PAGE 3