Erbium (III) Carbonate Hydrate 1691 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/erbium-iii-carbonate-hydrate-prochem-en-42f32220b7

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Erbium (III) Carbonate Hydrate

PRODUCT NUMBER:  1691

CAS NUMBER:        22992-83-2

SYNONYMS:         Carbonic acid, erbium salt

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not classified as a hazardous substance according to the GHS.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 0

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation to the skin.

INHALATION: May be irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May cause irritation.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Er2(CO3)3  XH2O

Molecular Weight: 514.54 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Erbium (III) Carbonate Hydrate                                            22992-83-2                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

ERBIUM (III) CARBONATE HYDRATE                                                         PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES (Cont.)

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Product does not present explosion hazards.

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistant foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Carbon oxides and erbium oxide

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Avoid formation of dusts and aerosols.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Hygroscopic. Store away from oxidizing agents,

water, and moisture.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Pink powder

ODOR: None

MOLECULAR WEIGHT: 514.54 (anhy.)

BOILING POINT: No data available

MELTING POINT: No data available

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

ERBIUM (III) CARBONATE HYDRATE                                                         PAGE 2

Page 3

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Strong acids

INCOMPATIBILITY: Oxidizing agents, moisture, water, mineral acids, and active metals.

DECOMPOSITION PRODUCTS: Carbon oxides and erbium oxide

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good.

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Not Listed

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/15

SDS DEPT.

ERBIUM (III) CARBONATE HYDRATE                                                         PAGE 3