Europium metal SDS

Publisher: Ames Laboratory, US DOE — Materials Preparation Center

Source: https://resds.com/document/europium-eu-metal-sds-en-d173bf5787

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SDS

Safety Data Sheet

per 29 CFR 1910.1200                                                        Page 1 of 4

Europium

SAFETY DATA SHEET

1 PRODUCT AND SUPPLIER IDENTIFICATION

Product Name:  Europium - pieces, ingot, sheet, foil, rod

Formula:       Eu

Supplier:       Ames Laboratory, US DOE

Materials Preparation Center

121 Metals Development

Ames, IA 50010 USA

Telephone:      515-294-5236

Fax:            515-294-8727

Email:         mpc@ameslab.gov

Emergency:     515-294-3483 (24 hour)

Recommended Uses:  Scientific Research

2 HAZARDS IDENTIFICATION

GHS Classification (29 CFR 1910.1200):  Water reactive solid, flammable.

GHS Label Elements:

Signal Word: Danger

Hazard Statements: H228 Flammable solid, H261 In contact with water releases flammable gas.

Precautionary Statements: P210 Keep away from heat/sparks/open flames/hot surfaces - No smoking, P223 Do not allow

contact with water, P231+P232 Handle under inert gas. Protect from moisture, P370+P378 In case of fire: Use Class D dry

chemical extinguishing agent for extinction, P422 Store contents under inert gas.

3 COMPOSITION/INFORMATION ON INGREDIENTS

Ingredient:     Europium

CAS#:           7440-53-1

%:             100

EC#:            231-161-7

4  FIRST AID MEASURES

INHALATION: Remove to fresh air, keep warm and quiet, give oxygen if breathing is difficult. Seek medical attention.

INGESTION: Quickly wipe material from mouth and rinse with water. Do not induce vomiting. Seek medical attention.

SKIN: Remove contaminated clothing, brush material off skin, wash affected area with soap and water. Seek medical

attention if symptoms persist.

EYES: Flush eyes with lukewarm water, including under upper and lower eyelids, for at least 15 minutes. Seek medical

attention if symptoms persist.

Most Important Symptoms/Effects, Acute and Delayed: May cause irritation. See section 11 for more information.

Indication of Immediate Medical Attention and Special Treatment: No other relevant information available.

5 FIREFIGHTING MEASURES

Extinguishing Media: Use Class D dry powder extinguishing agent.

Unsuitable Extinguishing Media: Do not use water.

Ames Laboratory is operated for the U.S. DOE by Iowa State University, DE-AC02-07CH11358            Page 1 of 4

Page 2

SDS

Safety Data Sheet

per 29 CFR 1910.1200                                                        Page 2 of 4

Specific Hazards Arising from the Material: Flammable when exposed to heat, spark or flame. Will react with water

liberating hydrogen gas and europium hydroxide. May emit fumes of europium oxide under fire conditions.

Special Protective Equipment and Precautions for Firefighters: Full face, self-contained breathing apparatus and full

protective clothing to prevent contact with skin and eyes.

6 ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment, and Emergency Procedures: Wear appropriate respiratory and protective

equipment specified in section 8. Avoid contact with skin and eyes. Avoid breathing dust or fume. Eliminate all sources of

ignition. Isolate spill area.

Methods and Materials for Containment and Cleaning Up: Sweep or scoop spilled product and place in a closed

container for further handling and disposal. Do not use water for spill clean-up. Use only non-sparking tools and natural

bristle brushes.

Environmental Precautions: Do not flush to sewer, stream, or other bodies of water. Do not allow to enter drains or to be

released to the environment.

7 HANDLING AND STORAGE

Precautions for Safe Handling: Europium is extremely reactive to oxygen and water/moisture. Handle in an enclosed,

controlled process. Handle under dry protective gas such as argon. Use non-sparking tools. Protect from sources of ignition.

Do not allow contact with water. Avoid contact with skin and eyes. Wash thoroughly before eating or smoking. See section 8

for information on personal protection equipment.

Conditions for Safe Storage, Including Any Incompatibilities: Europium metal should be stored in tightly-closed

containers under argon or mineral oil. Keep away from sparks, heat and flame. Storage area should be free of combustibles

and ignition sources. Protect from water/moisture. Do not store together with acids or oxidizers. See section 10 for more

information on incompatible materials.

8 EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits:   Europium

OSHA/PEL:       No exposure limit established

ACGIH/TLV:       No exposure limit established

Appropriate Engineering Controls: Handle in a humidity controlled atmosphere. Handle in an enclosed, controlled process

under dry argon. Use local exhaust to maintain exposure at low levels. Whenever possible the use of local exhaust ventilation

or other engineering controls is the preferred method of controlling exposure to airborne dust and fume to meet established

occupational exposure limits. Do not use tobacco or food in work area. Wash thoroughly before eating or smoking. Do not

blow dust off clothing or skin with compressed air. Prepare for the possibility of a fire. Keep extinguishing agents, tools for

handling and protective clothing readily available.

Individual Protection Measures, Such as Personal Protective Equipment:

Respiratory Protection: Wear a NIOSH/MSHA approved respirator when high concentrations are present.

Eye Protection: Always wear approved chemical splash proof goggles.

Skin Protection: Rubber gloves, flame retardant protective work clothing.

9 PHYSICAL AND CHEMICAL PROPERTIES

Appearance:

Form:              Solid in various forms

Color:           Gray metallic

Odor:            Not determined

Odor Threshold:  Not determined

pH:                          N/A

Melting Point:                822 oC

Boiling Point:                 1527 oC

Flash Point:                  N/A

Evaporation Rate:            N/A

Flammability:                 Flammable solid

Upper Flammable Limit:      No data

Lower Flammable Limit:      No data

Vapor Pressure:             No data

Vapor Density:                N/A

Relative Density (Specific Gravity):   5.25 g/cc

Solubility in H2O:             Decomposes

Ames Laboratory is operated for the U.S. DOE by Iowa State University, DE-AC02-07CH11358            Page 2 of 4

Page 3

SDS

Safety Data Sheet

per 29 CFR 1910.1200                                                        Page 3 of 4

Partition Coefficient (n-octanol/water):  Not determined

Autoignition Temperature:     No data

Decomposition Temperature:   No data

Viscosity:                     N/A

10  STABILITY AND REACTIVITY

Reactivity: No data

Chemical Stability: Stable under recommended storage conditions.

Possibility of Hazardous Reactions: Finely divided metal may ignite spontaneously in air. Reacts with water releasing

flammable hydrogen gas.

Incompatible Materials: Water/moisture, air, strong acids, acid chlorides, strong oxidizing agents, halogens.

Hazardous Decomposition Products: Europium oxides, hydrogen gas.

11 TOXICOLOGICAL INFORMATION

Likely Routes of Exposure: Inhalation, skin and eyes.

Symptoms of Exposure: May cause irritation.

Acute and Chronic Effects: No data

Acute Toxicity: No data

Carcinogenicity: NTP: Not identified as carcinogenic  IARC: Not identified as carcinogenic

To the best of our knowledge the chemical, physical and toxicological characteristics of the substance are not fully known.

12 ECOLOGICAL INFORMATION

Ecotoxicity: No data

Persistence and Degradability: No data

Bioaccumulative Potential: No data

Mobility in Soil: No data

Other Adverse Effects: Do not allow material to be released to the environment. No further relevant information available.

13 DISPOSAL CONSIDERATIONS

Waste Disposal Method:

Product: Dispose of in accordance with Federal, State and Local regulations.

Packaging: Dispose of in accordance with Federal, State and Local regulations.

14 TRANSPORT INFORMATION

DOT/ADR/IATA/IMDG Regulations:

UN Number:              UN3132

UN Proper Shipping Name:  Water reactive solid, flammable, n.o.s. (Europium)

Transport Hazard Class:     4.3, 4.1

Packing Group:                  III

Marine Pollutant:         No

Special Precautions:        Warning: flammable solid. releases flammable gas in contact with water.

15 REGULATORY INFORMATION

TSCA Listed: All components are listed.

Regulation (EC) No 1272/2008 (CLP): Flammable solid, category 1; Substances which, in contact with water, emit

flammable gases, category 1.

Canada WHMIS Classification (CPR, SOR/88-66): Class B, Division 4 - Flammable solid; Class B, Division 6 - Reactive

flammable material.

HMIS Ratings: Health: 1   Flammability: 3  Physical: 2

NFPA Ratings: Health: 1   Flammability: 3  Reactivity: 2  Special Hazard: W

Chemical Safety Assessment: A chemical safety assessment has not been carried out.

16 OTHER INFORMATION

Ames Laboratory is operated for the U.S. DOE by Iowa State University, DE-AC02-07CH11358            Page 3 of 4

Page 4

SDS

Safety Data Sheet

per 29 CFR 1910.1200                                                        Page 4 of 4

The above information is believed to be correct, but does not purport to be all inclusive and shall be used only as a guide. The

Materials Preparation Center, Ames Laboratory, and Iowa State University, shall not be held liable for any damages resulting

from handling or from contact with the above product and make no warranties, expressed or implied, including, but not

limited to, any implied warranty of merchantability or fitness for a particular purpose or course of performance or usage of

trade.

Prepared by:        The Materials Preparation Center at Ames Laboratory

Revised/Reviewed:  March 23, 2018

Ames Laboratory is operated for the U.S. DOE by Iowa State University, DE-AC02-07CH11358            Page 4 of 4