Europium (III) Bromide Hydrate 1762 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/europium-iii-bromide-hydrate-prochem-en-ab07a09b50

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Europium (III) Bromide Hydrate

PRODUCT NUMBER:  1762

CAS NUMBER:        56069-78-5

SYNONYMS:         Europium Tribromide

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P312                   Call a POISON CENTER or doctor/physician if you feel unwell.

P362               Take off contaminated clothing and wash before reuse.

P403+P233           Store in a well-ventilated place. Keep container tightly closed.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                  Reactivity Hazard: 1

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 1

EUROPIUM (III) BROMIDE HYDRATE                                                         PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes serious eye irritation.

SKIN CONTACT: Causes skin irritation.

INHALATION: May cause respiratory tract irritation.

INGESTION: May cause irritation if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: EuBr3  XH2O

Molecular Weight: 391.68 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Europium(III) Bromide Hydrate                                           560069-78-5                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention.

INGESTION: Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Get medical attention.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Product does not present an explosion hazard.

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistant foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen bromide gas and europium oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Avoid formation of dusts and aerosols. Handle under inert gas.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store under inert gas. Hygroscopic.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

EUROPIUM (III) BROMIDE HYDRATE                                                         PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers, and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White crystalline

ODOR: None

MOLECULAR WEIGHT: 391.68 (anhy.)

BOILING POINT: No data available

MELTING POINT: > 702°C

SPECIFIC GRAVITY No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Hygroscopic

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: Hydrogen bromide and europium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

Lanthanons can cause delayed blood clotting leading to hemorrhages. Exposure may also lead to sensitivity heat, itching, increased

awareness of odor and taste, and liver damage.

Inorganic bromides may produce depression, emaciation, and in severe cases, psychosis and mental deterioration. Bromoderma, a

bromide rash, often occurs when bromide inhalation or administration is prolonged. This rash is usually found on the face and

resembles acne and furunculosis.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good.

EUROPIUM (III) BROMIDE HYDRATE                                                         PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

SARA 302/304: Not Listed

SARA 311/312: Acute Health Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Europium (III) Bromide hydrate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Europium (III) Bromide hydrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 06/15

SDS DEPT.

EUROPIUM (III) BROMIDE HYDRATE                                                         PAGE 4