Europium (III) Nitrate Hydrate 1780 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/europium-iii-nitrate-hydrate-prochem-en-80175f9d8b

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Europium (III) Nitrate Hydrate

PRODUCT NUMBER:  1780

CAS NUMBER:        100587-95-9

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire; oxidizer.

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.

P220                 Keep/store away from clothing/combustible material.

P221               Take any precaution to avoid mixing with combustibles.

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P362               Take off contaminated clothing and wash before reuse.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction.

P403+P233           Store in a well-ventilated place. Keep container tightly closed.

EUROPIUM (III) NITRATE                                                                 PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 2

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 2       Specific Hazard: OX

EYE CONTACT: Causes irritation to the eyes.

SKIN CONTACT: Causes irritation to the skin.

INHALATION: Causes irritation to respiratory system.

INGESTION: May cause irritation.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Eu(NO3)3 XH2O

Molecular Weight: 337.98 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Europium (III) Nitrate Hydrate                                            100587-95-9                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water fore several minutes. Assure adequate flushing

under eyelids. Consult a physician.

SKIN EXPOSURE: Wash affected area with soap and water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation or redness persists.

INHALATION: Remove to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in breathing,

coughing, wheezing, or pain. In such cases, seek immediate medical assistance. Give oxygen if deemed necessary.  If not breathing

administer CPR. Get emergency care immediately.

INGESTION: Seek medical assistance immediately. Keep the victim calm.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use carbon dioxide, extinguishing powder, or foam. Water may be ineffective but may be used for cooling

exposed containers.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides and europium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Absorb spill with inert materials, then place in suitable

container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Wash thoroughly after using. Keep container closed when not in use. Avoid formation of dusts and aerosols. This

substance is an oxidizer and can reduce the ignition temperature of flammable substances. Keep away from sources of ignition – No

smoking.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry well ventilated area. Store away from flammable substances, reducing agents,

organic materials, and metal powders.

EUROPIUM (III) NITRATE                                                                 PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Colorless crystals

ODOR: None

MOLECULAR WEIGHT: 337.98 (anhy.)

BOILING POINT: No data available

MELTING POINT: 85° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Avoid moisture

INCOMPATIBILITY: Organic materials, strong reducing agents, and strong acids.

DECOMPOSITION PRODUCTS: Nitrogen oxides and Europium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – Rat – >5000mg/kg

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: LE8050000

Lanthanons can cause delayed blood clotting leading to hemorrhages. Exposure may also lead to sensitivity heat, itching, increased

awareness of odor and taste, and liver damage.

Small doses of nitrates may cause weakness, general depression, headache and mental impairment. Larger doses may cause

dizziness abdominal cramps, vomiting, bloody diarrhea, convulsions, and collapse.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

EUROPIUM (III) NITRATE                                                                 PAGE 3

Page 4

SECTION 14  TRANSPORTATION DATA

UN1477

Nitrates, inorganic, n.o.s.

(Europium Nitrate)

CLASS 5.1

PG III

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

SARA 302/304: Not Listed

SARA 311/312: Reactivity Hazard, Acute Health Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Europium nitrate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Europium nitrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 12/14

EUROPIUM (III) NITRATE                                                                 PAGE 4