Europium (III) Sulfate Octahydrate 1793 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/europium-iii-sulfate-octahydrate-prochem-en-217530a7cd

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Europium (III) Sulfate Octahydrate

PRODUCT NUMBER:  1793

CAS NUMBER:        10031-55-7

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 1

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 1

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight irritation of the skin.

INHALATION: Inhalation of dust can lead to irritation of the respiratory tract.

INGESTION: No information on the physiological effects of ingestion.

EUROPIUM (III) SULFATE                                                                 PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Eu2(SO4)3  8H2O

Molecular Weight: 736.22

CHEMICAL NAME                                           CAS#                               %

Europium (III) Sulfate                                                    10031-55-7                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only If conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Product does not present an explosion hazard.

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistance foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Sulfur oxides and europium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep unprotected

persons away. Avoid dust formation. Avoid breathing dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in a suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from moisture.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White to light red solid

ODOR: None

MOLECULAR WEIGHT: 736.22

EUROPIUM (III) SULFATE                                                                 PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

BOILING POINT: No data available

MELTING POINT: 375° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: No data available

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: Sulfur oxides and europium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: No data

Lanthanons can cause delayed blood clotting leading to hemorrhages. Exposure may also lead to sensitivity heat, itching, increased

awareness of odor and taste, and liver damage.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good.

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: No SARA Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Europium (III) sulfate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Europium (III) sulfate

EUROPIUM (III) SULFATE                                                                 PAGE 3

Page 4

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 01/15

SDS DEPT.

EUROPIUM (III) SULFATE                                                                 PAGE 4