Gadobutrol - USP reference standard SDS

Publisher: USP

Source: https://resds.com/document/gadobutrol-usp-reference-standard-sds-en-c740d355ca

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

1. Identification

Product identifier              Gadobutrol

Other means of identification

Catalog number           1287449

CAS number               198637-52-4

Chemical name                    Gadolinium(III) [10-[(2RS,3SR)-1,3,4-trihydroxybutan-2-yl]-1,4,7,10-tetraazacyclododecane-

1,4,7-triacetate] monohydrate

Recommended use              For analytical laboratory use only.

Recommended restrictions      Not for use as a drug. Not for administration to humans or animals.

Manufacturer/Importer/Supplier/Distributor information

Manufacturer

Company name              U. S. Pharmacopeia

Address                   12601 Twinbrook Parkway

Rockville

MD

20852-1790

United States

Telephone                  Customer Service          301-881-0666

Website                     www.usp.org

E-mail                   RSTECH@usp.org

Emergency phone number     CHEMTREC within US &    1-800-424-9300

Canada

CHEMTREC outside US &  +1 703-527-3887

Canada

2. Hazard(s) identification

Physical hazards               Not classified.

Health hazards                 Not classified.

Environmental hazards          Not classified.

OSHA defined hazards          Not classified.

Label elements

Hazard symbol             None.

Signal word                None.

Hazard statement           Not available.

Precautionary statement

Prevention             Not available.

Response              Not available.

Storage                Not available.

Disposal               Not available.

Hazard(s) not otherwise          This product is supplied in a small quantity which does not constitute a combustible dust hazard.

classified (HNOC)             The physical properties of this material indicate that in large quantities accumulated dust may be

hazardous.

Supplemental information        Pharmacologically active material.

3. Composition/information on ingredients

Substance

Chemical name             Common name and synonyms          CAS number         %

Gadobutrol                                                                 198637-52-4              100

Material name: Gadobutrol                                                                                                  USP SDS US

1287449  Version #: 05  Revision date: 04-27-2023  Issue date: 01-08-2020                                               1 / 6

Page 2

4. First-aid measures

Inhalation                  Move to fresh air. Call a physician if symptoms develop or persist.

Skin contact                    Rinse skin with water/shower. Get medical attention if irritation develops and persists.

Eye contact                    Rinse with water. Get medical attention if irritation develops and persists.

Ingestion                       Rinse mouth. If ingestion of a large amount does occur, call a poison control center immediately.

Most important                  Pharmacologically active material. Occupational exposure may cause physiological effects.

symptoms/effects, acute and

delayed

Indication of immediate          Treat symptomatically.

medical attention and special

treatment needed

General information          Remove from exposure. Remove contaminated clothing. For treatment advice, seek guidance from

an occupational health physician or other licensed health-care provider familiar with workplace

chemical exposures. In the United States, the national poison control center phone number is

1-800-222-1222. If person is not breathing, give artificial respiration. If breathing is difficult, give

oxygen if available. Persons developing serious hypersensitivity (anaphylactic) reactions must

receive immediate medical attention.

5. Fire-fighting measures

Suitable extinguishing media    Water. Foam. Dry chemical or CO2. Use fire-extinguishing media appropriate for surrounding

materials.

Unsuitable extinguishing       None known.

media

Specific hazards arising from    Explosion hazard: Avoid generating dust; fine dust dispersed in air in sufficient concentrations and

the chemical                          in the presence of an ignition source is a potential dust explosion hazard.

Special protective equipment    Wear suitable protective equipment.

and precautions for firefighters

Fire fighting                 Use water spray to cool unopened containers. As with all fires, evacuate personnel to a safe area.

equipment/instructions            Firefighters should use self-contained breathing equipment and protective clothing.

Specific methods             Use standard firefighting procedures and consider the hazards of other involved materials.

6. Accidental release measures

Personal precautions,          Keep unnecessary personnel away. Dust deposits should not be allowed to accumulate on

protective equipment and        surfaces, as these may form an explosive mixture if they are released into the atmosphere in

emergency procedures             sufficient concentration. Wear appropriate personal protective equipment. Avoid inhalation of dust

from the spilled material. Ensure adequate ventilation. Do not touch damaged containers or spilled

material unless wearing appropriate protective clothing. For personal protection, see section 8 of

the SDS.

Methods and materials for       Avoid the generation of dusts during clean-up. Sweep up or vacuum up spillage and collect in

containment and cleaning up     suitable container for disposal. Clean surface thoroughly to remove residual contamination. For

waste disposal, see section 13 of the SDS.

Environmental precautions      Avoid discharge into drains, water courses or onto the ground.

7. Handling and storage

Precautions for safe handling   As a general rule, when handling USP materials, avoid all contact and inhalation of dust, mists,

and/or vapors associated with the material. Clean equipment and work surfaces with suitable

detergent or solvent after use. After removing gloves, wash hands and other exposed skin

thoroughly. Avoid significant deposits of material, especially on horizontal surfaces, which may

become airborne and form combustible dust clouds and may contribute to secondary explosions.

Combustible dust clouds may be created where operations produce fine material (dust). Select

and use containment devices and personal protective equipment based on a risk assessment of

material potency and exposure potential.

Conditions for safe storage,      Store in tight container. This material should be handled and stored per label instructions to ensure

including any incompatibilities   product integrity.

8. Exposure controls/personal protection

Occupational exposure limits   No exposure limits noted for ingredient(s).

Biological limit values         No biological exposure limits noted for the ingredient(s).

Material name: Gadobutrol                                                                                                  USP SDS US

1287449  Version #: 05  Revision date: 04-27-2023  Issue date: 01-08-2020                                               2 / 6

Page 3

Appropriate engineering         For laboratory operations, use local exhaust ventilation or a ventilated enclosure for high energy

controls                          operations such as particle sizing. Control exposures to below the occupational exposure level (if

available). Select and use containment devices and personal protective equipment based on a risk

assessment of exposure potential. Cover all containers for solutions and slurries while being

transferred.

Individual protection measures, such as personal protective equipment

Eye/face protection        Wear safety glasses with side shields, chemical splash goggles, or full face shield, if necessary.

Base the choice of protection on the job activity and potential for contact with eyes or face. An

emergency eye wash station should be available.

Skin protection

Hand protection       Wear nitrile or other impervious gloves if skin contact is possible. When the material is dissolved

or suspended in an organic solvent, wear gloves that provide protection against the solvent.

Other                    Train employees in proper gowning and degowning practices. Wear lab coat. Base the choice of

skin protection on the job activity, potential for skin contact and solvents and reagents in use. Do

not wear protective garments in common areas (e.g., cafeterias) or out-of-doors.

Respiratory protection       Respirators are generally not required for laboratory operations. Use a tight-fitting full-face

respirator with HEPA filters for spill cleanup. Choose respiratory protection appropriate to the task

and the level of existing engineering controls.

Thermal hazards          Wear appropriate thermal protective clothing, when necessary.

General hygiene                 Handling practices in this SDS are recommendations for laboratory use of USP materials.

considerations

9. Physical and chemical properties

Appearance                   Appearance descriptions are general information and not specific to any USP lot.

Physical state                  Solid.

Form                           Crystalline powder.

Color                        White.

Odor                          Not available.

Odor threshold                 Not available.

pH                            Not available.

Melting point/freezing point     > 572 °F (> 300 °C)

Initial boiling point and boiling   Not available.

range

Flash point                    Not available.

Evaporation rate               Not available.

Flammability (solid, gas)        Not available.

Upper/lower flammability or explosive limits

Flammability limit - lower    Not available.

(%)

Flammability limit - upper    Not available.

(%)

Explosive limit - lower (%)   Not available.

Explosive limit - upper (%)   Not available.

Vapor pressure                 Not available.

Vapor density                  Not available.

Relative density                Not available.

Solubility(ies)

Solubility (water)           Not available.

Auto-ignition temperature       Not available.

Decomposition temperature     Not available.

Viscosity                      Not available.

Material name: Gadobutrol                                                                                                  USP SDS US

1287449  Version #: 05  Revision date: 04-27-2023  Issue date: 01-08-2020                                               3 / 6

Page 4

Other information

Dust explosion properties

Minimum ignition       30 - 100 mJ

energy (MIE) - dust

cloud

Molecular formula        C18H31GdN4O9 · H20

Molecular weight           622.73 g/mol

10. Stability and reactivity

Reactivity                    The product is stable and non-reactive under normal conditions of use, storage and transport.

Chemical stability                 Material is stable under normal conditions.

Possibility of hazardous       No dangerous reaction known under conditions of normal use.

reactions

Conditions to avoid              Contact with incompatible materials.

Incompatible materials         None known.

Hazardous decomposition      NOx. Gadolinium. Irritating and/or toxic fumes or gases. Emits toxic fumes under fire conditions.

products

11. Toxicological information

Information on likely routes of exposure

Inhalation                 Knowledge about health hazard is incomplete.

Skin contact               Knowledge about health hazard is incomplete.

Eye contact               Knowledge about health hazard is incomplete.

Ingestion                 Knowledge about health hazard is incomplete.

Symptoms related to the          Gastrointestinal disturbances. Dizziness. Headache.

physical, chemical and

toxicological characteristics

Information on toxicological effects

Acute toxicity

Product                        Species                                      Test Results

Gadobutrol (CAS 198637-52-4)

Oral

LD50                    Rat                                     > 12000 mg/kg

Skin corrosion/irritation        Knowledge about health hazard is incomplete.

Serious eye damage/eye        Knowledge about health hazard is incomplete.

irritation

Respiratory or skin sensitization

Respiratory sensitization    Knowledge about health hazard is incomplete.

Skin sensitization          Knowledge about health hazard is incomplete.

Germ cell mutagenicity         Knowledge about mutagenicity is incomplete.

Carcinogenicity               Knowledge about carcinogenicity is incomplete.

IARC Monographs. Overall Evaluation of Carcinogenicity

Not listed.

OSHA Specifically Regulated Substances (29 CFR 1910.1001-1053)

Not listed.

US. National Toxicology Program (NTP) Report on Carcinogens

Not listed.

Reproductive toxicity           Knowledge about health hazard is incomplete.

Specific target organ toxicity -   Knowledge about health hazard is incomplete.

single exposure

Specific target organ toxicity -   Knowledge about health hazard is incomplete.

repeated exposure

Aspiration hazard             Based on available data, the classification criteria are not met.

Further information              Pharmacologically active material. Occupational exposure may cause physiological effects.

Material name: Gadobutrol                                                                                                  USP SDS US

1287449  Version #: 05  Revision date: 04-27-2023  Issue date: 01-08-2020                                               4 / 6

Page 5

12. Ecological information

Ecotoxicity                   The product is not classified as environmentally hazardous. However, this does not exclude the

possibility that large or frequent spills can have a harmful or damaging effect on the environment.

Persistence and degradability   No data is available on the degradability of this product.

Bioaccumulative potential      No data available.

Mobility in soil              No data available.

Other adverse effects         No other adverse environmental effects (e.g. ozone depletion, photochemical ozone creation

potential, endocrine disruption, global warming potential) are expected from this component.

13. Disposal considerations

Disposal instructions           Dispose in accordance with all applicable regulations. Under RCRA, it is the responsibility of the

user of the product to determine, at the time of disposal, whether the product meets RCRA criteria

for hazardous waste.

Local disposal regulations       Dispose in accordance with all applicable regulations.

Hazardous waste code         The waste code should be assigned in discussion between the user, the producer and the waste

disposal company.

Waste from residues / unused   Empty containers or liners may retain some product residues. This material and its container must

products                     be disposed of in a safe manner (see: Disposal instructions).

Contaminated packaging        Since emptied containers may retain product residue, follow label warnings even after container is

emptied.

14. Transport information

DOT

Not regulated as dangerous goods.

IATA

Not regulated as dangerous goods.

Transport in bulk according to   Not applicable.

Annex II of MARPOL 73/78 and

the IBC Code

General information                       It is the shipper's responsibility to determine the correct transport classification at the time of

shipment.

15. Regulatory information

US federal regulations           This product is not known to be a "Hazardous Chemical" as defined by the OSHA Hazard

Communication Standard, 29 CFR 1910.1200.

Toxic Substances Control Act (TSCA)

TSCA Section 12(b) Export Notification (40 CFR 707, Subpt. D)

Not regulated.

CERCLA Hazardous Substance List (40 CFR 302.4)

Not listed.

SARA 304 Emergency release notification

Not regulated.

OSHA Specifically Regulated Substances (29 CFR 1910.1001-1053)

Not listed.

Superfund Amendments and Reauthorization Act of 1986 (SARA)

SARA 302 Extremely hazardous substance

Not listed.

SARA 311/312 Hazardous   No

chemical

SARA 313 (TRI reporting)

Not regulated.

Other federal regulations

Clean Air Act (CAA) Section 112 Hazardous Air Pollutants (HAPs) List

Not regulated.

Clean Air Act (CAA) Section 112(r) Accidental Release Prevention (40 CFR 68.130)

Not regulated.

Material name: Gadobutrol                                                                                                  USP SDS US

1287449  Version #: 05  Revision date: 04-27-2023  Issue date: 01-08-2020                                               5 / 6

Page 6

Safe Drinking Water Act     Not regulated.

(SDWA)

US state regulations

California Proposition 65

California Safe Drinking Water and Toxic Enforcement Act of 1986 (Proposition 65): This material

is not known to contain any chemicals currently listed as carcinogens or reproductive toxins. For

more information go to www.P65Warnings.ca.gov.

International Inventories

Country(s) or region         Inventory name                                     On inventory (yes/no)*

Australia                       Australian Inventory of Chemical Substances (AICS)                              No

Canada                    Domestic Substances List (DSL)                                            No

Canada                    Non-Domestic Substances List (NDSL)                                       No

China                         Inventory of Existing Chemical Substances in China (IECSC)                        No

Europe                    European Inventory of Existing Commercial Chemical                             No

Substances (EINECS)

Europe                    European List of Notified Chemical Substances (ELINCS)                          No

Japan                         Inventory of Existing and New Chemical Substances (ENCS)                        No

Korea                          Existing Chemicals List (ECL)                                              No

New Zealand            New Zealand Inventory                                                  No

Philippines                     Philippine Inventory of Chemicals and Chemical Substances                         No

(PICCS)

Taiwan                    Taiwan Chemical Substance Inventory (TCSI)                                   No

United States & Puerto Rico   Toxic Substances Control Act (TSCA) Inventory                                 No

*A "Yes" indicates that all components of this product comply with the inventory requirements administered by the governing country(s)

A "No" indicates that one or more components of the product are not listed or exempt from listing on the inventory administered by the governing

country(s).

16. Other information, including date of preparation or last revision

Issue date                      01-08-2020

Revision date                   04-27-2023

Version #                     05

Further information              Refer to NFPA 654, Standard for the Prevention of Fire and Dust Explosions from the

Manufacturing, Processing, and Handling of Combustible Particulate Solids, for safe handling.

Disclaimer                USP materials are sold for analytical laboratory use only, and NOT for human consumption. The

information contained herein is applicable solely to the chemical substance when used for

analytical laboratory use and does not necessarily relate to any other use of the substance

described, (i.e. at different concentrations, in drug dosage forms, or in bulk quantities). USP

materials are intended for use by persons having technical skill and at their own discretion and

risk. This information has been developed by USP staff from sources considered reliable but has

not been independently verified by the USP. Therefore, the USP Convention cannot guarantee the

accuracy of the information in these sources nor should the statements contained herein be

considered an official expression. NO REPRESENTATION OR WARRANTY, EXPRESS OR

IMPLIED, INCLUDING THE WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE is made with respect to the information contained herein.

Material name: Gadobutrol                                                                                                  USP SDS US

1287449  Version #: 05  Revision date: 04-27-2023  Issue date: 01-08-2020                                               6 / 6