Gadolinium, chips, 99.9% trace rare earth metals basis 263087 SDS

Publisher: Sigma-Aldrich

Source: https://resds.com/document/gadolinium-chips-99-9-trace-rare-earth-metals-basis-sds-sigma-en-345d62230c

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SIGMA-ALDRICH                                                                                sigma-aldrich.com

SAFETY DATA SHEET

Version 4.6

Revision Date 08/10/2015

Print Date 11/10/2018

1. PRODUCT AND COMPANY IDENTIFICATION

1.1   Product identifiers

Product name                        :  Gadolinium, chips, 99.9% trace rare earth metals

basis

Product Number                     :  263087

Brand                                     :   Aldrich

CAS-No.                                 :  7440-54-2

1.2   Relevant identified uses of the substance or mixture and uses advised against

Identified uses                        :  Laboratory chemicals, Synthesis of substances

1.3    Details of the supplier of the safety data sheet

Company                               :  Sigma-Aldrich

3050 Spruce Street

SAINT LOUIS MO 63103

USA

Telephone                              :  +1 800-325-5832

Fax                                         :  +1 800-325-5052

1.4   Emergency telephone number

Emergency Phone #              :  +1-703-527-3887 (CHEMTREC)

2. HAZARDS IDENTIFICATION

2.1    Classification of the substance or mixture

GHS Classification in accordance with 29 CFR 1910 (OSHA HCS)

Substances and mixtures, which in contact with water, emit flammable gases (Category 3), H261

For the full text of the H-Statements mentioned in this Section, see Section 16.

2.2  GHS Label elements, including precautionary statements

Pictogram

Signal word                 Warning

Hazard statement(s)

H261                             In contact with water releases flammable gases.

Precautionary statement(s)

P231 + P232                Handle under inert gas. Protect from moisture.

P280                    Wear protective gloves/ eye protection/ face protection.

P370 + P378                     In case of fire: Use dry sand, dry chemical or alcohol-resistant foam to

extinguish.

P402 + P404                  Store in a dry place. Store in a closed container.

P501                       Dispose of contents/ container to an approved waste disposal plant.

2.3   Hazards not otherwise classified (HNOC) or not covered by GHS - none

Aldrich - 263087                                                                                                Page 1 of 7

Page 2

3. COMPOSITION/INFORMATION ON INGREDIENTS

3.1   Substances

Formula                                  :  Gd

Molecular weight                    :  157.25 g/mol

CAS-No.                                 :  7440-54-2

EC-No.                                   :  231-162-2

Hazardous components

Component                                                Classification                  Concentration

Gadolinium

Water-react. 3; H261         <= 100 %

For the full text of the H-Statements mentioned in this Section, see Section 16.

4. FIRST AID MEASURES

4.1   Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.Move out of dangerous area.

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact

Flush eyes with water as a precaution.

If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

4.2   Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

4.3    Indication of any immediate medical attention and special treatment needed

No data available

5. FIREFIGHTING MEASURES

5.1   Extinguishing media

Suitable extinguishing media

Dry powder

5.2   Special hazards arising from the substance or mixture

gadolinium oxides

5.3   Advice for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

5.4   Further information

No data available

6. ACCIDENTAL RELEASE MEASURES

6.1   Personal precautions, protective equipment and emergency procedures

Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe

areas.

For personal protection see section 8.

6.2   Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Aldrich - 263087                                                                                                Page 2 of 7

Page 3

6.3   Methods and materials for containment and cleaning up

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-

brushing and place in container for disposal according to local regulations (see section 13). Do not flush with water.

Keep in suitable, closed containers for disposal.

6.4   Reference to other sections

For disposal see section 13.

7. HANDLING AND STORAGE

7.1   Precautions for safe handling

Further processing of solid materials may result in the formation of combustible dusts. The potential for combustible

dust formation should be taken into consideration before additional processing occurs.

Provide appropriate exhaust ventilation at places where dust is formed.Keep away from sources of ignition - No

smoking.

For precautions see section 2.2.

7.2   Conditions for safe storage, including any incompatibilities

Keep container tightly closed in a dry and well-ventilated place.

Never allow product to get in contact with water during storage.

Air and moisture sensitive.

Storage class (TRGS 510): Hazardous materials, which set free flammable gases upon contact with water

7.3    Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1   Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

8.2   Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of

workday.

Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved

under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without

touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after

use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

Full contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm

Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

Splash contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm

Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method:

EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the

supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an

Aldrich - 263087                                                                                                Page 3 of 7

Page 4

industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It

should not be construed as offering an approval for any specific use scenario.

Body Protection

Flame retardant protective clothing, The type of protective equipment must be selected according to the

concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type

N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the

sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and

approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Control of environmental exposure

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

9. PHYSICAL AND CHEMICAL PROPERTIES

9.1   Information on basic physical and chemical properties

a)  Appearance            Form: chips

Colour: light grey

b)  Odour              No data available

c)  Odour Threshold      No data available

d)  pH                No data available

e)  Melting point/freezing     Melting point/range: 1,313 °C (2,395 °F) - lit.

point

f)    Initial boiling point and   3,273 °C (5,923 °F) - lit.

boiling range

g)  Flash point           No data available

h)  Evaporation rate       No data available

i)   Flammability (solid, gas) No data available

j)   Upper/lower          No data available

flammability or

explosive limits

k)  Vapour pressure       No data available

l)   Vapour density        No data available

m)  Relative density         7.886 g/mL at 25 °C (77 °F)

n)  Water solubility           insoluble

o)   Partition coefficient: n-   No data available

octanol/water

p)  Auto-ignition          No data available

temperature

q)  Decomposition        No data available

temperature

r)   Viscosity            No data available

s)  Explosive properties    No data available

t)   Oxidizing properties    No data available

9.2   Other safety information

No data available

Aldrich - 263087                                                                                                Page 4 of 7

Page 5

10. STABILITY AND REACTIVITY

10.1  Reactivity

No data available

10.2  Chemical stability

Stable under recommended storage conditions.

10.3  Possibility of hazardous reactions

Reacts violently with water.

10.4  Conditions to avoid

Exposure to moisture

10.5  Incompatible materials

Oxygen, Strong acids, Strong oxidizing agents

10.6  Hazardous decomposition products

Other decomposition products - No data available

In the event of fire: see section 5

11. TOXICOLOGICAL INFORMATION

11.1  Information on toxicological effects

Acute toxicity

No data available

Inhalation: No data available

Dermal: No data available

No data available

Skin corrosion/irritation

No data available

Serious eye damage/eye irritation

No data available

Respiratory or skin sensitisation

No data available

Germ cell mutagenicity

No data available

Carcinogenicity

Carcinogenicity - Mouse - Implant

Tumorigenic:Equivocal tumorigenic agent by RTECS criteria. Tumorigenic:Tumors at site or application.

IARC:    No component of this product present at levels greater than or equal to 0.1% is identified as

probable, possible or confirmed human carcinogen by IARC.

ACGIH:   No component of this product present at levels greater than or equal to 0.1% is identified as a

carcinogen or potential carcinogen by ACGIH.

NTP:     No component of this product present at levels greater than or equal to 0.1% is identified as a

known or anticipated carcinogen by NTP.

OSHA:    No component of this product present at levels greater than or equal to 0.1% is identified as a

carcinogen or potential carcinogen by OSHA.

Reproductive toxicity

No data available

No data available

Specific target organ toxicity - single exposure

No data available

Aldrich - 263087                                                                                                Page 5 of 7

Page 6

Specific target organ toxicity - repeated exposure

No data available

Aspiration hazard

No data available

Additional Information

RTECS: Not available

12. ECOLOGICAL INFORMATION

12.1  Toxicity

No data available

12.2  Persistence and degradability

No data available

12.3  Bioaccumulative potential

No data available

12.4  Mobility in soil

No data available

12.5  Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

12.6  Other adverse effects

No data available

13. DISPOSAL CONSIDERATIONS

13.1  Waste treatment methods

Product

Contact a licensed professional waste disposal service to dispose of this material. Burn in a chemical incinerator

equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer

surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging

Dispose of as unused product.

14. TRANSPORT INFORMATION

DOT (US)

UN number: 3208      Class: 4.3              Packing group: III

Proper shipping name: Metallic substance, water-reactive, n.o.s. (Gadolinium)

Reportable Quantity (RQ):

Poison Inhalation Hazard: No

IMDG

UN number: 3208      Class: 4.3              Packing group: III         EMS-No: F-G, S-N

Proper shipping name: METALLIC SUBSTANCE, WATER-REACTIVE, N.O.S. (Gadolinium)

IATA

UN number: 3208      Class: 4.3              Packing group: III

Proper shipping name: Metallic substance, water-reactive, n.o.s. (Gadolinium)

15. REGULATORY INFORMATION

SARA 302 Components

No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 313 Components

This material does not contain any chemical components with known CAS numbers that exceed the threshold (De

Minimis) reporting levels established by SARA Title III, Section 313.

Aldrich - 263087                                                                                                Page 6 of 7

Page 7

SARA 311/312 Hazards

Reactivity Hazard

Massachusetts Right To Know Components

No components are subject to the Massachusetts Right to Know Act.

Pennsylvania Right To Know Components

CAS-No.           Revision Date

Gadolinium                                                7440-54-2

New Jersey Right To Know Components

CAS-No.           Revision Date

Gadolinium                                                7440-54-2

California Prop. 65 Components

This product does not contain any chemicals known to State of California to cause cancer, birth defects, or any other

reproductive harm.

16. OTHER INFORMATION

Full text of H-Statements referred to under sections 2 and 3.

H261                In contact with water releases flammable gases.

Water-react.       Substances and mixtures, which in contact with water, emit flammable gases

HMIS Rating

Health hazard:           0

Chronic Health Hazard:

Flammability:            3

Physical Hazard         1

NFPA Rating

Health hazard:           0

Fire Hazard:            3

Reactivity Hazard:        1

Special hazard.I:     W

Further information

Copyright 2015 Sigma-Aldrich Co. LLC. License granted to make unlimited paper copies for internal use only.

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a

guide. The information in this document is based on the present state of our knowledge and is applicable to the

product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the

product. Sigma-Aldrich Corporation and its Affiliates shall not be held liable for any damage resulting from handling

or from contact with the above product. See www.sigma-aldrich.com and/or the reverse side of invoice or packing

slip for additional terms and conditions of sale.

Preparation Information

Sigma-Aldrich Corporation

Product Safety – Americas Region

1-800-521-8956

Version: 4.6                      Revision Date: 08/10/2015          Print Date: 11/10/2018

Aldrich - 263087                                                                                                Page 7 of 7