Gadolinium Chloride Hexahydrate, 99.9% 074-02663,078-02661 SDS

Publisher: FUJIFILM Wako Pure Chemical Corporation

Source: https://resds.com/document/gadolinium-chloride-hexahydrate-99-9-sds-fujifilm-wako-pure-chemical-en-7b178f50b4

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

According to JIS Z 7253:2019

Issue Date 05-Mar-2026

Revision Number  2.03

Section 1: PRODUCT AND COMPANY IDENTIFICATION

Product Name            Gadolinium Chloride Hexahydrate, 99.9%

Product Code            074-02663,078-02661

Supplier                       FUJIFILM Wako Pure Chemical Corporation

1-2 Doshomachi 3-Chome, Chuo-ku, Osaka 540-8605, Japan

Phone: +81-6-6203-3741

Fax: +81-6-6203-2029

Emergency telephone number     +81-6-6203-3741 / +81-3-3270-8571

Recommended uses              For research use only

Restrictions on use             Seek expert judgment when using for purposes other than those recommended.

Section 2: HAZARDS IDENTIFICATION

GHS classification

Classification of the substance or mixture

Skin corrosion/irritation                                                       Category 2

Serious eye damage/eye irritation                                              Category 2A

Pictograms

Signal word                     Warning

Hazard statements

H315 - Causes skin irritation

H319 - Causes serious eye irritation

Precautionary statements-(Prevention)

• Wash face, hands and any exposed skin thoroughly after handling

• Wear protective gloves/protective clothing/eye protection/face protection

Precautionary statements-(Response)

• IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue

rinsing

• If eye irritation persists: Get medical advice/attention

• IF ON SKIN: Wash with plenty of soap and water

• If skin irritation occurs: Get medical advice/attention

• Take off contaminated clothing and wash before reuse

Precautionary statements-(Storage)

• Not applicable

Precautionary statements-(Disposal)

• Not applicable

Others

Other hazards                    Not available

Page 1 / 6

W01W0107-0266 JGHEEN                                        Gadolinium Chloride Hexahydrate, 99.9%

Page 2

Issue Date 05-Mar-2026

Section 3: COMPOSITION/INFORMATION ON INGREDIENTS

Single Substance or Mixture      Substance

Formula                      GdCl3·6H2O

Chemical Name            Weight-%       Molecular weight      ENCS          ISHL No.       CAS RN

Gadolinium Chloride            99.9             371.70            N/A             N/A           13450-84-5

Hexahydrate                 (subtracting

method)

Note on ISHL No.:                     * in the table means announced chemical substances.

Section 4: FIRST AID MEASURES

Inhalation

Remove to fresh air. If symptoms persist, call a physician.

Skin contact

Wash off immediately with soap and plenty of water. If symptoms persist, call a physician.

Eye contact

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue

rinsing. Immediate medical attention is required.

Ingestion

Rinse mouth. Never give anything by mouth to an unconscious person. Call a physician or poison control center

immediately. Do not induce vomiting without medical advice.

Protection of first-aiders

Use personal protective equipment as required.

Section 5: FIRE FIGHTING MEASURES

Suitable extinguishing media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment

Unsuitable extinguishing media

No information available

Specific hazards arising from the chemical product

Thermal decomposition can lead to release of irritating and toxic gases and vapors.

Special extinguishing method

No information available

Special protective actions for fire-fighters

Use personal protective equipment as required. Firefighters should wear self-contained breathing apparatus and full

firefighting turnout gear.

Section 6: ACCIDENTAL RELEASE MEASURES

Personal precautions, protective equipment and emergency procedures

For indoor, provide adequate ventilation process until the end of working. Deny unnecessary entry other than the people

involved by, for example, using a rope. While working, wear appropriate protective equipments to avoid adhering it on skin,

or inhaling the gas. Work from windward, and retract the people downwind.

Environmental precautions

To be careful not discharged to the environment without being properly handled waste water contaminated.

Methods and materials for contaminent and methods and materials for cleaning up

Sweep up and gather scattered particles, and collect it in an empty airtight container.

Recoverly, neutralization

No information available

Secondary disaster prevention measures

Page 2 / 6

W01W0107-0266 JGHEEN                                        Gadolinium Chloride Hexahydrate, 99.9%

Page 3

Issue Date 05-Mar-2026

Clean contaminated objects and areas thoroughly observing environmental regulations.

Section 7: HANDLING AND STORAGE

Handling

Technical measures

Avoid contact with strong oxidizing agents. Avoids contact with acids. Use with local exhaust ventilation.

Precautions

Do not rough handling containers, such as upsetting, falling, giving a shock, and dragging. Prevent leakage, overflow, and

scattering. Not to generate steam and dust in vain. Seal the container after use. After handling, wash hands and face, and

then gargle. In places other than those specified, should not be smoking or eating and drinking. Should not be brought

contaminated protective equipment and gloves to rest stops. Deny unnecessary entry of non-emergency personnel to the

handling area.

Safety handling precautions

Use personal protective equipment as required. Avoid contact with skin, eyes or clothing.

Storage

Safe storage conditions

Storage conditions             Store away from sunlight in well-ventilated place at room temperature (preferably cool).

Keep container tightly closed.

Safe packaging material       Glass

Incompatible substances          Strong oxidizing agents, Acids

Section 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

Engineering controls

In case of indoor workplace, seal the source or use a local exhaust system. Provide the safety shower facility, and hand-

and eye-wash facility. And display their position clearly.

Exposure limits                    This product, as supplied, does not contain any hazardous materials with occupational

exposure limits established by the region specific regulatory bodies.

Personal protective equipment

Respiratory protection         Dust mask ( JIS T 8151 )

Hand protection               chemical protective gloves  ( JIS T 8116 )

Eye protection                   protective eyeglasses or chemical safety goggles (JIS T 8147)

Skin and body　protection     Long-sleeved work clothes

General hygiene considerations

Handle in accordance with good industrial hygiene and safety practice.

If this product is classified as ”Chemical Substances Hazardous to Skin, etc.”, use appropriate protective equipment to

them.

Section 9: PHYSICAL AND CHEMICAL PROPERTIES

Form

Color                                                   white

Appearance                                                 crystals

Odor                                              no data available

Melting point/freezing point                           609 °C

Boiling point, initial boiling point and boiling range       no data available

Flammability                                        no data available

Evaporation rate:                                    no data available

Flammability (solid, gas):                             no data available

Upper/lower flammability or explosive limits

Upper：                                        no data available

Lower：                                        no data available

Flash point                                         no data available

Page 3 / 6

W01W0107-0266 JGHEEN                                        Gadolinium Chloride Hexahydrate, 99.9%

Page 4

Issue Date 05-Mar-2026

Auto-ignition temperature:                            no data available

Decomposition temperature:                          no data available

pH                                                no data available

Viscosity (coefficient of viscosity)                      no data available

Dynamic viscosity                                   no data available

Solubilities                                               water and Ethanol : soluble .

n-Octanol/water partition coefficient:(log Pow)           no data available

Vapour pressure                                    no data available

Specific Gravity / Relative density                        2.424

Vapour density                                      no data available

Particle characteristics                               no data available

Section 10: STABILITY AND REACTIVITY

Stability

Reactivity                   no data available

Chemical stability              Stable under recommended storage conditions.

Hazardous reactions

None under normal processing

Conditions to avoid

Extremes of temperature and direct sunlight, Moisture

Incompatible materials

Strong oxidizing agents, Acids

Hazardous decomposition products

Metal oxides, Halides

Section 11: TOXICOLOGICAL INFORMATION

*NITE: National Institute of Technology and Evaluation (JAPAN)

https://www.chem-info.nite.go.jp/en/chem/chrip/chrip_search/srhInput

Acute toxicity                                       no data available

Skin irritation/corrosion                              no data available

Serious eye damage/ irritation                         no data available

Respiratory or skin sensitization                       no data available

Reproductive cell mutagenicity                        no data available

Carcinogenicity                                     no data available

Reproductive toxicity                                no data available

STOT-single exposure                               no data available

STOT-repeated exposure                             no data available

Aspiration hazard                                   no data available

Section 12: ECOLOGICAL INFORMATION

*NITE: National Institute of Technology and Evaluation (JAPAN)

https://www.chem-info.nite.go.jp/en/chem/chrip/chrip_search/srhInput

Ecotoxicity                     no data available

Other data                     no data available

Persistence and degradability    No information available

Bioaccumulative potential       No information available

Mobility in soil                No information available

Hazard to the ozone layer       No information available

Page 4 / 6

W01W0107-0266 JGHEEN                                        Gadolinium Chloride Hexahydrate, 99.9%

Page 5

Issue Date 05-Mar-2026

Section 13: DISPOSAL CONSIDERATIONS

Waste from residues

Disposal should be in accordance with applicable regional, national and local laws and regulations.

Contaminated container and contaminated packaging

Disposal should be in accordance with applicable regional, national and local laws and regulations.

Section 14: TRANSPORT INFORMATION

ADR/RID                        Not regulated

UN number                             -

Proper shipping name:

UN classfication

Subsidiary hazard class

Packing group

Marine pollutant              Not applicable

IMDG                           Not regulated

UN number                             -

Proper shipping name:

UN classfication

Subsidiary hazard class

Packing group

Marine pollutant (Sea)         Not applicable

Transport in bulk according to No information available

Annex II of MARPOL 73/78 and

the IBC Code

IATA                            Not regulated

UN number                             -

Proper shipping name:

UN classfication

Subsidiary hazard class

Packing group

Environmentally Hazardous    Not applicable

Substance

Section 15: REGULATORY INFORMATION

Japanese regulations

Fire Service Act               Not applicable

Poisonous and Deleterious    Not applicable

Substances Control Law

Industrial Safety and Health Act Not applicable

Regulations for the carriage    Not applicable

and storage of dangerous

goods in ship

Civil Aeronautics Law         Not applicable

Pollutant Release and Transfer Not applicable

Register Law

(2023.4.1-)

Section 16: OTHER INFORMATION

Key literature references and      NITE: National Institute of Technology and Evaluation (JAPAN)

sources for data etc.                https://www.chem-info.nite.go.jp/en/chem/chrip/chrip_search/srhInput

IATA dangerous Goods Regulations

Page 5 / 6

W01W0107-0266 JGHEEN                                        Gadolinium Chloride Hexahydrate, 99.9%

Page 6

Issue Date 05-Mar-2026

RTECS:Registry of Toxic Effects of Chemical Substances

Japan Industrial Safety and Health Association GHS Model SDS

Dictionary of Synthetic Oraganic Chemistry , SSOCJ, Koudansha Scientific Co.Ltd.

Chemical Dictionary, Kyouritsu Publishing Co., Ltd.

etc

Record of SDS revisions         The following contents were revised. Prodauct and company Identification. Handling and

storage. Exposure controls/personal protection. Stability and reactivity. Ecological

information. Regulatory information.

Disclaimer

This SDS is according to JIS Z 7253: 2019. The information provided in this Safety Data Sheet is correct to the best of our

knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe

handling, and is not to be considered a warranty or quality specification. The information relates only to the specific material

designated and may not be valid for such material used in combination with any other materials or in any process, unless

specified in the text.

GHS Classification is according to JIS Z 7252:2019. *JIS: Japanese Industrial Standards

End of Safety Data Sheet

Page 6 / 6

W01W0107-0266 JGHEEN                                        Gadolinium Chloride Hexahydrate, 99.9%