Gadolinium Doped Ceria SDS

Publisher: Nexceris

Source: https://resds.com/document/gadolinium-doped-ceria-sds-en-0b477e0e01

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Gadolinium Doped Ceria

Nexceris

Part Number: Not Available                                                                                                           Issue Date: 06/12/2025

Version No: 1.2                                                                                                                   Revision Date: 06/12/2025

Safety Data Sheet according to OSHA HazCom Standard (2024) requirements                                                                       Print Date: 06/12/2025

L.GHS.USA.EN

SECTION 1 Identification

Product Identifier

Product name    Gadolinium Doped Ceria

Synonyms   GDC, CGO

Other means of

Not Available

identification

Recommended use of the chemical and restrictions on use

Relevant identified uses    Laboratory chemicals and manufacture of substances

Name, address, and telephone number of the chemical manufacturer, importer, or other responsible party

Registered company name    Nexceris

Address   404 Enterprise Drive Lewis Center Ohio 43035 United States

Telephone    +1-614-842-6606

Fax    Not Available

Website    www.nexceris.com

Email    info@nexceris.com

Emergency phone number

Association / Organisation    InfoTrac

Emergency telephone

+1-800-535-5053

number(s)

Other emergency

Not Available

telephone number(s)

SECTION 2 Hazard(s) identification

Classification of the substance or mixture

NFPA 704 diamond

Note: The hazard category numbers found in GHS classification in

section 2 of this SDSs are NOT to be used to fill in the NFPA 704

diamond. Blue = Health Red = Fire Yellow = Reactivity White =

Special (Oxidizer or water reactive substances)

Acute Toxicity (Oral) Category 4, Skin Corrosion/Irritation Category 2, Serious Eye Damage/Eye Irritation Category 2A, Specific

Classification

Target Organ Toxicity - Single Exposure (Respiratory Tract Irritation) Category 3

Label elements

Hazard pictogram(s)

Page 1 continued...

Page 2

Part Number: Not Available                                           Page 2 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Signal word   Warning

Hazard statement(s)

H302    Harmful if swallowed.

H315   Causes skin irritation.

H319   Causes serious eye irritation.

H335   May cause respiratory irritation.

Hazard(s) not otherwise classified

Not Applicable

Precautionary statement(s) Prevention

P271   Use only outdoors or in a well-ventilated area.

P261    Avoid breathing dust/fumes.

P264   Wash all exposed external body areas thoroughly after handling.

P270   Do not eat, drink or smoke when using this product.

P280   Wear protective gloves, protective clothing, eye protection and face protection.

Precautionary statement(s) Response

P305+P351+P338    IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P312    Call a POISON CENTER/doctor/physician/first aider/if you feel unwell.

P337+P313       If eye irritation persists: Get medical advice/attention.

P301+P312    IF SWALLOWED: Call a POISON CENTER/doctor/physician/first aider/if you feel unwell.

P302+P352    IF ON SKIN: Wash with plenty of water and soap.

P304+P340    IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P330    Rinse mouth.

P332+P313       If skin irritation occurs: Get medical advice/attention.

P362+P364   Take off contaminated clothing and wash it before reuse.

Precautionary statement(s) Storage

P405    Store locked up.

P403+P233    Store in a well-ventilated place. Keep container tightly closed.

Precautionary statement(s) Disposal

P501    Dispose of contents/container to authorised hazardous or special waste collection point in accordance with any local regulation.

SECTION 3 Composition / information on ingredients

Substances

See section below for composition of Mixtures

Mixtures

CAS No                     %[weight]                  Name

1306-38-3                    80-100                          Cerium Oxide (1306-38-3)

12064-62-9                   10-30                             Gadolinium Oxide (12064-62-9)

The specific chemical identity and/or exact percentage (concentration) of composition has been withheld as a trade secret.

SECTION 4 First-aid measures

Description of first aid measures

If this product comes in contact with the eyes:

Wash out immediately with fresh running water.

Ensure complete irrigation of the eye by keeping eyelids apart and away from eye and moving the eyelids by occasionally

Eye Contact

lifting the upper and lower lids.

Seek medical attention without delay; if pain persists or recurs seek medical attention.

Removal of contact lenses after an eye injury should only be undertaken by skilled personnel.

Skin Contact       If skin contact occurs:

Continued...

Page 3

Part Number: Not Available                                           Page 3 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Immediately remove all contaminated clothing, including footwear.

Flush skin and hair with running water (and soap if available).

Seek medical attention in event of irritation.

If fumes, aerosols or combustion products are inhaled remove from contaminated area.

Inhalation

Other measures are usually unnecessary.

Immediately give a glass of water.

Ingestion

First aid is not generally required. If in doubt, contact a Poisons Information Centre or a doctor.

Most important symptoms and effects, both acute and delayed

See Section 11

Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

As in all cases of suspected poisoning, follow the ABCDEs of emergency medicine (airway, breathing, circulation, disability, exposure), then the ABCDEs of

toxicology (antidotes, basics, change absorption, change distribution, change elimination).

For poisons (where specific treatment regime is absent):

--------------------------------------------------------------

BASIC TREATMENT

--------------------------------------------------------------

Establish a patent airway with suction where necessary.

Watch for signs of respiratory insufficiency and assist ventilation as necessary.

Administer oxygen by non-rebreather mask at 10 to 15 L/min.

Monitor and treat, where necessary, for pulmonary oedema.

Monitor and treat, where necessary, for shock.

Anticipate seizures.

DO NOT use emetics. Where ingestion is suspected rinse mouth and give up to 200 ml water (5 ml/kg recommended) for dilution where patient is able to

swallow, has a strong gag reflex and does not drool.

--------------------------------------------------------------

ADVANCED TREATMENT

--------------------------------------------------------------

Consider orotracheal or nasotracheal intubation for airway control in unconscious patient or where respiratory arrest has occurred.

Positive-pressure ventilation using a bag-valve mask might be of use.

Monitor and treat, where necessary, for arrhythmias.

Start an IV D5W TKO. If signs of hypovolaemia are present use lactated Ringers solution. Fluid overload might create complications.

Drug therapy should be considered for pulmonary oedema.

Hypotension with signs of hypovolaemia requires the cautious administration of fluids. Fluid overload might create complications.

Treat seizures with diazepam.

Proparacaine hydrochloride should be used to assist eye irrigation.

BRONSTEIN, A.C. and CURRANCE, P.L.

EMERGENCY CARE FOR HAZARDOUS MATERIALS EXPOSURE: 2nd Ed. 1994

SECTION 5 Fire-fighting measures

Extinguishing media

There is no restriction on the type of extinguisher which may be used.

Use extinguishing media suitable for surrounding area.

Special hazards arising from the substrate or mixture

Fire Incompatibility   None known.

Special protective equipment and precautions for fire-fighters

Alert Fire Brigade and tell them location and nature of hazard.

Wear breathing apparatus plus protective gloves in the event of a fire.

Prevent, by any means available, spillage from entering drains or water courses.

Use fire fighting procedures suitable for surrounding area.

Fire Fighting

DO NOT approach containers suspected to be hot.

Cool fire exposed containers with water spray from a protected location.

If safe to do so, remove containers from path of fire.

Equipment should be thoroughly decontaminated after use.

Under certain conditions the material may become combustible because of the ease of ignition which occurs after the material

reaches a high specific area ratio (thin sections, fine particles, or molten states). However, the same material in massive solid

form is comparatively difficult to ignite. Nearly all metals will burn in air under certain conditions. Some are oxidised rapidly in the

presence of air or moisture, generating sufficient heat to reach their ignition temperatures.

Others oxidise so slowly that heat generated during oxidation is dissipated before the metal becomes hot enough to ignite.

Fire/Explosion Hazard    Particle size, shape, quantity, and alloy are important factors to be considered when evaluating metal combustibility.

Combustibility of metallic alloys may differ and vary widely from the combustibility characteristics of the alloys' constituent

elements.

Decomposition may produce toxic fumes of:

metal oxides

May emit poisonous fumes.

Continued...

Page 4

Part Number: Not Available                                           Page 4 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

SECTION 6 Accidental release measures

Personal precautions, protective equipment and emergency procedures

See section 8

Environmental precautions

See section 12

Methods and material for containment and cleaning up

Clean up all spills immediately.

Avoid breathing vapours/ aerosols/ or dusts and avoid contact with skin and eyes.

Minor Spills        Control personal contact with the substance, by using protective equipment.

Contain and absorb spill with sand, earth, inert material or vermiculite.

Place in a suitable, labelled container for waste disposal.

Clear area of personnel and move upwind.

Alert Fire Brigade and tell them location and nature of hazard.

Wear breathing apparatus plus protective gloves.

Prevent, by any means available, spillage from entering drains or water course.

Stop leak if safe to do so.

Contain spill with sand, earth or vermiculite.

Major Spills

Collect recoverable product into labelled containers for recycling.

Neutralise/decontaminate residue (see Section 13 for specific agent).

Collect solid residues and seal in labelled drums for disposal.

Wash area and prevent runoff into drains.

After clean up operations, decontaminate and launder all protective clothing and equipment before storing and re-using.

If contamination of drains or waterways occurs, advise emergency services.

Personal Protective Equipment advice is contained in Section 8 of the SDS.

SECTION 7 Handling and storage

Precautions for safe handling

Avoid all personal contact, including inhalation.

Wear protective clothing when risk of exposure occurs.

Use in a well-ventilated area.

Prevent concentration in hollows and sumps.

DO NOT enter confined spaces until atmosphere has been checked.

DO NOT allow material to contact humans, exposed food or food utensils.

Avoid contact with incompatible materials.

When handling, DO NOT eat, drink or smoke.

Safe handling

Keep containers securely sealed when not in use.

Avoid physical damage to containers.

Always wash hands with soap and water after handling.

Work clothes should be laundered separately. Launder contaminated clothing before re-use.

Use good occupational work practice.

Observe manufacturer's storage and handling recommendations contained within this SDS.

Atmosphere should be regularly checked against established exposure standards to ensure safe working conditions are

maintained.

Other information

Conditions for safe storage, including any incompatibilities

Polyethylene or polypropylene container.

Suitable container       Packing as recommended by manufacturer.

Check all containers are clearly labelled and free from leaks.

Storage incompatibility    Derivative of very electropositive metal.

+          x          +          o          +          +          +

X — Must not be stored together

0 — May be stored together with specific preventions

+ — May be stored together

Note: Depending on other risk factors, compatibility assessment based on the table above may not be relevant to storage situations, particularly where large volumes

of dangerous goods are stored and handled. Reference should be made to the Safety Data Sheets for each substance or article and risks assessed accordingly.

SECTION 8 Exposure controls / personal protection

Continued...

Page 5

Part Number: Not Available                                           Page 5 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Control parameters

Occupational Exposure Limits (OEL)

INGREDIENT DATA

Not Available

Emergency Limits

Ingredient                 TEEL-1                            TEEL-2                            TEEL-3

Gadolinium Doped Ceria       Not Available                           Not Available                           Not Available

Ingredient                     Original IDLH                                          Revised IDLH

Gadolinium Doped Ceria       Not Available                                             Not Available

MATERIAL DATA

as cerium

CEL TWA: 1 mg/m3 (compare TLV-TWA yttrium)

(CEL=Chemwatch Exposure Limit)

Exposure to the vapours of some rare earth salts reportedly produces sensitivity to heat, itching and an increased perception of odour and taste. Other effects may

include bronchiolitis, subacute bronchitis, acute transient chemical pneumonitis, focal hypertrophic emphysema, regional bronchiolar stricturing and cellular

eosinophilia. In rare fatal cases of exposure to the rare-earth fluoride and/or oxide mixtures, delayed chemical hyperaemia has occurred. Lung granulomas have

also been seen in experimental animals.

as gadolinium

CEL TWA: 1 mg/m3 (compare TLV-TWA yttrium)

(CEL = Chemwatch Exposure Limit)

Exposure to the vapours of some rare earth salts reportedly produces sensitivity to heat, itching and an increased perception of odour and taste. Other effects may

include bronchiolitis, subacute bronchitis, acute transient chemical pneumonitis, focal hypertrophic emphysema, regional bronchiolar stricturing and cellular

eosinophilia. In rare fatal cases of exposure to the rare-earth fluoride and/or oxide mixtures, delayed chemical hyperaemia has occurred. Lung granulomas have

also been seen in experimental animals.

The concentration of dust, for application of respirable dust limits, is to be determined from the fraction that penetrates a separator whose size collection efficiency

is described by a cumulative log-normal function with a median aerodynamic diameter of 4.0 um (+-) 0.3 um and with a geometric standard deviation of 1.5 um

(+-) 0.1 um, i.e..generally less than 5 um.

Exposure controls

Appropriate engineering    Engineering controls are used to remove a hazard or place a barrier between the worker and the hazard. Well-designed

controls    engineering controls can be highly effective in protecting workers and will typically be independent of worker interactions to

provide this high level of protection.

The basic types of engineering controls are:

Process controls which involve changing the way a job activity or process is done to reduce the risk.

Enclosure and/or isolation of emission source which keeps a selected hazard "physically" away from the worker and ventilation

that strategically "adds" and "removes" air in the work environment. Ventilation can remove or dilute an air contaminant if

designed properly. The design of a ventilation system must match the particular process and chemical or contaminant in use.

Employers may need to use multiple types of controls to prevent employee overexposure.

General exhaust is adequate under normal operating conditions. If risk of overexposure exists, wear SAA approved respirator.

Correct fit is essential to obtain adequate protection. Provide adequate ventilation in warehouse or closed storage areas. Air

contaminants generated in the workplace possess varying "escape" velocities which, in turn, determine the "capture velocities" of

fresh circulating air required to effectively remove the contaminant.

Type of Contaminant:                                                                                               Air Speed:

0.25-0.5 m/s (50-

solvent, vapours, degreasing etc., evaporating from tank (in still air)

100 f/min)

aerosols, fumes from pouring operations, intermittent container filling, low speed conveyer transfers,         0.5-1 m/s (100-

welding, spray drift, plating acid fumes, pickling (released at low velocity into zone of active generation)     200 f/min.)

direct spray, spray painting in shallow booths, drum filling, conveyer loading, crusher dusts, gas discharge   1-2.5 m/s (200-

(active generation into zone of rapid air motion)                                                 500 f/min)

grinding, abrasive blasting, tumbling, high speed wheel generated dusts (released at high initial velocity      2.5-10 m/s (500-

into zone of very high rapid air motion).                                                        2000 f/min.)

Within each range the appropriate value depends on:

Lower end of the range                            Upper end of the range

1: Room air currents minimal or favourable to capture     1: Disturbing room air currents

2: Contaminants of low toxicity or of nuisance value only  2: Contaminants of high toxicity

3: Intermittent, low production.                              3: High production, heavy use

4: Large hood or large air mass in motion                  4: Small hood - local control only

Simple theory shows that air velocity falls rapidly with distance away from the opening of a simple extraction pipe. Velocity

generally decreases with the square of distance from the extraction point (in simple cases). Therefore the air speed at the

Continued...

Page 6

Part Number: Not Available                                           Page 6 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

extraction point should be adjusted, accordingly, after reference to distance from the contaminating source. The air velocity at the

extraction fan, for example, should be a minimum of 1-2 m/s (200-400 f/min.) for extraction of solvents generated in a tank 2

meters distant from the extraction point. Other mechanical considerations, producing performance deficits within the extraction

apparatus, make it essential that theoretical air velocities are multiplied by factors of 10 or more when extraction systems are

installed or used.

Individual protection

measures, such as

personal protective

equipment

Safety glasses with side shields

Chemical goggles. [AS/NZS 1337.1, EN166 or national equivalent]

Contact lenses may pose a special hazard; soft contact lenses may absorb and concentrate irritants. A written policy

document, describing the wearing of lenses or restrictions on use, should be created for each workplace or task. This should

Eye and face protection        include a review of lens absorption and adsorption for the class of chemicals in use and an account of injury experience.

Medical and first-aid personnel should be trained in their removal and suitable equipment should be readily available. In the

event of chemical exposure, begin eye irrigation immediately and remove contact lens as soon as practicable. Lens should

be removed at the first signs of eye redness or irritation - lens should be removed in a clean environment only after workers

have washed hands thoroughly. [CDC NIOSH Current Intelligence Bulletin 59].

Skin protection   See Hand protection below

Wear chemical protective gloves, e.g. PVC.

Hands/feet protection

Wear safety footwear or safety gumboots, e.g. Rubber

Body protection   See Other protection below

Overalls.

P.V.C apron.

Other protection        Barrier cream.

Skin cleansing cream.

Eye wash unit.

Respiratory protection

Type -P Filter of sufficient capacity. (AS/NZS 1716 & 1715, EN 143:2000 & 149:2001, ANSI Z88 or national equivalent)

Required Minimum Protection Factor                Half-Face Respirator          Full-Face Respirator        Powered Air Respirator

P1                                         -                        PAPR-P1

up to 10 x ES

Air-line*                                  -                                           -

up to 50 x ES                                                  Air-line**                  P2                      PAPR-P2

up to 100 x ES                                                         -                        P3                                        -

Air-line*                                  -

100+ x ES                                                               -                                     Air-line**                  PAPR-P3

* - Negative pressure demand ** - Continuous flow

A(All classes) = Organic vapours, B AUS or B1 = Acid gasses, B2 = Acid gas or hydrogen cyanide(HCN), B3 = Acid gas or hydrogen cyanide(HCN), E = Sulfur

dioxide(SO2), G = Agricultural chemicals, K = Ammonia(NH3), Hg = Mercury, NO = Oxides of nitrogen, MB = Methyl bromide, AX = Low boiling point organic

compounds(below 65 degC)

· Respirators may be necessary when engineering and administrative controls do not adequately prevent exposures.

· The decision to use respiratory protection should be based on professional judgment that takes into account toxicity information, exposure measurement data,

and frequency and likelihood of the worker's exposure - ensure users are not subject to high thermal loads which may result in heat stress or distress due to

personal protective equipment (powered, positive flow, full face apparatus may be an option).

· Published occupational exposure limits, where they exist, will assist in determining the adequacy of the selected respiratory protection. These may be

government mandated or vendor recommended.

· Certified respirators will be useful for protecting workers from inhalation of particulates when properly selected and fit tested as part of a complete respiratory

protection program.

· Where protection from nuisance levels of dusts are desired, use type N95 (US) or type P1 (EN143) dust masks. Use respirators and components tested and

approved under appropriate government standards such as NIOSH (US) or CEN (EU)

· Use approved positive flow mask if significant quantities of dust becomes airborne.

· Try to avoid creating dust conditions.

Class P2 particulate filters are used for protection against mechanically and thermally generated particulates or both.

P2 is a respiratory filter rating under various international standards, Filters at least 94% of airborne particles

Suitable for:

· Relatively small particles generated by mechanical processes eg. grinding, cutting, sanding, drilling, sawing.

· Sub-micron thermally generated particles e.g. welding fumes, fertilizer and bushfire smoke.

· Biologically active airborne particles under specified infection control applications e.g. viruses, bacteria, COVID-19, SARS

SECTION 9 Physical and chemical properties

Information on basic physical and chemical properties

Appearance    Beige

Continued...

Page 7

Part Number: Not Available                                           Page 7 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Relative density (Water =

Physical state   Powder                                                                  Not Available

1)

Partition coefficient n-

Odour    Not Available                                                             Not Available

octanol / water

Auto-ignition temperature

Odour threshold    Not Available                                                             Not Available

(°C)

Decomposition

pH (as supplied)    Not Available                                                             Not Available

temperature (°C)

Melting point / freezing

Not Available                                               Viscosity (cSt)    Not Available

point (°C)

Initial boiling point and

Not Available                                   Molecular weight (g/mol)    Not Available

boiling range (°C)

Flash point (°C)    Not Available                                                      Taste    Not Available

Evaporation rate    Not Available                                       Explosive properties    Not Available

Flammability    Not Applicable                                       Oxidising properties    Not Available

Surface Tension (dyn/cm

Upper Explosive Limit (%)    Not Available                                                             Not Available

or mN/m)

Lower Explosive Limit (%)    Not Available                                        Volatile Component (%vol)    Not Available

Vapour pressure (kPa)    Not Available                                        Gas group    Not Available

Solubility in water    Immiscible                                pH as a solution (1%)    Not Available

Vapour density (Air = 1)    Not Available                                    VOC g/L    Not Available

Heat of Combustion (kJ/g)    Not Available                                           Ignition Distance (cm)    Not Available

Flame Height (cm)    Not Available                                     Flame Duration (s)    Not Available

Enclosed Space Ignition

Enclosed Space Ignition

Not Available                                          Deflagration Density    Not Available

Time Equivalent (s/m3)

(g/m3)

Nanoform Particle

Nanoform Solubility    Not Available                                                             Not Available

Characteristics

Particle Size    Not Available

SECTION 10 Stability and reactivity

Reactivity   See section 7

Chemical stability    Product is considered stable and hazardous polymerisation will not occur.

Possibility of hazardous

See section 7

reactions

Conditions to avoid   See section 7

Incompatible materials   See section 7

Hazardous decomposition

See section 5

products

SECTION 11 Toxicological information

Information on toxicological effects

a) Acute Toxicity    There is sufficient evidence to classify this material as acutely toxic.

b) Skin Irritation/Corrosion    There is sufficient evidence to classify this material as skin corrosive or irritating.

c) Serious Eye

There is sufficient evidence to classify this material as eye damaging or irritating

Damage/Irritation

d) Respiratory or Skin

Based on available data, the classification criteria are not met.

sensitisation

e) Mutagenicity   Based on available data, the classification criteria are not met.

f) Carcinogenicity   Based on available data, the classification criteria are not met.

g) Reproductivity   Based on available data, the classification criteria are not met.

h) STOT - Single Exposure    There is sufficient evidence to classify this material as toxic to specific organs through single exposure

i) STOT - Repeated

Based on available data, the classification criteria are not met.

Exposure

j) Aspiration Hazard   Based on available data, the classification criteria are not met.

Inhaled   The material is not thought to produce either adverse health effects or irritation of the respiratory tract following inhalation (as

classified by EC Directives using animal models). Nevertheless, adverse systemic effects have been produced following

Continued...

Page 8

Part Number: Not Available                                           Page 8 of 13                                                       Issue Date: 06/12//2025

Version No: 1.2                                                                                                                               Revision Date: 06/12//2025                                                   Gadolinium Doped Ceria

Print Date: 06/12//2025

exposure of animals by at least one other route and good hygiene practice requires that exposure be kept to a minimum and that

suitable control measures be used in an occupational setting.

The toxicology of rare earth metal oxides has been determined by pathological and biochemical examination of rodents exposed

to the oxides by oral, intraperitoneal or endotracheal routes. Weakly expressed general toxic action of the oxides is seen in acute

and prolonged exposure. The dusts cause pronounced changes in the lungs. (The oxides of the rare earth metals are

significantly less toxic than their salts.)

Symptoms of exposure to rare earth oxides are coughing, congestion, granuloma in lungs and haemoglobinaemia.

Rare earths may cause impairment of blood clotting.

Exposure to rare earth oxide vapours has been reported to result in sensitivity to heat, itching, and an increased awareness of

odour and taste, bronchiolitis, sub-acute bronchiolitis (inflammation of the bronchial tubes), acute transient chemical pneumonitis

(inflammation of the lungs caused by chemical irritation), focal hypertrophia (excessive development of an organ), emphysema,

regional bronchiolar stricturing, cellular eosinophilia (abnormal increase in the number of leucocytes with cytoplasmic inclusions,

in the blood that is characteristic of allergic reactions), and, in some cases, fatal delayed chemical hyperemia (excess of blood in

a body part).

Intratracheal administration to animals of some rare earth oxides, has been reported to cause changes ranging from mild to

marked fibrosis (a condition marked by the increase of interstitial fibrous tissue), emphysema ( a condition of the lungs marked

by abnormal dilation of the its air spaces and distension of its walls), small white nodules, granulomas (a mass or nodule of

chronically inflamed tissue with granulations that are generally associated with an infective process), giant cells, and

accumulation of dust in the lungs.

In rare fatal cases of exposure to the rare-earth fluoride and/or oxide mixtures, delayed chemical hyperaemia has occurred. Lung

granulomas have also been seen in experimental animals.

Inhalation of dusts, generated by the material during the course of normal handling, may be damaging to the health of the

individual.

Effects on lungs are significantly enhanced in the presence of respirable particles. Overexposure to respirable dust may produce

wheezing, coughing and breathing difficulties leading to or symptomatic of impaired respiratory function.

Gadolinium is a member of the so-called heavy-group (the yttriums) of the rare earths (or lanthanoids). No occupational diseases

or cases of poisoning in workers producing rare earth elements have been described. Lanthanoids entering the human body due

to exposure to various industrial processes can affect metabolic processes. Trivalent lanthanoid ions, especially lanthanum 3+

and gadolinium 3+, can interfere with calcium channels in human and animal cells. Lanthanoids can also alter or even inhibit the

action of various enzymes. Lanthanoid ions found in neurons can regulate synaptic transmission, as well as block some

receptors (for example, glutamate receptors). Lanthanoids target the liver causing fatty liver degeneration and a decrease in liver

glycogen and blood glucose levels.

Lanthanoids because of their high density can produce significant abnormalities on chest X-rays but these lesions typically have

little or no clinical importance and generally are not felt to be fibrogenic

The toxicity of all elements in the yttrium group has been investigated in workers and animals alike. Effects on peripheral blood

including a decrease haemoglobin and erythrocyte content and changes in the leucocyte formula have been recorded. Animal

lungs show productive inflammation and a tendency to develop nodular or diffuse sclerosis following administration by

intratracheal injection. The main risks to workers involved in the production of rare earths are due to dust inhalation.

A risk of nephrogenic systemic fibrosis (NSF) with gadolinium-based contrast agents (GBCAs) has been described. This relates

to patients with acute or chronic severe renal insufficiency (glomerular filtration rate <30 mL/min/1.73m2) and patients with acute

renal insufficiency of any severity due to the hepato-renal syndrome or in the peri-operative liver transplantation period.

Healthcare professionals should avoid the use of a GBCA in these patients unless the diagnostic information is essential and not

available with non-contrast enhanced magnetic resonance imaging.

Ingestion   Based on the available toxicity data, the rare earth chlorides appear to have moderate acute and chronic toxicity. However these

substances cause severe eye irritation and severe irritation in abraded skin. They are poorly absorbed by the gastrointestinal

tract and by unbroken skin but slight liver damage has been demonstrated in subchronic oral toxicity studies at high doses. The

literature indicates that chronic inhalation exposure to the rare earth chlorides may cause pneumoconiosis in humans. There are

no indications of carcinogenicity in the rare earth chlorides. Mutagenicity studies on these substances have mixed results, but

are predominantly negative.

* IUPAC currently recommends the name lanthanoid rather than lanthanide, as the suffix "-ide" generally indicates negative ions

whereas the suffix "-oid" indicates similarity to one of the members of the containing family of elements. In the older literature, the

name "lanthanon" was often used.

The material has NOT been classified by EC Directives or other classification systems as "harmful by ingestion". This is because

of the lack of corroborating animal or human evidence. The material may still be damaging to the health of the individual,

following ingestion, especially where pre-existing organ (e.g liver, kidney) damage is evident. Present definitions of harmful or

toxic substances are generally based on doses producing mortality rather than those producing morbidity (disease, ill-health).

Gastrointestinal tract discomfort may produce nausea and vomiting. In an occupational setting however, ingestion of insignificant

quantities is not thought to be cause for concern.

Symptoms of acute lanthanide toxicity in rats are immediate defecation, writhing, ataxia (the inability to coordinate voluntary

muscular movement), sedation, laboured respiration and reduced activity. Death is due mainly to respiratory and cardiac failure.

The rare earths exhibit low toxicity following ingestion but may be toxic by the intraperitoneal route and mildly toxic when

administered by the subcutaneous route.The production of skin and lung granulomas, following exposure, may also occur.

Accidental ingestion of the material may be harmful; animal experiments indicate that ingestion of less than 150 gram may be

fatal or may produce serious damage to the health of the individual.

Skin Contact

The material is not thought to be a skin irritant (i.e. is unlikely to produce irritant dermatitis as described in EC Directives using

animal models). Temporary discomfort, however, may result from prolonged dermal exposures. Good hygiene practice requires

that exposure be kept to a minimum and that suitable gloves be used in an occupational setting.

Skin contact with the material may damage the health of the individual; systemic effects may result following absorption.

Open cuts, abraded or irritated skin should not be exposed to this material

Entry into the blood-stream through, for example, cuts, abrasions, puncture wounds or lesions, may produce systemic injury with

harmful effects. Examine the skin prior to the use of the material and ensure that any external damage is suitably protected.

Continued...

Page 9

Part Number: Not Available                                           Page 9 of 13                                                        Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Absorption by skin may readily exceed vapour inhalation exposure. Symptoms for skin absorption are the same as for inhalation.

Eye    Although the material is not thought to be an irritant (as classified by EC Directives), direct contact with the eye may produce

transient discomfort characterised by tearing or conjunctival redness (as with windburn).

Long-term exposure to the product is not thought to produce chronic effects adverse to health (as classified by EC Directives

using animal models); nevertheless exposure by all routes should be minimised as a matter of course.

Toxic: danger of serious damage to health by prolonged exposure through inhalation, in contact with skin and if swallowed.

Serious damage (clear functional disturbance or morphological change which may have toxicological significance) is likely to be

caused by repeated or prolonged exposure. As a rule the material produces, or contains a substance which produces severe

lesions. Such damage may become apparent following direct application in subchronic (90 day) toxicity studies or following sub-

acute (28 day) or chronic (two-year) toxicity tests.

Cerium is a member of the so-called light-group (the ceriums) of the rare earths (or lanthanoids* - previously lanthanides). No

occupational diseases or cases of poisoning in workers producing rare earth elements have been described.

Lanthanoids entering the human body due to exposure to various industrial processes can affect metabolic processes. Trivalent

lanthanoid ions, especially lanthanum 3+ and gadolinium 3+, can interfere with calcium channels in human and animal cells.

Lanthanoids can also alter or even inhibit the action of various enzymes. Lanthanoid ions found in neurons can regulate synaptic

transmission, as well as block some receptors (for example, glutamate receptors). Lanthanoids target the liver causing fatty liver

degeneration and a decrease in liver glycogen and blood glucose levels.

Lanthanoids because of their high density can produce significant abnormalities on chest X-rays but these lesions typically have

little or no clinical importance. Lanthanoids generally are not felt to be fibrogenic Occasional cases of suspected pneumoconiosis

Chronic   have however been reported. The toxicity of all elements in the cerium group has been investigated and found to be insignificant.

The respiratory tracts of rats show pathogenic effects when injected intratracheally (the oxides produce a lesser effect than the

salts). The main risks to workers involved in the production of rare earths are due to dust inhalation. "Cer-pneumoconiosis" has

been reported as a lung condition found in graphic arts workers who use ceria-enhanced carbon arc lights in their occupation.

Systemically cerium ion induces hepatic (liver) damage. This is reflected by massive lipid accumulation, death of individual liver

cells and binding of the ion to the endoplasmic reticulum. Serum levels of cholesterol and the transaminases rise whilst those of

the triglycerides fall following such damage.

Based on the available toxicity data, the rare earth chlorides appear to have moderate acute and chronic toxicity. However these

substances cause severe eye irritation and severe irritation in abraded skin. They are poorly absorbed by the gastrointestinal

tract and by unbroken skin but slight liver damage has been demonstrated in subchronic oral toxicity studies at high doses. The

literature indicates that chronic inhalation exposure to the rare earth chlorides may cause pneumoconiosis in humans. There are

no indications of carcinogenicity in the rare earth chlorides. Mutagenicity studies on these substances have mixed results, but

are predominantly negative.

* IUPAC currently recommends the name lanthanoid rather than lanthanide, as the suffix "-ide" generally indicates negative ions

whereas the suffix "-oid" indicates similarity to one of the members of the containing family of elements. In the older literature, the

name "lanthanon" was often used.

TOXICITY                                           IRRITATION

Gadolinium Doped Ceria

Not Available                                            Not Available

Legend:    1. Value obtained from Europe ECHA Registered Substances - Acute toxicity 2. Value obtained from manufacturer's SDS.

Unless otherwise specified data extracted from RTECS - Register of Toxic Effect of chemical Substances

Symptoms of acute lanthanide toxicity in rats are immediate defecation, writhing, ataxia (the inability to coordinate voluntary

muscular movement), sedation, laboured respiration and reduced activity. Death is due mainly to respiratory and cardiac failure.

The rare earths exhibit low toxicity following ingestion but may be toxic by the intraperitoneal route and mildly toxic when

administered by the subcutaneous route.The production of skin and lung granulomas, following exposure, may also occur.

for typical lanthanides:

The symptoms of toxicity of the rare earth elements include writhing, ataxia, labored respiration, walking on the toes with arched

back and sedation. The rare earth elements exhibit low toxicity by ingestion exposure. However, the intraperitoneal route may be

highly toxic while the subcutaneous route is poison to moderately toxic. The production of skin and lung granulomas after

exposure to them requires extensive protection to prevent such exposure.

Gadolinium Doped Ceria   Chronic Inhalation Toxicity: An accumulation of insoluble lanthanide particles has been observed in the respiratory tract of

humans following chronic occupational exposure and in rodents following chronic exposure to a similar lanthanide cerium oxide.

Lymphoid hyperplasia in the bronchial lymph nodes was the critical inhalation health effect identified by the USEPA in a 2008

toxicological review of cerium oxide.

Developmental/Reproductive Toxicity: Lanthanum carbonate, did not affect fertility or produce any harm to the fetus in a rat

study.

Mutagenicity: Cerium oxide, was negative in the Ames bacterial mutagenic test using bacterial strains TA135, TA1537, TA98,

TA100, TA102, and WP2uvrA., and in the mouse in vivo micronucleus assay.

Carcinogenicity: Lanthanum carbonate, was not carcinogenic in a two-year oral rat study. Not assessed by IARC, NTP, or

USEPA.

Acute Toxicity                                                           Carcinogenicity

Skin Irritation/Corrosion                                                            Reproductivity

Serious Eye

STOT - Single Exposure

Damage/Irritation

Respiratory or Skin

STOT - Repeated Exposure

sensitisation

Continued...

Page 10

Part Number: Not Available                                           Page 10 of 13                                                       Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Mutagenicity                                                         Aspiration Hazard

Legend:      – Data either not available or does not fill the criteria for classification

– Data available to make classification

SECTION 12 Ecological information

Toxicity

Endpoint    Test Duration (hr)                     Species                               Value      Source

Gadolinium Doped Ceria     Not                                                                                     Not        Not

Not Available                           Not Available

Available                                                                                             Available    Available

Legend:    Extracted from 1. IUCLID Toxicity Data 2. Europe ECHA Registered Substances - Ecotoxicological Information - Aquatic Toxicity

4. US EPA, Ecotox database - Aquatic Toxicity Data 5. ECETOC Aquatic Hazard Assessment Data 6. NITE (Japan) -

Bioconcentration Data 7. METI (Japan) - Bioconcentration Data 8. Vendor Data

For Metal:

Atmospheric Fate - Metal-containing inorganic substances generally have negligible vapour pressure and are not expected to partition to air.

Environmental Fate: Environmental processes, such as oxidation, the presence of acids or bases and microbiological processes, may transform insoluble metals

to more soluble ionic forms. Environmental processes may enhance bioavailability and may also be important in changing solubilities.

Aquatic/Terrestrial Fate: When released to dry soil, most metals will exhibit limited mobility and remain in the upper layer; some will leach locally into ground water

and/ or surface water ecosystems when soaked by rain or melt ice. A metal ion is considered infinitely persistent because it cannot degrade further. Once released

to surface waters and moist soils their fate depends on solubility and dissociation in water. A significant proportion of dissolved/ sorbed metals will end up in

sediments through the settling of suspended particles. The remaining metal ions can then be taken up by aquatic organisms. Ionic species may bind to dissolved

ligands or sorb to solid particles in water.

Ecotoxicity: Even though many metals show few toxic effects at physiological pH levels, transformation may introduce new or magnified effects.

Crops can take up cerium. Cerium has affinity for humic substances that may alter its availability in aquatic systems. Current fate and transport studies are limited

and may not adequately address long term environmental exposure risks to both humans and other organisms. Although cerium has low acute toxicity, long term

health and environmental effects are less well understood.

The form cerium takes can also influence its biological and environmental fate. Oxides and hydroxides of cerium are poorly soluble in body fluids and are slow to

clear from the organism. Cerium can affect the respiratory tract and associated lymph nodes (inhalation exposure) and once in the circulatory system can partition

to the skeleton, liver, kidney and spleen. Studies subjecting animals to large dosages of cerium show evidence of neurological effects, possibly due to cerium

competing with calcium binding sites in the brain. Long term human expose to cerium is correlated with rare earth pneumoconiosis, but the precise role of cerium

in this disease is not well characterized due to confounding metal aerosols

For lanthanoids (formerly lanthanides; syn rare earth metals and their salts):

Environmental fate:

The natural occurrence of rare earths in the lithosphere is well established at a concentration level of a few hundred part per million. They are therefore not "rare".

Rare earth chlorides are very poorly soluble in water. Modeled water solubilities range from 10-2 to 10-5 mg/l. They are expected to strongly sorb to soil and not

expected to volatilise.

Water: Lanthanoid emissions to the environment increase as a result of the growing industrial applications of these elements. However, robust data to evaluate the

environmental fate of lanthanoids are scarce.

Changing environmental conditions may influence the fate and bioavailability of lanthanoids (part of the rare earth elements [Ln]) in estuaries.Equilibrium model

calculations indicate that dissolved lanthanoids are complexed mainly to carbonates and dissolved organic matter. In the water phase, the relative abundance of

the free ion, LnCO3, and humic complexes decreases from lanthanum to lutetium, whereas the relative abundance of Ln(CO3)2 increases. Cerium and europium

anomalies were found in water. Europium anomalies were also found in some biota. The biota sediment accumulation factors (BSAFs) decreased across the

series from lanthanum to lutetium. Regression analysis revealed that alkalinity correlated negatively with lanthanide uptake. This suggests that increasing

complexation reduced bioavailability under the prevailing conditions. The BSAFs did not depend on salinity or pH, which may simplify sediment-quality criteria for

fresh versus saline waters. Field BSAFs were significantly lower than laboratory values for the same sediments, which is explained by adaptation of the organisms

to lanthanides.

Plant uptake: Lanthanum concentrations in plants and medium and the amounts sorbed to glass vessels were quantified by using the radioisotope 140La. The

amount of La adsorbed on the glass reached values of 25% of the total La present. A model was formulated to describe La uptake in exponentially growing

duckweed in the presence of an adsorptive surface. Growth-induced dilution appeared more efficient in lowering plant La concentrations than actual elimination.

An elimination study revealed two compartments, of which the smallest eliminated 50 times faster than the bigger compartment, which eliminated mainly by growth

dilution. The average bioconcentration factor was 2,000 L/kg fresh weight and 30,000 L/kg dry weight, comparable with those of other higher plants. At the applied

concentration of 10 nM, no effects were observed on duckweed growth. However, the high bioconcentration factor warrants monitoring of lanthanide emissions.

Ecotoxicity:

For cerium oxide (a typical oxide of this group):

Fish LC50 (96 h): fathead minnow >50000 mg/l (low toxicity)

Green algae IC25: 34484 mg/l (low toxicity)

Daphnia LC50 (48 h): Ceriodaphnia dubia >50000 mg/l (low toxicity)

Rare earth chlorides exhibit acute aquatic toxicity at concentrations exceeding 100 ppm and chronic toxicity, persisting for more than 21 days, at concentrations

greater than 30 ppm (based on structure activity relationships - QSAR). Industrial processes have little impact on altering background levels. Lanthanum 3+ is toxic

to some aquatic organisms.

Continued...

Page 11

Part Number: Not Available                                           Page 11 of 13                                                       Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Persistence and degradability

Ingredient                    Persistence: Water/Soil                                     Persistence: Air

No Data available for all ingredients                     No Data available for all ingredients

Bioaccumulative potential

Ingredient                  Bioaccumulation

No Data available for all ingredients

Mobility in soil

Ingredient                     Mobility

No Data available for all ingredients

Other adverse effects

No evidence of ozone depleting properties were found in the current literature.

SECTION 13 Disposal considerations

Waste treatment methods

Avoid release to the environment.

It may be necessary to collect all wash water for treatment before disposal.

In all cases disposal to sewer may be subject to local laws and regulations and these should be considered first.

Where in doubt contact the responsible authority.

Product / Packaging

Recycle wherever possible or consult manufacturer for recycling options.

disposal

Consult State Land Waste Management Authority for disposal.

Bury residue in an authorised landfill.

Recycle containers if possible, or dispose of in an authorised landfill.

SECTION 14 Transport information

Labels Required

Marine Pollutant  NO

Land transport (DOT): NOT REGULATED FOR TRANSPORT OF DANGEROUS GOODS

Air transport (ICAO-IATA / DGR): NOT REGULATED FOR TRANSPORT OF DANGEROUS GOODS

Sea transport (IMDG-Code / GGVSee): NOT REGULATED FOR TRANSPORT OF DANGEROUS GOODS

14.7. Maritime transport in bulk according to IMO instruments

14.7.1. Transport in bulk according to Annex II of MARPOL and the IBC code

Not Applicable

14.7.2. Transport in bulk in accordance with MARPOL Annex V and the IMSBC Code

Product name              Group

Cerium Oxide (1306-38-3)      Not Available

Gadolinium Oxide (12064-

Not Available

62-9)

14.7.3. Transport in bulk in accordance with the IGC Code

Product name               Ship Type

Cerium Oxide (1306-38-3)      Not Available

Gadolinium Oxide (12064-

Not Available

62-9)

SECTION 15 Regulatory information

Safety, health and environmental regulations / legislation specific for the substance or mixture

Continued...

Page 12

Part Number: Not Available                                           Page 12 of 13                                                       Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

Additional Regulatory Information

Not Applicable

Federal Regulations

Superfund Amendments and Reauthorization Act of 1986 (SARA)

Section 311/312 hazard categories

Flammable (Gases, Aerosols, Liquids, or Solids)                                                                     No

Gas under pressure                                                                                         No

Explosive                                                                                                No

Self-heating                                                                                              No

Pyrophoric (Liquid or Solid)                                                                                    No

Pyrophoric Gas                                                                                            No

Corrosive to metal                                                                                          No

Oxidizer (Liquid, Solid or Gas)                                                                                  No

Organic Peroxide                                                                                           No

Self-reactive                                                                                              No

In contact with water emits flammable gas                                                                          No

Combustible Dust                                                                                          No

Carcinogenicity                                                                                            No

Acute toxicity (any route of exposure)                                                                                          Yes

Reproductive toxicity                                                                                        No

Skin Corrosion or Irritation                                                                                                   Yes

Respiratory or Skin Sensitization                                                                                No

Serious eye damage or eye irritation                                                                                           Yes

Specific target organ toxicity (single or repeated exposure)                                                              No

Aspiration Hazard                                                                                          No

Germ cell mutagenicity                                                                                       No

Simple Asphyxiant                                                                                          No

Hazards Not Otherwise Classified                                                                               No

US. EPA CERCLA Hazardous Substances and Reportable Quantities (40 CFR 302.4)

None Reported

US. EPCRA Section 313 Toxic Release Inventory (TRI) (40 CFR 372)

None Reported

Additional Federal Regulatory Information

Not Applicable

State Regulations

US. California Proposition 65

None Reported

Additional State Regulatory Information

Not Applicable

National Inventory Status

National Inventory            Status

Australia - AIIC / Australia

Yes

Non-Industrial Use

Canada - DSL               Yes

Canada - NDSL            No (Cerium Oxide (1306-38-3); Gadolinium Oxide (12064-62-9))

China - IECSC               Yes

Europe - EINEC / ELINCS /

Yes

NLP

Japan - ENCS               Yes

Korea - KECI                Yes

Continued...

Page 13

Part Number: Not Available                                           Page 13 of 13                                                       Issue Date: 06/12/2025

Version No: 1.2                                                                                                                                Revision Date: 06/12/2025                                                   Gadolinium Doped Ceria

Print Date: 06/12/2025

National Inventory            Status

New Zealand - NZIoC         Yes

Philippines - PICCS           Yes

USA - TSCA                         All chemical substances in this product have been designated as TSCA Inventory ‘Active'

Taiwan - TCSI                Yes

Mexico - INSQ             No (Gadolinium Oxide (12064-62-9))

Vietnam - NCI                Yes

Russia - FBEPH              Yes

Yes = All CAS declared ingredients are on the inventory

Legend:                 No = One or more of the CAS listed ingredients are not on the inventory. These ingredients may be exempt or will require

registration.

SECTION 16 Other information

Revision Date    06/12/2025

Initial Date    06/12/2025

Other information

The SDS is a Hazard Communication tool and should be used to assist in the Risk Assessment. Many factors determine whether the reported Hazards are Risks

in the workplace or other settings. Risks may be determined by reference to Exposures Scenarios. Scale of use, frequency of use and current or available

engineering controls must be considered.

Definitions and abbreviations

PC

end of SDS