Gadolinium fluoride anhydrous SDS

Publisher: Alfa-Aesar

Source: https://resds.com/document/gadolinium-fluoride-anhydrous-sds-alfa-en-15b2646c69

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet                                                                                                     Page 1/5

per OSHA HazCom 2012                                                                                                                            Printing date 11/24/2015

Reviewed on 04/27/2003

1 Identification

Product identifier

Product name: Gadolinium fluoride anhydrous

Stock number: 11292

CAS Number:

13765-26-9

EC number:

237-369-4

Relevant identified uses of the substance or mixture and uses advised against.

Identified use: SU24  Scientific research and development

Details of the supplier of the safety data sheet

Manufacturer/Supplier:

Alfa Aesar

Thermo Fisher Scientific Chemicals, Inc.

30 Bond Street

Ward Hill, MA 01835-8099

Tel: 800-343-0660

Fax: 800-322-4757

Email: tech@alfa.com

www.alfa.com

Information Department: Health, Safety and Environmental Department

Emergency telephone number:

During normal business hours (Monday-Friday, 8am-7pm EST), call (800) 343-0660. After normal business hours, call Carechem 24 at (866) 928-0789.

2 Hazard(s) identification

Classification of the substance or mixture in accordance with 29 CFR 1910 (OSHA HCS)

GHS06 Skull and crossbones

Acute Tox. 3 H301 Toxic if swallowed.

Acute Tox. 3 H311 Toxic in contact with skin.

Acute Tox. 3 H331 Toxic if inhaled.

Hazards not otherwise classified No information known.

Label elements

GHS label elements The product is classified and labeled in accordance with 29 CFR 1910 (OSHA HCS)

Hazard pictograms

GHS06

Signal word Danger

Hazard statements

H301+H311+H331 Toxic if swallowed, in contact with skin or if inhaled.

Precautionary statements

P261      Avoid breathing dust/fume/gas/mist/vapours/spray.

P280     Wear protective gloves/protective clothing/eye protection/face protection.

P301+P310 IF SWALLOWED: Immediately call a POISON CENTER/ doctor/…

P361      Take off immediately all contaminated clothing.

P405       Store locked up.

P501      Dispose of contents/container in accordance with local/regional/national/international regulations.

WHMIS classification

D1A - Very toxic material causing immediate and serious toxic effects

Classification system

HMIS ratings (scale 0-4)

(Hazardous Materials Identification System)

HEALTH     2  Health (acute effects) = 2

FIRE        0  Flammability = 0

REACTIVITY 1  Physical Hazard = 1

Other hazards

Results of PBT and vPvB assessment

PBT: Not applicable.

vPvB: Not applicable.

3 Composition/information on ingredients

Chemical characterization: Substances

CAS# Description:

13765-26-9 Gadolinium fluoride anhydrous

Identification number(s):

EC number: 237-369-4

4 First-aid measures

Description of first aid measures

General information

Immediately remove any clothing soiled by the product.

Remove breathing apparatus only after contaminated clothing has been completely removed.

In case of irregular breathing or respiratory arrest provide artificial respiration.

After inhalation

In case of unconsciousness place patient stably in side position for transportation.

Supply fresh air. If required, provide artificial respiration. Keep patient warm.

(Contd. on page 2)

USA

38.0.2

Page 2

Safety Data Sheet                                                                                                     Page 2/5

per OSHA HazCom 2012                                                                                                                            Printing date 11/24/2015

Reviewed on 04/27/2003

Product name: Gadolinium fluoride anhydrous

(Contd. of page 1)

Seek immediate medical advice.

After skin contact

Immediately wash with water and soap and rinse thoroughly.

Seek immediate medical advice.

After eye contact Rinse opened eye for several minutes under running water. Then consult a doctor.

After swallowing

Drink lots of water.

Induce vomiting if patient is conscious.

Do not induce vomiting; immediately call for medical help.

Information for doctor

Most important symptoms and effects, both acute and delayed No further relevant information available.

Indication of any immediate medical attention and special treatment needed No further relevant information available.

5 Fire-fighting measures

Extinguishing media

Suitable extinguishing agents Product is not flammable. Use fire-fighting measures that suit the surrounding fire.

Special hazards arising from the substance or mixture

If this product is involved in a fire, the following can be released:

Hydrogen chloride (HCl)

Metal oxide fume

Advice for firefighters

Protective equipment:

Wear self-contained respirator.

Wear fully protective impervious suit.

6 Accidental release measures

Personal precautions, protective equipment and emergency procedures

Mount respiratory protective device.

Wear protective equipment. Keep unprotected persons away.

Ensure adequate ventilation

Environmental precautions:

Do not allow to enter sewers/ surface or ground water.

Do not allow material to be released to the environment without proper governmental permits.

Methods and material for containment and cleaning up:

Dispose of contaminated material as waste according to section 13.

Ensure adequate ventilation.

Prevention of secondary hazards: No special measures required.

Reference to other sections

See Section 7 for information on safe handling

See Section 8 for information on personal protection equipment.

See Section 13 for disposal information.

7 Handling and storage

Handling

Precautions for safe handling

Thoroughly remove all dust particles.

Keep container tightly sealed.

Store in cool, dry place in tightly closed containers.

Ensure good ventilation at the workplace.

Open and handle container with care.

Information about protection against explosions and fires: No information known.

Conditions for safe storage, including any incompatibilities

Storage

Requirements to be met by storerooms and receptacles: No special requirements.

Information about storage in one common storage facility:

Store away from oxidizing agents.

Do not store together with acids.

Further information about storage conditions:

Keep container tightly sealed.

Store in cool, dry conditions in well sealed containers.

Specific end use(s) No further relevant information available.

8 Exposure controls/personal protection

Additional information about design of technical systems:

Properly operating chemical fume hood designed for hazardous chemicals and having an average face velocity of at least 100 feet per minute.

Control parameters

Components with limit values that require monitoring at the workplace:

Fluorides (as F)

mg/m3

ACGIH TLV        2.5

Austria MAK       2.5

Belgium TWA       2.5

Finland TWA       2.5

France TWA       2.5

Germany MAK       2.5

Hungary TWA       1; 2-STEL

Netherlands MAC-K   3.5

Norway TWA       0.6

Poland TWA        1; 3-STEL

Sweden NGV      2

Switzerland MAK-W   1.5; 3-KZG-W

United Kingdom TWA  2.5

Russia TWA      2

Denmark TWA       2.5

USA PEL         2.5

Additional information:

The exposure limits that were valid when the MSDS was created were used.

No data

(Contd. on page 3)

USA

38.0.2

Page 3

Safety Data Sheet                                                                                                     Page 3/5

per OSHA HazCom 2012                                                                                                                            Printing date 11/24/2015

Reviewed on 04/27/2003

Product name: Gadolinium fluoride anhydrous

(Contd. of page 2)

Exposure controls

Personal protective equipment

General protective and hygienic measures

The usual precautionary measures for handling chemicals should be followed.

Keep away from foodstuffs, beverages and feed.

Remove all soiled and contaminated clothing immediately.

Wash hands before breaks and at the end of work.

Store protective clothing separately.

Avoid contact with the eyes and skin.

Maintain an ergonomically appropriate working environment.

Breathing equipment:

In case of brief exposure or low pollution use respiratory filter device. In case of intensive or longer exposure use respiratory protective device that is independent of

circulating air.

Use self-contained respiratory protective device in emergency situations.

Recommended filter device for short term use: Filter P1

Protection of hands:

Impervious gloves

Protective gloves.

Check protective gloves prior to each use for their proper condition.

The selection of suitable gloves not only depends on the material, but also on quality. Quality will vary from manufacturer to manufacturer.

Penetration time of glove material (in minutes) Not determined

Eye protection: Safety glasses

Body protection: Protective work clothing.

9 Physical and chemical properties

Information on basic physical and chemical properties

General Information

Appearance:

Form:                                  Crystalline

Color:                            White

Odor:                               Odorless

Odor threshold:                     Not determined.

pH-value:                          Not applicable.

Change in condition

Melting point/Melting range:       1231 °C (2248 °F)

Boiling point/Boiling range:        Not determined

Sublimation temperature / start:    Not determined

Flash point:                        Not applicable

Flammability (solid, gaseous)         Product is not flammable.

Ignition temperature:                Not determined

Decomposition temperature:         Not determined

Auto igniting:                       Not determined.

Danger of explosion:                 Product does not present an explosion hazard.

Explosion limits:

Lower:                           Not determined

Upper:                           Not determined

Vapor pressure:                    Not applicable.

Density at 20 °C (68 °F):              7.047 g/cm³ (58.807 lbs/gal)

Relative density                     Not determined.

Vapor density                      Not applicable.

Evaporation rate                    Not applicable.

Solubility in / Miscibility with

Water:                            Not determined

Partition coefficient (n-octanol/water): Not determined.

Viscosity:

dynamic:                         Not applicable.

kinematic:                        Not applicable.

Other information               No further relevant information available.

10 Stability and reactivity

Reactivity Contact with acids liberates very toxic gas.

Chemical stability Stable under recommended storage conditions.

Thermal decomposition / conditions to be avoided: Decomposition will not occur if used and stored according to specifications.

Possibility of hazardous reactions Contact with acids liberates very toxic gas.

Conditions to avoid No further relevant information available.

Incompatible materials: Acids

Hazardous decomposition products:

Hydrogen fluoride

Metal oxide fume

11 Toxicological information

Information on toxicological effects

Acute toxicity:

Toxic in contact with skin.

Toxic if inhaled.

Toxic if swallowed.

Danger through skin absorption.

LD/LC50 values that are relevant for classification: No data

Skin irritation or corrosion: Irritant to skin and mucous membranes.

Eye irritation or corrosion: Irritating effect.

Sensitization: No sensitizing effects known.

Germ cell mutagenicity: No effects known.

Carcinogenicity: No classification data on carcinogenic properties of this material is available from the EPA, IARC, NTP, OSHA or ACGIH.

Reproductive toxicity: No effects known.

Specific target organ system toxicity - repeated exposure: No effects known.

Specific target organ system toxicity - single exposure: No effects known.

Aspiration hazard: No effects known.

(Contd. on page 4)

USA

38.0.2

Page 4

Safety Data Sheet                                                                                                     Page 4/5

per OSHA HazCom 2012                                                                                                                            Printing date 11/24/2015

Reviewed on 04/27/2003

Product name: Gadolinium fluoride anhydrous

(Contd. of page 3)

Subacute to chronic toxicity:

Fluorides may cause salivation, nausea, vomiting, diarrhea and abdominal pain, followed by weakness, tremors, shallow respiration, convulsions and coma. May

cause brain and kidney damage. Chronic fluoride poisoning can cause severe bone changes, loss of weight, anorexia, anemia and dental defects.

Lanthanons can cause delayed blood clotting leading to hemorrhages. Exposure may also lead to sensitivity to heat, itching, increased awareness of odor and taste,

and liver damage.

Subacute to chronic toxicity: No effects known.

Additional toxicological information:

To the best of our knowledge the acute and chronic toxicity of this substance is not fully known.

The substance is not subject to classification.

12 Ecological information

Toxicity

Aquatic toxicity: No further relevant information available.

Persistence and degradability No further relevant information available.

Bioaccumulative potential No further relevant information available.

Mobility in soil No further relevant information available.

Additional ecological information:

General notes:

Do not allow material to be released to the environment without proper governmental permits.

Do not allow undiluted product or large quantities to reach ground water, water course or sewage system.

Avoid transfer into the environment.

Results of PBT and vPvB assessment

PBT: Not applicable.

vPvB: Not applicable.

Other adverse effects No further relevant information available.

13 Disposal considerations

Waste treatment methods

Recommendation Consult state, local or national regulations to ensure proper disposal.

Waste disposal key number according to the European Waste Catalogue:

Contaminated salts:

06 03 05  Solid salts containing chlorides, fluorides and other

halogenides

Uncleaned packagings:

Recommendation: Disposal must be made according to official regulations.

14 Transport information

UN-Number

DOT, IMDG, IATA                                             UN3288

UN proper shipping name

DOT                                                                      Toxic solid, inorganic, n.o.s. (Gadolinium fluoride anhydrous)

IMDG, IATA                                                  TOXIC SOLID, INORGANIC, N.O.S. (Gadolinium fluoride anhydrous)

Transport hazard class(es)

DOT

Class                                                                       6.1 Toxic substances.

Label                                                                        6.1

Class                                                                       6.1 (T5) Toxic substances

Label                                                                        6.1

IMDG, IATA

Class                                                                       6.1 Toxic substances.

Label                                                                        6.1

Packing group

DOT, IMDG, IATA                                                                                                   III

Environmental hazards:                                               Not applicable.

Special precautions for user                                            Warning: Toxic substances

Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code Not applicable.

Transport/Additional information:

DOT

Marine Pollutant (DOT):                                       No

UN "Model Regulation":                                           UN3288, Toxic solid, inorganic, n.o.s. (Gadolinium fluoride anhydrous), 6.1, III

15 Regulatory information

Safety, health and environmental regulations/legislation specific for the substance or mixture

GHS label elements The product is classified and labeled in accordance with 29 CFR 1910 (OSHA HCS)

Hazard pictograms

GHS06

Signal word Danger

Hazard statements

H301+H311+H331 Toxic if swallowed, in contact with skin or if inhaled.

Precautionary statements

P261      Avoid breathing dust/fume/gas/mist/vapours/spray.

P280     Wear protective gloves/protective clothing/eye protection/face protection.

(Contd. on page 5)

USA

38.0.2

Page 5

Safety Data Sheet                                                                                                     Page 5/5

per OSHA HazCom 2012                                                                                                                            Printing date 11/24/2015

Reviewed on 04/27/2003

Product name: Gadolinium fluoride anhydrous

(Contd. of page 4)

P301+P310 IF SWALLOWED: Immediately call a POISON CENTER/ doctor/…

P361      Take off immediately all contaminated clothing.

P405       Store locked up.

P501      Dispose of contents/container in accordance with local/regional/national/international regulations.

National regulations

All components of this product are listed in the U.S. Environmental Protection Agency Toxic Substances Control Act Chemical substance Inventory.

All components of this product are listed on the Canadian Non-Domestic Substances List (NDSL).

SARA Section 313 (specific toxic chemical listings) Substance is not listed.

California Proposition 65

Prop 65 - Chemicals known to cause cancer Substance is not listed.

Prop 65 - Developmental toxicity Substance is not listed.

Prop 65 - Developmental toxicity, female Substance is not listed.

Prop 65 - Developmental toxicity, male Substance is not listed.

Information about limitation of use: For use only by technically qualified individuals.

Other regulations, limitations and prohibitive regulations

Substance of Very High Concern (SVHC) according to the REACH Regulations (EC) No. 1907/2006. Substance is not listed.

The conditions of restrictions according to Article 67 and Annex XVII of the Regulation (EC) No 1907/2006 (REACH) for the manufacturing, placing on the

market and use must be observed.

Substance is not listed.

Annex XIV of the REACH Regulations (requiring Authorisation for use) Substance is not listed.

Chemical safety assessment: A Chemical Safety Assessment has not been carried out.

16 Other information

This information is based on our present knowledge. However, this shall not constitute a guarantee for any specific product features and shall not establish a legally

valid contractual relationship.

Employers should use this information only as a supplement to other information gathered by them, and should make independent judgement of suitability of this

information to ensure proper use and protect the health and safety of employees. This information is furnished without warranty, and any use of the product not in

conformance with this Material Safety Data Sheet, or in combination with any other product or process, is the responsibility of the user.

Department issuing SDS: Global Marketing Department

Date of preparation / last revision 11/24/2015 / 1

Abbreviations and acronyms:

RID: Règlement international concernant le transport des marchandises dangereuses par chemin de fer (Regulations Concerning the International Transport of Dangerous Goods by Rail)

IATA-DGR: Dangerous Goods Regulations by the "International Air Transport Association" (IATA)

ICAO: International Civil Aviation Organization

ICAO-TI: Technical Instructions by the "International Civil Aviation Organization" (ICAO)

ADR: Accord européen sur le transport des marchandises dangereuses par Route (European Agreement concerning the International Carriage of Dangerous Goods by Road)

IMDG: International Maritime Code for Dangerous Goods

DOT: US Department of Transportation

IATA: International Air Transport Association

EINECS: European Inventory of Existing Commercial Chemical Substances

CAS: Chemical Abstracts Service (division of the American Chemical Society)

HMIS: Hazardous Materials Identification System (USA)

WHMIS: Workplace Hazardous Materials Information System (Canada)

LC50: Lethal concentration, 50 percent

LD50: Lethal dose, 50 percent

vPvB: very Persistent and very Bioaccumulative

ACGIH: American Conference of Governmental Industrial Hygienists (USA)

OSHA: Occupational Safety and Health Administration (USA)

NTP: National Toxicology Program (USA)

IARC: International Agency for Research on Cancer

EPA: Environmental Protection Agency (USA)

USA

38.0.2