Gadolinium foil SDS

Publisher: Alfa-Aesar

Source: https://resds.com/document/gadolinium-foil-sds-alfa-en-ca5206d41e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet                                                                                                     Page 1/5

acc. to OSHA HCS                                                                                                                                  Printing date 04/11/2017

Revision date 04/10/2017

Version 1

1 Identification

Product identifier

Product name: Gadolinium foil

Stock number: 00115

CAS Number:

7440-54-2

EC number:

231-162-2

Details of the supplier of the safety data sheet

Manufacturer/Supplier:

Alfa Aesar

Thermo Fisher Scientific Chemicals, Inc.

30 Bond Street

Ward Hill, MA 01835-8099

Tel: 800-343-0660

Fax: 800-322-4757

Email: tech@alfa.com

www.alfa.com

Information Department: Health, Safety and Environmental Department

Emergency telephone number:

During normal business hours (Monday-Friday, 8am-7pm EST), call (800) 343-0660. After normal business hours, call Carechem 24 at (866) 928-0789.

2 Hazard(s) identification

Classification of the substance or mixture in accordance with 29 CFR 1910 (OSHA HCS)

GHS02 Flame

Flam. Sol. 1   H228 Flammable solid.

Water-react. 1 H260 In contact with water releases flammable gases, which may ignite spontaneously.

Hazards not otherwise classified No information known.

Label elements

GHS label elements The product is classified and labeled in accordance with 29 CFR 1910 (OSHA HCS)

Hazard pictograms

GHS02

Signal word Danger

Hazard statements

H228 Flammable solid.

H260 In contact with water releases flammable gases, which may ignite spontaneously.

Precautionary statements

P223     Do not allow contact with water.

P210     Keep away from heat/sparks/open flames/hot surfaces. No smoking.

P231+P232 Handle under inert gas. Protect from moisture.

P280     Wear protective gloves/protective clothing/eye protection/face protection.

P240      Ground/bond container and receiving equipment.

P501      Dispose of contents/container in accordance with local/regional/national/international regulations.

WHMIS classification

B4 - Flammable solid

B6 - Reactive flammable material

Classification system

HMIS ratings (scale 0-4)

(Hazardous Materials Identification System)

HEALTH     1  Health (acute effects) = 1

FIRE        3  Flammability = 3

REACTIVITY 2  Physical Hazard = 2

Other hazards

Results of PBT and vPvB assessment

PBT: Not applicable.

vPvB: Not applicable.

3 Composition/information on ingredients

Chemical characterization: Substances

CAS# Description:

7440-54-2 Gadolinium

Concentration: ื100%

Identification number(s):

EC number: 231-162-2

4 First-aid measures

Description of first aid measures

After inhalation

Supply fresh air. If required, provide artificial respiration. Keep patient warm.

Seek immediate medical advice.

After skin contact

Immediately wash with water and soap and rinse thoroughly.

Seek immediate medical advice.

(Contd. on page 2)

USA

44.0

Page 2

Safety Data Sheet                                                                                                     Page 2/5

acc. to OSHA HCS                                                                                                                                  Printing date 04/11/2017

Revision date 04/10/2017

Version 1

Product name: Gadolinium foil

(Contd. of page 1)

After eye contact Rinse opened eye for several minutes under running water. Then consult a doctor.

After swallowing Seek medical treatment.

Information for doctor

Most important symptoms and effects, both acute and delayed No further relevant information available.

Indication of any immediate medical attention and special treatment needed No further relevant information available.

5 Fire-fighting measures

Extinguishing media

Suitable extinguishing agents Special powder for metal fires. Do not use water.

For safety reasons unsuitable extinguishing agents Water

Special hazards arising from the substance or mixture

If this product is involved in a fire, the following can be released:

Gadolinium oxide

Advice for firefighters

Protective equipment:

Wear self-contained respirator.

Wear fully protective impervious suit.

6 Accidental release measures

Personal precautions, protective equipment and emergency procedures

Wear protective equipment. Keep unprotected persons away.

Ensure adequate ventilation

Keep away from ignition sources

Environmental precautions: Do not allow material to be released to the environment without proper governmental permits.

Methods and material for containment and cleaning up:

Keep away from ignition sources.

Ensure adequate ventilation.

Do not flush with water or aqueous cleansing agents

Prevention of secondary hazards: Keep away from ignition sources.

Reference to other sections

See Section 7 for information on safe handling

See Section 8 for information on personal protection equipment.

See Section 13 for disposal information.

Protective Action Criteria for Chemicals

PAC-1: 30 mg/m3

PAC-2: 330 mg/m3

PAC-3: 2,000 mg/m3

7 Handling and storage

Handling

Precautions for safe handling

Handle under dry protective gas.

Keep container tightly sealed.

Store in cool, dry place in tightly closed containers.

Ensure good ventilation at the workplace.

Information about protection against explosions and fires: Protect against electrostatic charges.

Conditions for safe storage, including any incompatibilities

Storage

Requirements to be met by storerooms and receptacles: Store in a cool location.

Information about storage in one common storage facility:

Do not store together with oxidizing and acidic materials.

Store away from air.

Store away from water/moisture.

Do not store together with acids.

Store away from strong bases.

Further information about storage conditions:

Store under dry inert gas.

This product is moisture sensitive.

This product is air sensitive.

Keep container tightly sealed.

Store in cool, dry conditions in well sealed containers.

Protect from humidity and water.

Specific end use(s) No further relevant information available.

8 Exposure controls/personal protection

Additional information about design of technical systems:

Properly operating chemical fume hood designed for hazardous chemicals and having an average face velocity of at least 100 feet per minute.

Control parameters

Components with limit values that require monitoring at the workplace:

The product does not contain any relevant quantities of materials with critical values that have to be monitored at the workplace.

Additional information: No data

Exposure controls

Personal protective equipment

General protective and hygienic measures

The usual precautionary measures for handling chemicals should be followed.

Keep away from foodstuffs, beverages and feed.

Remove all soiled and contaminated clothing immediately.

Wash hands before breaks and at the end of work.

Maintain an ergonomically appropriate working environment.

Breathing equipment: Use suitable respirator when high concentrations are present.

Protection of hands:

Impervious gloves

Check protective gloves prior to each use for their proper condition.

The selection of suitable gloves not only depends on the material, but also on quality. Quality will vary from manufacturer to manufacturer.

Penetration time of glove material (in minutes) Not determined

Eye protection:

Full face protection

Safety glasses with side shields / NIOSH (US) or EN 166(EU)

(Contd. on page 3)

USA

44.0

Page 3

Safety Data Sheet                                                                                                     Page 3/5

acc. to OSHA HCS                                                                                                                                  Printing date 04/11/2017

Revision date 04/10/2017

Version 1

Product name: Gadolinium foil

(Contd. of page 2)

Body protection: Protective work clothing.

9 Physical and chemical properties

Information on basic physical and chemical properties

General Information

Appearance:

Form:                                 Solid in various forms

Odor:                              Not determined

Odor threshold:                     Not determined.

pH-value:                          Not applicable.

Change in condition

Melting point/Melting range:       1312 °C (2394 °F)

Boiling point/Boiling range:        3273 °C (5923 °F)

Sublimation temperature / start:    Not determined

Flammability (solid, gaseous)         Highly flammable.

Contact with water liberates extremely flammable gases.

Ignition temperature:                Not determined

Decomposition temperature:         Not determined

Auto igniting:                       Not determined.

Danger of explosion:                Not determined.

Explosion limits:

Lower:                           Not determined

Upper:                           Not determined

Vapor pressure at 20 °C (68 °F):      0 hPa

Density at 20 °C (68 °F):               7.9 g/cm³ (65.926 lbs/gal)

Relative density                     Not determined.

Vapor density                      Not applicable.

Evaporation rate                    Not applicable.

Solubility in / Miscibility with

Water:                             Contact with water releases flammable gases

Partition coefficient (n-octanol/water): Not determined.

Viscosity:

dynamic:                         Not applicable.

kinematic:                        Not applicable.

Other information               No further relevant information available.

10 Stability and reactivity

Reactivity In contact with water releases flammable gases, which may ignite spontaneously.

Chemical stability Stable under recommended storage conditions.

Thermal decomposition / conditions to be avoided: Decomposition will not occur if used and stored according to specifications.

Possibility of hazardous reactions

Spontaneously flammable in air.

Contact with water releases flammable gases

Conditions to avoid No further relevant information available.

Incompatible materials:

Acids

Air

Oxidizing agents

Bases

Water/moisture

Hazardous decomposition products: Gadolinium oxide

11 Toxicological information

Information on toxicological effects

Acute toxicity: No effects known.

LD/LC50 values that are relevant for classification: No data

Skin irritation or corrosion: May cause irritation

Eye irritation or corrosion: May cause irritation

Sensitization: No sensitizing effects known.

Germ cell mutagenicity: No effects known.

Carcinogenicity:

The Registry of Toxic Effects of Chemical Substances (RTECS) contains tumorigenic and/or carcinogenic and/or neoplastic data for this substance.

No classification data on carcinogenic properties of this material is available from the EPA, IARC, NTP, OSHA or ACGIH.

Reproductive toxicity: No effects known.

Specific target organ system toxicity - repeated exposure: No effects known.

Specific target organ system toxicity - single exposure: No effects known.

Aspiration hazard: No effects known.

Subacute to chronic toxicity: No effects known.

Additional toxicological information: To the best of our knowledge the acute and chronic toxicity of this substance is not fully known.

12 Ecological information

Toxicity

Aquatic toxicity: No further relevant information available.

Persistence and degradability No further relevant information available.

Bioaccumulative potential No further relevant information available.

Mobility in soil No further relevant information available.

Additional ecological information:

General notes:

Do not allow material to be released to the environment without proper governmental permits.

Do not allow undiluted product or large quantities to reach ground water, water course or sewage system.

Avoid transfer into the environment.

Results of PBT and vPvB assessment

PBT: Not applicable.

vPvB: Not applicable.

Other adverse effects No further relevant information available.

USA

(Contd. on page 4)

44.0

Page 4

Safety Data Sheet                                                                                                     Page 4/5

acc. to OSHA HCS                                                                                                                                  Printing date 04/11/2017

Revision date 04/10/2017

Version 1

Product name: Gadolinium foil

(Contd. of page 3)

13 Disposal considerations

Waste treatment methods

Recommendation Consult state, local or national regulations to ensure proper disposal.

Uncleaned packagings:

Recommendation: Disposal must be made according to official regulations.

14 Transport information

UN-Number

DOT, IMDG, IATA                                             UN3178

UN proper shipping name

DOT                                                             Flammable solid, inorganic, n.o.s. (Gadolinium foil)

ADR                                                            3178 Flammable solid, inorganic, n.o.s. (Gadolinium foil)

IMDG, IATA                                             FLAMMABLE SOLID, INORGANIC, N.O.S. (Gadolinium foil)

Transport hazard class(es)

DOT

Class                                                                       4.1 Flammable solids, self-reactive substances and solid desensitised explosives

Label                                                                        4.1

ADR

Class                                                                       4.1 (F3) Flammable solids, self-reactive substances and solid desensitised

explosives

Label                                                                        4.1

IMDG, IATA

Class                                                                       4.1 Flammable solids, self-reactive substances and solid desensitised explosives

Label                                                                        4.1

Packing group

DOT, ADR, IMDG, IATA                                                                                          II

Environmental hazards:                                               Not applicable.

Special precautions for user                                            Warning: Flammable solids, self-reactive substances and solid desensitised

explosives

EMS Number:                                                        F-A,S-G

Stowage Category                                         B

Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code Not applicable.

Transport/Additional information:

DOT

Quantity limitations                                        On passenger aircraft/rail: 15 kg

On cargo aircraft only: 50 kg

Marine Pollutant (DOT):                                       No

IMDG

Limited quantities (LQ)                                             1 kg

Excepted quantities (EQ)                                             Code: E2

Maximum net quantity per inner packaging: 30 g

Maximum net quantity per outer packaging: 500 g

UN "Model Regulation":                                  UN 3178 FLAMMABLE SOLID, INORGANIC, N.O.S. (GADOLINIUM FOIL), 4.1, II

15 Regulatory information

Safety, health and environmental regulations/legislation specific for the substance or mixture

GHS label elements The product is classified and labeled in accordance with 29 CFR 1910 (OSHA HCS)

Hazard pictograms

GHS02

Signal word Danger

Hazard statements

H228 Flammable solid.

H260 In contact with water releases flammable gases, which may ignite spontaneously.

Precautionary statements

P223     Do not allow contact with water.

P210     Keep away from heat/sparks/open flames/hot surfaces. No smoking.

P231+P232 Handle under inert gas. Protect from moisture.

P280     Wear protective gloves/protective clothing/eye protection/face protection.

P240      Ground/bond container and receiving equipment.

P501      Dispose of contents/container in accordance with local/regional/national/international regulations.

National regulations

All components of this product are listed in the U.S. Environmental Protection Agency Toxic Substances Control Act Chemical substance Inventory.

All components of this product are listed on the Canadian Non-Domestic Substances List (NDSL).

SARA Section 313 (specific toxic chemical listings) Substance is not listed.

California Proposition 65

Prop 65 - Chemicals known to cause cancer Substance is not listed.

Prop 65 - Developmental toxicity Substance is not listed.

(Contd. on page 5)

USA

44.0

Page 5

Safety Data Sheet                                                                                                     Page 5/5

acc. to OSHA HCS                                                                                                                                  Printing date 04/11/2017

Revision date 04/10/2017

Version 1

Product name: Gadolinium foil

(Contd. of page 4)

Prop 65 - Developmental toxicity, female Substance is not listed.

Prop 65 - Developmental toxicity, male Substance is not listed.

Information about limitation of use: For use only by technically qualified individuals.

Other regulations, limitations and prohibitive regulations

Substance of Very High Concern (SVHC) according to the REACH Regulations (EC) No. 1907/2006. Substance is not listed.

The conditions of restrictions according to Article 67 and Annex XVII of the Regulation (EC) No 1907/2006 (REACH) for the manufacturing, placing on the

market and use must be observed.

Substance is not listed.

Annex XIV of the REACH Regulations (requiring Authorisation for use) Substance is not listed.

Chemical safety assessment: A Chemical Safety Assessment has not been carried out.

16 Other information

Employers should use this information only as a supplement to other information gathered by them, and should make independent judgement of suitability of this

information to ensure proper use and protect the health and safety of employees. This information is furnished without warranty, and any use of the product not in

conformance with this Safety Data Sheet, or in combination with any other product or process, is the responsibility of the user.

Department issuing SDS: Global Marketing Department

Date of preparation/Revision: Print date, revision date and version number are in the header of each page.

Abbreviations and acronyms:

RID: Règlement international concernant le transport des marchandises dangereuses par chemin de fer (Regulations Concerning the International Transport of Dangerous Goods by Rail)

ICAO: International Civil Aviation Organisation

ADR: Accord européen sur le transport des marchandises dangereuses par Route (European Agreement concerning the International Carriage of Dangerous Goods by Road)

IMDG: International Maritime Code for Dangerous Goods

DOT: US Department of Transportation

IATA: International Air Transport Association

EINECS: European Inventory of Existing Commercial Chemical Substances

CAS: Chemical Abstracts Service (division of the American Chemical Society)

HMIS: Hazardous Materials Identification System (USA)

WHMIS: Workplace Hazardous Materials Information System (Canada)

LC50: Lethal concentration, 50 percent

LD50: Lethal dose, 50 percent

PBT: Persistent, Bioaccumulative and Toxic

SVHC: Substances of Very High Concern

vPvB: very Persistent and very Bioaccumulative

ACGIH: American Conference of Governmental Industrial Hygienists (USA)

OSHA: Occupational Safety and Health Administration (USA)

NTP: National Toxicology Program (USA)

IARC: International Agency for Research on Cancer

EPA: Environmental Protection Agency (USA)

Flam. Sol. 1: Flammable solids – Category 1

Water-react. 1: Substances and mixtures which in contact with water emit flammable gases – Category 1

USA

44.0