Gadolinium(III) fluoride, anhydrous, REacton® 13655 SDS

Publisher: Thermo Fisher Scientific

Source: https://resds.com/document/gadolinium-iii-fluoride-anhydrous-reacton-sds-thermo-fisher-en-857e207591

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Revision Date 22-Dec-2025                                                               Revision Number 4

This safety data sheet was created pursuant to the requirements of: US OSHA Hazard Communication Standard 2024 (29

CFR 1910.1200)

1. Identification

Product Name                Gadolinium(III) fluoride, anhydrous, REacton®

Cat No. :                  13655

CAS No                           13765-26-9

Synonyms                   No information available

Recommended Use                 Laboratory chemicals.

Uses advised against              Food, drug, pesticide or biocidal product use.

Details of the supplier of the safety data sheet

Company

Thermo Fisher Scientific Chemicals, Inc.

30 Bond Street

Ward Hill, MA 01835-8099

Tel: 800-343-0660

Fax: 800-322-4757

Emergency Telephone Number

For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

2. Hazard(s) identification

Classification

This chemical is considered hazardous according to [US] OSHA (29 CFR 1910.1200, 2024)

Skin Corrosion/Irritation                                    Category 1 B

Serious Eye Damage/Eye Irritation                           Category 1

Label Elements

Signal Word

Danger

Hazard Statements

Causes severe skin burns and eye damage

______________________________________________________________________________________________

Page 1 / 9

Page 2

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

Precautionary Statements

Prevention

Do not breathe dust/fume/gas/mist/vapors/spray

Wash face, hands and any exposed skin thoroughly after handling

Wear protective gloves/protective clothing/eye protection/face protection

Response

Immediately call a POISON CENTER or doctor

Inhalation

IF INHALED: Remove person to fresh air and keep comfortable for breathing

Skin

IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

Take off contaminated clothing and wash before reuse

Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

Ingestion

IF SWALLOWED: Rinse mouth. DO NOT induce vomiting

Storage

Store locked up

Disposal

Dispose of contents/container to an approved waste disposal plant

Hazards not otherwise classified (HNOC)

None identified

Hazards classified under paragraph (d)(1)(ii) of 1910.1200

Hazards resulting from a reaction with other chemicals under normal conditions of use

Contact with acids liberates toxic gas.

3. Composition/information on Ingredients

Component                      CAS No                        Weight %

Gadolinium fluoride (GdF3)                       13765-26-9                       > 99.9

4. First-aid measures

Eye Contact                      Immediate medical attention is required. Rinse immediately with plenty of water, also under

the eyelids, for at least 15 minutes.

Skin Contact                  Wash off immediately with soap and plenty of water while removing all contaminated

clothes and shoes. Immediate medical attention is required.

Inhalation                    Remove from exposure, lie down. Remove to fresh air. If not breathing, give artificial

respiration. Immediate medical attention is required.

Ingestion                    Do NOT induce vomiting. Call a physician immediately.

Most important symptoms and     Causes burns by all exposure routes. Product is a corrosive material. Use of gastric

______________________________________________________________________________________________

Page 2 / 9

Page 3

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

effects                             lavage or emesis is contraindicated. Possible perforation of stomach or esophagus should

be investigated: Ingestion causes severe swelling, severe damage to the delicate tissue

and danger of perforation

Notes to Physician                  Treat symptomatically

5. Fire-fighting measures

Suitable Extinguishing Media      Carbon dioxide (CO 2). Dry chemical. Chemical foam.

Unsuitable Extinguishing Media    No information available

Flash Point                No information available

Method -                  No information available

Autoignition Temperature        No information available

Explosion Limits

Upper                    No data available

Lower                    No data available

Sensitivity to Mechanical Impact No information available

Sensitivity to Static Discharge  No information available

Specific Hazards Arising from the Chemical

Keep product and empty container away from heat and sources of ignition.

Hazardous Combustion Products

Gaseous hydrogen fluoride (HF).

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

NFPA

Health                   Flammability                     Instability                Physical hazards

3                         0                         0                        N/A

6. Accidental release measures

Personal Precautions              Ensure adequate ventilation. Use personal protective equipment as required.

Environmental Precautions        See Section 12 for additional Ecological Information.

Methods for Containment and Clean Avoid dust formation. Sweep up and shovel into suitable containers for disposal. Do not let

Up                                         this chemical enter the environment.

7. Handling and Storage

Handling                     Do not breathe dust. Do not get in eyes, on skin, or on clothing. Do not ingest. If swallowed

then seek immediate medical assistance. Handle product only in closed system or provide

appropriate exhaust ventilation.

Storage.                       Keep in a dry, cool and well-ventilated place. Keep container tightly closed. Keep away

from acids. Corrosives area. Keep containers tightly closed in a dry, cool and well-ventilated

place. Incompatible Materials. Acids. Strong oxidizing agents. Strong reducing agents.

8. Exposure controls / personal protection

Exposure Guidelines

Component            ACGIH TLV           OSHA PEL              NIOSH            Mexico OEL (TWA)

Gadolinium fluoride (GdF3)     TWA: 2.5 mg/m3       (Vacated) TWA: 2.5 mg/m3      IDLH: 250 mg/m3         TWA: 2.5 mg/m3

Legend

______________________________________________________________________________________________

Page 3 / 9

Page 4

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

Engineering Measures             Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations

and safety showers are close to the workstation location.

Personal Protective Equipment

Eye/face Protection           Wear appropriate protective eyeglasses or chemical safety goggles as described by

OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard

EN166.

Skin and body protection      Wear appropriate protective gloves and clothing to prevent skin exposure.

Respiratory Protection          Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard

EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if

exposure limits are exceeded or if irritation or other symptoms are experienced.

Recommended Filter type:       Particulates filter conforming to EN 143.

Hygiene Measures             Handle in accordance with good industrial hygiene and safety practice.

9. Physical and chemical properties

Appearance

Physical State                   Powder Solid

Color                             White

Odor                              Odorless

Odor Threshold               No information available

Property                         Values                       Remarks   •Method

Melting Point/Range            No data available

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Flammability (liquid)               Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Autoignition Temperature        No data available

Decomposition Temperature      No data available

pH                         No information available

Viscosity                         Not applicable                         Solid

Water Solubility                      Insoluble

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Vapor Pressure                No information available

Density  / Specific Gravity        No data available

Bulk Density                  No data available

Vapor Density                     Not applicable                         Solid

Particle characteristics          No data available

Other Information

Molecular Formula               F3 Gd

Molecular Weight                  214.24

Evaporation Rate                  Not applicable  - Solid

______________________________________________________________________________________________

Page 4 / 9

Page 5

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

10. Stability and reactivity

Reactive Hazard                 Yes

Stability                             Stable under normal conditions.

Conditions to Avoid                Avoid dust formation. Incompatible products.

Incompatible Materials               Acids, Strong oxidizing agents, Strong reducing agents

Hazardous Decomposition Products Gaseous hydrogen fluoride (HF)

Hazardous Polymerization        No information available.

Hazardous Reactions            None under normal processing.

11. Toxicological information

Information on expected route of exposure

Inhalation                         Avoid breathing dust or spray mist. May be harmful if inhaled.

Ingestion                     May be harmful if swallowed.

Eyes                              Avoid contact with eyes. Corrosive to the eyes and may cause severe damage including

blindness.

Skin                               Avoid contact with skin. Causes burns. Skin Corrosion/Irritation.

Toxicology data for the components

Toxicologically Synergistic       No information available

Products

(b) skin corrosion/irritation;        Category 1 B

(c) serious eye damage/irritation;   No data available

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;

The table below indicates whether each agency has listed any ingredient as a carcinogen

Component       CAS No          IARC          NTP          ACGIH        OSHA           Mexico

Gadolinium fluoride      13765-26-9         Not listed          Not listed          Not listed          Not listed          Not listed

(GdF3)

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

______________________________________________________________________________________________

Page 5 / 9

Page 6

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

(i) STOT-repeated exposure;      No data available

Target Organs              No information available.

(j) aspiration hazard;               Not applicable

Solid

Other Adverse Effects            The toxicological properties have not been fully investigated.

Symptoms  / effects,both acute and Product is a corrosive material. Use of gastric lavage or emesis is contraindicated.

delayed                             Possible perforation of stomach or esophagus should be investigated. Ingestion causes

severe swelling, severe damage to the delicate tissue and danger of perforation.

Other Adverse Effects            The toxicological properties have not been fully investigated.

Endocrine Disrupting Properties    This product does not contain any known or suspected endocrine disruptors.

12. Ecological information

Ecotoxicity

Do not empty into drains.

Persistence and Degradability       Insoluble in water

Bioaccumulation/ Accumulation   No information available.

Mobility                                   Is not likely mobile in the environment due its low water solubility.

13. Disposal considerations

Waste Disposal Methods           Chemical waste generators must determine whether a discarded chemical is classified as a

hazardous waste. Chemical waste generators must also consult local, regional, and

national hazardous waste regulations to ensure complete and accurate classification.

14. Transport information

DOT

UN-No                     UN3260

Hazard Class                 8

Packing Group                               III

TDG

UN-No                     UN3260

Hazard Class                 8

Packing Group                               III

IATA

UN-No                     UN3260

Proper Shipping Name       CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S.*

Hazard Class                 8

Packing Group                               III

IMDG/IMO

UN-No                     UN3260

Proper Shipping Name          Corrosive solid, acidic, inorganic, n.o.s.

Hazard Class                 8

Packing Group                               III

15. Regulatory Information

United States of America Inventory

Component           CAS No          TSCA        TSCA Inventory notification -    TSCA - EPA Regulatory

Active-Inactive                   Flags

______________________________________________________________________________________________

Page 6 / 9

Page 7

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

Gadolinium fluoride (GdF3)        13765-26-9          X                   ACTIVE                                       -

Legend:

TSCA US EPA (TSCA) - Toxic Substances Control Act, (40 CFR Part 710)

X - Listed

'-' - Not Listed

TSCA - Per 40 CFR 751, Regulation of Certain Chemical     Not applicable

Substances & Mixtures, Under TSCA Section 6(h) (PBT)

TSCA 12(b) - Notices of Export                             Not applicable

International Inventories

Canada (DSL/NDSL), Europe (EINECS/ELINCS/NLP), Philippines (PICCS), Japan (ENCS), Japan (ISHL), Australia (AICS), China (IECSC), Korea

(KECL).

Component          CAS No     DSL   NDSL   EINECS  PICCS  ENCS    ISHL    AICS   IECSC   KECL

Gadolinium fluoride (GdF3)      13765-26-9          -      X     237-369-4       -      X      X            -             -     KE-17398

KECL - NIER number or KE number (http://ncis.nier.go.kr/en/main.do)

U.S. Federal Regulations

SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product does not contain any

chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372

SARA 311/312 Hazard Categories

Should this product meet EPCRA 311/312 Tier reporting criteria at 40 CFR 370, refer to Section 2 of this SDS for appropriate

classifications.

CWA (Clean Water Act)            Not applicable

Clean Air Act                     Not applicable

OSHA - Occupational Safety and     Not applicable

Health Administration

CERCLA

This material, as supplied, does not contain any substances regulated as hazardous substances under the Comprehensive

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund Amendments and

Reauthorization Act (SARA) (40 CFR 355). There may be specific reporting requirements at the local, regional, or state level

pertaining to releases of this material.

California Proposition 65            This product does not contain any Proposition 65 chemicals.

U.S. State Right-to-Know

Regulations

Component         Massachusetts      New Jersey        Pennsylvania              Illinois         Rhode Island

Gadolinium fluoride                    -               X                              -                              -               X

(GdF3)

U.S. Department of Transportation

Reportable Quantity (RQ):        N

DOT Marine Pollutant           N

DOT Severe Marine Pollutant      N

______________________________________________________________________________________________

Page 7 / 9

Page 8

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

U.S. Department of Homeland       This product does not contain any DHS chemicals.

Security

Other International Regulations

Mexico - Grade                No information available

Authorisation/Restrictions according to EU REACH         Not applicable

Component              CAS No        REACH (1907/2006) -   REACH (1907/2006) -   REACH Regulation (EC

Annex XIV - Substances Annex XVII - Restrictions  1907/2006) article 59  -

Subject to Authorization  on Certain Dangerous      Candidate List of

Substances      Substances of Very High

Concern (SVHC)

Gadolinium fluoride (GdF3)           13765-26-9                             -                                    -                                    -

Safety, health and environmental regulations/legislation specific for the substance or mixture

Component          CAS No        OECD HPV       Persistent Organic   Ozone Depletion       Restriction of

Pollutant             Potential          Hazardous

Substances (RoHS)

Gadolinium fluoride (GdF3)       13765-26-9         Not applicable        Not applicable        Not applicable        Not applicable

Contains component(s) that meet a 'definition' of per & poly fluoroalkyl substance (PFAS)?

Not applicable

Other International Regulations

Component          CAS No       Seveso III Directive  Seveso III Directive      Rotterdam       Basel Convention

(2012/18/EC) -        (2012/18/EC) -     Convention (PIC)   (Hazardous Waste)

Qualifying Quantities Qualifying Quantities

for Major Accident    for Safety Report

Notification        Requirements

Gadolinium fluoride (GdF3)       13765-26-9         Not applicable        Not applicable        Not applicable        Not applicable

16. Other Information

Prepared By                         Health, Safety and Environmental Department

Email: chem.techinfo@thermofisher.com

www.thermofisher.com

Revision Date                     22-Dec-2025

Print Date                        22-Dec-2025

Revision Summary              UpdatedtotheU.S.DepartmentofLabor’sOccupationalSafetyandHealthAdministration

(OSHA) which published its Final Rule in the Federal Register revising the Hazard

Communication Standard (HCS/HazCom), 29 CFR 1910.1200 (2024) (HCS §1910.1200,

2024), May 20, 2024, effective July 19, 2024.

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

______________________________________________________________________________________________

Page 8 / 9

Page 9

Gadolinium(III) fluoride, anhydrous, REacton®                                        Revision Date 22-Dec-2025

______________________________________________________________________________________________

End of SDS

______________________________________________________________________________________________

Page 9 / 9