Gadolinium(III) nitrate hexahydrate 316760000 SDS

Publisher: Acros-Organics

Source: https://resds.com/document/gadolinium-iii-nitrate-hexahydrate-sds-acros-en-507e634e98

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Creation Date 22-Sep-2009                   Revision Date 28-Aug-2015                         Revision Number 3

SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE

COMPANY/UNDERTAKING

1.1. Product identification

Product Description:                Gadolinium(III) nitrate hexahydrate

Cat No. :                         316760000; 316760250

Synonyms                                 Nitric Acid, Gadolinium(3+) Salt, Hexahydrate.

CAS-No                           19598-90-4

Molecular Formula             Gd N3 O9 . 6 H2 O

1.2. Relevant identified uses of the substance or mixture and uses advised against

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

1.3. Details of the supplier of the safety data sheet

Company                         Acros Organics BVBA

Janssen Pharmaceuticalaan 3a

2440 Geel, Belgium

E-mail address                    begel.sdsdesk@thermofisher.com

1.4. Emergency telephone number

For information US call: 001-800-ACROS-01 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No.US:001-800-424-9300 / Europe:001-703-527-3887

SECTION 2: HAZARDS IDENTIFICATION

2.1. Classification of the substance or mixture

CLP Classification - Regulation (EC) No 1272/2008

Physical hazards

Oxidizing solids                                                               Category 2

Health hazards

Serious Eye Damage/Eye Irritation                                              Category 1

Environmental hazards

Based on available data, the classification criteria are not met

2.2. Label elements

______________________________________________________________________________________________

ACR31676

Page 1 / 9

Page 2

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

Signal Word                    Danger

Hazard Statements

H272 - May intensify fire; oxidizer

H318 - Causes serious eye damage

Precautionary Statements

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and

easy to do. Continue rinsing

P310 - Immediately call a POISON CENTER or doctor/ physician

P280 - Wear protective gloves/ protective clothing/ eye protection/ face protection

P210 - Keep away from heat/sparks/open flames/hot surfaces. - No smoking

P221 - Take any precaution to avoid mixing with combustibles

P371 + P380 + P375 - In case of major fire and large quantities: Evacuate area. Fight fire remotely due to the risk of explosion

2.3. Other hazards

No information available

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

3.1. Substances

Component              CAS-No          EC-No.         Weight %   CLP Classification - Regulation (EC) No

1272/2008

Gadolinium (III) nitrate, hexahydrate      19598-90-4                              99.9               Eye Dam. 1 (H318)

Ox. Sol. 2 (H272)

Full text of Hazard Statements: see section 16

SECTION 4: FIRST AID MEASURES

4.1. Description of first aid measures

General Advice                                    If symptoms persist, call a physician.

Eye Contact                       Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes.

Obtain medical attention.

Skin Contact                  Wash off immediately with plenty of water for at least 15 minutes. Obtain medical attention.

Ingestion                         Clean mouth with water and drink afterwards plenty of water. Get medical attention if

symptoms occur.

Inhalation                     Move to fresh air. If breathing is difficult, give oxygen. Obtain medical attention.

Protection of First-aiders         Use personal protective equipment.

4.2. Most important symptoms and effects, both acute and delayed

None reasonably foreseeable. Causes eye burns.

4.3. Indication of any immediate medical attention and special treatment needed

Notes to Physician                  Treat symptomatically.

SECTION 5: FIREFIGHTING MEASURES

5.1. Extinguishing media

______________________________________________________________________________________________

ACR31676

Page 2 / 9

Page 3

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

Suitable Extinguishing Media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Extinguishing media which must not be used for safety reasons

No information available.

5.2. Special hazards arising from the substance or mixture

Burning produces obnoxious and toxic fumes. Oxidizer: Contact with combustible/organic material may cause fire. May ignite

combustibles (wood paper, oil, clothing, etc.).

Hazardous Combustion Products

Nitrogen oxides (NOx).

5.3. Advice for firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

SECTION 6: ACCIDENTAL RELEASE MEASURES

6.1. Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Ensure adequate ventilation. Avoid dust formation.

6.2. Environmental precautions

Should not be released into the environment.

6.3. Methods and material for containment and cleaning up

Sweep up or vacuum up spillage and collect in suitable container for disposal. Keep in suitable, closed containers for disposal.

Soak up with inert absorbent material. Sweep up and shovel into suitable containers for disposal.

6.4. Reference to other sections

Refer to protective measures listed in Sections 8 and 13.

SECTION 7: HANDLING AND STORAGE

7.1. Precautions for safe handling

Wear personal protective equipment. Ensure adequate ventilation. Do not get in eyes, on skin, or on clothing. Avoid ingestion and

inhalation. Avoid dust formation. Keep away from clothing and other combustible materials.

7.2. Conditions for safe storage, including any incompatibilities

Keep containers tightly closed in a dry, cool and well-ventilated place. Do not store near combustible materials.

7.3. Specific end use(s)

Use in laboratories

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1. Control parameters

Exposure limits

This product, as supplied, does not contain any hazardous materials with occupational exposure limits established by the region

specific regulatory bodies

______________________________________________________________________________________________

ACR31676

Page 3 / 9

Page 4

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

Biological limit values

This product, as supplied, does not contain any hazardous materials with biological limits established by the region specific

regulatory bodies.

Monitoring methods

BS EN 14042:2003 Title Identifier: Workplace atmospheres. Guide for the application and use of procedures for the assessment of

exposure to chemical and biological agents.

MDHS14/3 General methods for sampling and gravimetric analysis of respirable and inhalable dust

Derived No Effect Level (DNEL)    No information available

Route of exposure        Acute effects (local)     Acute effects       Chronic effects      Chronic effects

(systemic)               (local)             (systemic)

Oral

Dermal

Inhalation

Predicted No Effect Concentration  No information available.

(PNEC)

8.2. Exposure controls

Engineering Measures

Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations and safety showers are close to the

workstation location.

Wherever possible, engineering control measures such as the isolation or enclosure of the process, the introduction of process or

equipment changes to minimise release or contact, and the use of properly designed ventilation systems, should be adopted to

control hazardous materials at source

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Natural rubber     See manufacturers               -          EN 374             (minimum requirement)

Nitrile rubber       recommendations

Neoprene

PVC

Skin and body protection       Long sleeved clothing

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use       Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

______________________________________________________________________________________________

ACR31676

Page 4 / 9

Page 5

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: Particulates filter conforming to EN 143

Small scale/Laboratory use        Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

limits are exceeded or if irritation or other symptoms are experienced.

Recommended half mask:- Particle filtering: EN149:2001

When RPE is used a face piece Fit Test should be conducted

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls  No information available.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

9.1. Information on basic physical and chemical properties

Appearance                       White

Physical State                        Solid

Odor                       No information available

Odor Threshold               No data available

pH                         No information available

Melting Point/Range            No data available

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Evaporation Rate                  Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Vapor Pressure                No data available

Vapor Density                     Not applicable                         Solid

Specific Gravity  / Density        No data available

Bulk Density                  No data available

Water Solubility                      soluble

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Autoignition Temperature

Decomposition Temperature      No data available

Viscosity                         Not applicable                         Solid

Explosive Properties            No information available

Oxidizing Properties                 Oxidizer

9.2. Other information

Molecular Formula             Gd N3 O9 . 6 H2 O

Molecular Weight                  451.4

SECTION 10: STABILITY AND REACTIVITY

10.1. Reactivity

Yes

10.2. Chemical stability

Stable under normal conditions: Oxidizer: Contact with combustible/organic material may

cause fire

10.3. Possibility of hazardous reactions

Hazardous Polymerization         Hazardous polymerization does not occur.

Hazardous Reactions            None under normal processing.

______________________________________________________________________________________________

ACR31676

Page 5 / 9

Page 6

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

10.4. Conditions to avoid

Incompatible products. Excess heat. Avoid dust formation. Combustible material.

10.5. Incompatible materials

Strong acids. Strong reducing agents. Metals. Powdered metals. Combustible material.

10.6. Hazardous decomposition products

Nitrogen oxides (NOx).

SECTION 11: TOXICOLOGICAL INFORMATION

11.1. Information on toxicological effects

Product Information            No acute toxicity information is available for this product

(a) acute toxicity;

Oral                        Based on available data, the classification criteria are not met

Dermal                   No data available

Inhalation                 No data available

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Gadolinium (III) nitrate, hexahydrate        LD50 > 5 g/kg  ( Rat )

(b) skin corrosion/irritation;       No data available

(c) serious eye damage/irritation;   Category 1

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;             No data available

There are no known carcinogenic chemicals in this product

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

Target Organs              No information available.

(j) aspiration hazard;               Not applicable

Solid

Other Adverse Effects            The toxicological properties have not been fully investigated. See actual entry in RTECS for

complete information

Symptoms  / effects,both acute and No information available

delayed

SECTION 12: ECOLOGICAL INFORMATION

12.1. Toxicity

Ecotoxicity effects                 Contains no substances known to be hazardous to the environment or that are not

degradable in waste water treatment plants.

______________________________________________________________________________________________

ACR31676

Page 6 / 9

Page 7

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

12.2. Persistence and degradability

Persistence                     Soluble in water, Persistence is unlikely, based on information available.

Degradability                  Not relevant for inorganic substances.

12.3. Bioaccumulative potential     Bioaccumulation is unlikely

12.4. Mobility in soil              The product is water soluble, and may spread in water systems Will likely be mobile in the

environment due to its water solubility. Highly mobile in soils

12.5. Results of PBT and vPvB     No data available for assessment.

assessment

12.6. Other adverse effects

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13: DISPOSAL CONSIDERATIONS

13.1. Waste treatment methods

Waste from Residues / Unused     Waste is classified as hazardous. Dispose of in accordance with the European Directives

Products                       on waste and hazardous waste. Dispose of in accordance with local regulations.

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point.

European Waste Catalogue (EWC)  According to the European Waste Catalogue, Waste Codes are not product specific, but

application specific.

Other Information                Waste codes should be assigned by the user based on the application for which the product

was used. Do not empty into drains. Do not dispose of waste into sewer.

SECTION 14: TRANSPORT INFORMATION

IMDG/IMO

14.1. UN number                UN1477

14.2. UN proper shipping name     NITRATES, INORGANIC, N.O.S

14.3. Transport hazard class(es)     5.1

14.4. Packing group                        III

ADR

14.1. UN number                UN1477

14.2. UN proper shipping name     NITRATES, INORGANIC, N.O.S

14.3. Transport hazard class(es)     5.1

14.4. Packing group                        III

IATA

14.1. UN number                UN1477

14.2. UN proper shipping name     NITRATES, INORGANIC, N.O.S

14.3. Transport hazard class(es)     5.1

14.4. Packing group                        III

14.5. Environmental hazards      No hazards identified

14.6. Special precautions for user  No special precautions required

14.7. Transport in bulk according to Not applicable, packaged goods

Annex II of MARPOL73/78 and the

IBC Code

______________________________________________________________________________________________

ACR31676

Page 7 / 9

Page 8

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

SECTION 15: REGULATORY INFORMATION

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

International Inventories         X = listed The product is classified and labeled according to EC directives or corresponding

national laws The product is classified and labeled in accordance with Directive 1999/45/EC

National Regulations

Take note of Control of Substances Hazardous to Health Regulations (COSHH) 2002 and 2005 Amendment.

Take note of Dir 94/33/EC on the protection of young people at work

Take note of Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents at

work

15.2. Chemical safety assessment

A Chemical Safety Assessment/Report (CSA/CSR) has not been conducted

SECTION 16: OTHER INFORMATION

Full Text of H-/EUH-Statements Referred to Under Section 3

H272 - May intensify fire; oxidizer

H318 - Causes serious eye damage

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ADR - European Agreement Concerning the International Carriage of    ICAO/IATA - International Civil Aviation Organization/International Air

Dangerous Goods by Road                                           Transport Association

IMO/IMDG - International Maritime Organization/International Maritime  MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods Code                                            Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - Volatile Organic Compounds

Key literature references and sources for data

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Use of personal protective equipment, covering appropriate selection, compatibility, breakthrough thresholds, care, maintenance, fit

and standards.

First aid for chemical exposure, including the use of eye wash and safety showers.

Creation Date                     22-Sep-2009

Revision Date                     28-Aug-2015

Revision Summary                Update to Format.

______________________________________________________________________________________________

ACR31676

Page 8 / 9

Page 9

SAFETY DATA SHEET

Gadolinium(III) nitrate hexahydrate                                                       Revision Date 28-Aug-2015

______________________________________________________________________________________________

This safety data sheet complies with the requirements of Regulation (EC) No. 1907/2006

Disclaimer

The information provided on this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guide for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered as a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

material or in any process, unless specified in the text.

End of Safety Data Sheet

______________________________________________________________________________________________

ACR31676

Page 9 / 9