Gadolinium(III) nitrate hexahydrate AC316760000 SDS

Publisher: Thermo Fisher Scientific

Source: https://resds.com/document/gadolinium-iii-nitrate-hexahydrate-sds-thermo-fisher-en-ffcaaa5e3f

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Creation Date 22-Sep-2009                  Revision Date 19-Dec-2025                          Revision Number 7

This safety data sheet was created pursuant to the requirements of: US OSHA Hazard Communication Standard 2024 (29

CFR 1910.1200)

1. Identification

Product Name                Gadolinium(III) nitrate hexahydrate

Cat No. :                  AC316760000; AC316760250; AC316761000

CAS No                           19598-90-4

Synonyms                                 Nitric Acid, Gadolinium(3+) Salt, Hexahydrate.

Recommended Use                 Laboratory chemicals.

Uses advised against              Food, drug, pesticide or biocidal product use.

Details of the supplier of the safety data sheet

Company

Fisher Scientific Company                 Acros Organics

One Reagent Lane                   One Reagent Lane

Fair Lawn, NJ 07410                          Fair Lawn, NJ 07410

Tel: (201) 796-7100

Emergency Telephone Number

For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

2. Hazard(s) identification

Classification

This chemical is considered hazardous according to [US] OSHA (29 CFR 1910.1200, 2024)

Oxidizing solids                                           Category 2

Serious Eye Damage/Eye Irritation                           Category 1

Label Elements

Signal Word

Danger

Hazard Statements

May intensify fire; oxidizer

Causes serious eye damage

______________________________________________________________________________________________

Page 1 / 9

Page 2

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

Precautionary Statements

Prevention

Wear protective gloves/protective clothing/eye protection/face protection

Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking

Keep away from clothing and other combustible materials

Take any precaution to avoid mixing with combustibles

Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

Immediately call a POISON CENTER or doctor/physician

Fire

In case of fire: Use CO2, dry chemical, or foam to extinguish

Disposal

Dispose of contents/container to an approved waste disposal plant

Hazards not otherwise classified (HNOC)

None identified

Hazards classified under paragraph (d)(1)(ii) of 1910.1200

No information available

3. Composition/information on Ingredients

Component                      CAS No                        Weight %

Gadolinium (III) nitrate, hexahydrate                   19598-90-4                             99.9

Gadolinium nitrate                           10168-81-7                                            -

4. First-aid measures

General Advice                                    If symptoms persist, call a physician.

Eye Contact                       Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Get

medical attention.

Skin Contact                  Wash off immediately with plenty of water for at least 15 minutes. If skin irritation persists,

call a physician.

Inhalation                    Remove to fresh air. If not breathing, give artificial respiration. Get medical attention if

symptoms occur.

Ingestion                         Clean mouth with water and drink afterwards plenty of water. Get medical attention if

symptoms occur.

Most important symptoms and     None reasonably foreseeable. Causes severe eye damage.

effects

Notes to Physician                  Treat symptomatically

5. Fire-fighting measures

Suitable Extinguishing Media      Use extinguishing measures that are appropriate to local circumstances and the

______________________________________________________________________________________________

Page 2 / 9

Page 3

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

surrounding environment. Dry chemical, CO 2, water spray or alcohol-resistant foam.

Unsuitable Extinguishing Media    No information available

Flash Point                No information available

Method -                  No information available

Autoignition Temperature        No information available

Explosion Limits

Upper                    No data available

Lower                    No data available

Oxidizing Properties                 Oxidizer

Sensitivity to Mechanical Impact No information available

Sensitivity to Static Discharge  No information available

Specific Hazards Arising from the Chemical

Burning produces obnoxious and toxic fumes. Oxidizer: Contact with combustible/organic material may cause fire. May ignite

combustibles (wood paper, oil, clothing, etc.).

Hazardous Combustion Products

Nitrogen oxides (NOx). Gadolinium oxide.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

NFPA

Health                   Flammability                     Instability                Physical hazards

3                         0                         1                   OX

6. Accidental release measures

Personal Precautions            Use personal protective equipment as required. Ensure adequate ventilation. Avoid dust

formation.

Environmental Precautions         Should not be released into the environment. Do not allow material to contaminate ground

water system. Do not flush into surface water or sanitary sewer system.

Methods for Containment and Clean Sweep up and shovel into suitable containers for disposal. Keep in suitable, closed

Up                                   containers for disposal. Soak up with inert absorbent material. Sweep up and shovel into

suitable containers for disposal.

7. Handling and Storage

Handling                      Wear personal protective equipment/face protection. Ensure adequate ventilation. Do not

get in eyes, on skin, or on clothing. Avoid ingestion and inhalation. Avoid dust formation.

Keep away from clothing and other combustible materials.

Storage.                       Keep containers tightly closed in a dry, cool and well-ventilated place. Do not store near

combustible materials. Store under an inert atmosphere. Air sensitive. Incompatible

Materials. Strong acids. Strong reducing agents. Finely powdered metals. Combustible

material.

8. Exposure controls / personal protection

Exposure Guidelines                This product does not contain any hazardous materials with occupational exposure

limitsestablished by the region specific regulatory bodies.

Engineering Measures             Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations

and safety showers are close to the workstation location.

______________________________________________________________________________________________

Page 3 / 9

Page 4

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

Personal Protective Equipment

Eye/face Protection              Tight sealing safety goggles.

Skin and body protection      Wear appropriate protective gloves and clothing to prevent skin exposure.

Respiratory Protection          Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard

EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirator if

exposure limits are exceeded or if irritation or other symptoms are experienced.

Recommended Filter type:       Particulates filter conforming to EN 143.

Hygiene Measures             Handle in accordance with good industrial hygiene and safety practice.

9. Physical and chemical properties

Appearance

Physical State                        Solid

Color                             White

Odor                       No information available

Odor Threshold               No information available

Property                         Values                       Remarks   •Method

Melting Point/Range               91 °C  / 195.8 °F

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Flammability (liquid)               Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Autoignition Temperature        No data available

Decomposition Temperature      No data available

pH                               Not applicable

Viscosity                         Not applicable                         Solid

Water Solubility                     Soluble

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Vapor Pressure                No data available

Density  / Specific Gravity        No data available

Bulk Density                  No data available

Vapor Density                     Not applicable                         Solid

Particle characteristics          No data available

Other Information

Molecular Formula             Gd N3 O9 . 6 H2 O

Molecular Weight                  451.4

Oxidizing Properties                 Oxidizer

Evaporation Rate                  Not applicable  - Solid

10. Stability and reactivity

Reactive Hazard                 Yes

Stability                               Oxidizer: Contact with combustible/organic material may cause fire. Hygroscopic.

Conditions to Avoid                 Incompatible products. Excess heat. Avoid dust formation. Combustible material. Exposure

______________________________________________________________________________________________

Page 4 / 9

Page 5

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

to moist air or water.

Incompatible Materials             Strong acids, Strong reducing agents, Finely powdered metals, Combustible material

Hazardous Decomposition Products Nitrogen oxides (NOx), Gadolinium oxide

Hazardous Polymerization         Hazardous polymerization does not occur.

Hazardous Reactions            None under normal processing.

11. Toxicological information

Information on expected route of exposure

Inhalation                        Not an expected route of exposure.

Ingestion                     May be harmful if swallowed.

Eyes                              Avoid contact with eyes. Corrosive to the eyes and may cause severe damage including

blindness.

Skin                               Avoid contact with skin. Skin Corrosion/Irritation.

Toxicology data for the components

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Gadolinium (III) nitrate, hexahydrate        LD50 > 5 g/kg  ( Rat )                              -                                            -

Gadolinium nitrate            LD50 = 3805 mg/kg  ( Rat )                          -                                            -

Toxicologically Synergistic       No information available

Products

(b) skin corrosion/irritation;       No data available

(c) serious eye damage/irritation;   Category 1

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;

The table below indicates whether each agency has listed any ingredient as a carcinogen

Component       CAS No          IARC          NTP          ACGIH        OSHA           Mexico

Gadolinium (III) nitrate,    19598-90-4         Not listed          Not listed          Not listed          Not listed          Not listed

hexahydrate

Gadolinium nitrate      10168-81-7         Not listed          Not listed          Not listed          Not listed          Not listed

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

______________________________________________________________________________________________

Page 5 / 9

Page 6

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

Target Organs               None known.

(j) aspiration hazard;               Not applicable

Solid

Symptoms  / effects,both acute and No information available.

delayed

Other Adverse Effects            The toxicological properties have not been fully investigated.

Endocrine Disrupting Properties    This product does not contain any known or suspected endocrine disruptors.

12. Ecological information

Ecotoxicity

May cause long-term adverse effects in the environment. Do not allow material to contaminate ground water system.

Persistence and Degradability      based on information available. May persist

Bioaccumulation/ Accumulation   No information available.

Mobility                                  Will likely be mobile in the environment due to its water solubility.

13. Disposal considerations

Waste Disposal Methods           Chemical waste generators must determine whether a discarded chemical is classified as a

hazardous waste. Chemical waste generators must also consult local, regional, and

national hazardous waste regulations to ensure complete and accurate classification.

14. Transport information

DOT

UN-No                     UN1477

Proper Shipping Name        NITRATES, INORGANIC, N.O.S.

Technical Shipping Name       Gadolinium (III) nitrate, hexahydrate

Hazard Class                    5.1

Packing Group                               II

TDG

UN-No                     UN1477

Proper Shipping Name        NITRATES, INORGANIC, N.O.S.

Technical Shipping Name       Gadolinium (III) nitrate, hexahydrate

Hazard Class                    5.1

Packing Group                               II

IATA

UN-No                     UN1477

Proper Shipping Name        NITRATES, INORGANIC, N.O.S.

Technical Shipping Name       Gadolinium (III) nitrate, hexahydrate

Hazard Class                    5.1

Packing Group                               II

IMDG/IMO

UN-No                     UN1477

Proper Shipping Name        NITRATES, INORGANIC, N.O.S.

Technical Shipping Name       Gadolinium (III) nitrate, hexahydrate

Hazard Class                    5.1

Packing Group                               II

15. Regulatory Information

United States of America Inventory

______________________________________________________________________________________________

Page 6 / 9

Page 7

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

Component           CAS No          TSCA        TSCA Inventory notification -    TSCA - EPA Regulatory

Active-Inactive                   Flags

Gadolinium (III) nitrate,          19598-90-4                     -                                        -                                            -

hexahydrate

Gadolinium nitrate            10168-81-7          X                   ACTIVE                                       -

Legend:

TSCA US EPA (TSCA) - Toxic Substances Control Act, (40 CFR Part 710)

X - Listed

'-' - Not Listed

TSCA - Per 40 CFR 751, Regulation of Certain Chemical     Not applicable

Substances & Mixtures, Under TSCA Section 6(h) (PBT)

TSCA 12(b) - Notices of Export                             Not applicable

International Inventories

No information available, X = listed, U.S.A. (TSCA), Canada (DSL/NDSL), Europe (EINECS/ELINCS/NLP), Australia (AICS), Korea (KECL), China

(IECSC), Japan (ENCS), Philippines (PICCS), Taiwan (TCSI), Japan (ISHL), New Zealand (NZIoC), Japan (ISHL).

Component          CAS No     DSL   NDSL   EINECS  PICCS  ENCS    ISHL    AICS   IECSC   KECL

Gadolinium (III) nitrate,        19598-90-4          -             -              -              -             -                          -             -               -

hexahydrate

Gadolinium nitrate          10168-81-7     X            -     233-437-2       -      X      X            -             -     KE-17376

KECL - NIER number or KE number (http://ncis.nier.go.kr/en/main.do)

U.S. Federal Regulations

SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product contains a chemical

or chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372.

Component           CAS No                Weight %        SARA 313 - Threshold    SARA 313 - Reporting

Values %                threasholds

Gadolinium (III) nitrate,         19598-90-4                    99.9                       1.0 %                                   -

hexahydrate

SARA 311/312 Hazard Categories

Should this product meet EPCRA 311/312 Tier reporting criteria at 40 CFR 370, refer to Section 2 of this SDS for appropriate

classifications.

CWA (Clean Water Act)            Not applicable

Clean Air Act                     Not applicable

OSHA - Occupational Safety and     Not applicable

Health Administration

CERCLA

This material, as supplied, does not contain any substances regulated as hazardous substances under the Comprehensive

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund Amendments and

Reauthorization Act (SARA) (40 CFR 355). There may be specific reporting requirements at the local, regional, or state level

pertaining to releases of this material.

California Proposition 65            This product does not contain any Proposition 65 chemicals.

U.S. State Right-to-Know

Regulations

______________________________________________________________________________________________

Page 7 / 9

Page 8

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

Component         Massachusetts      New Jersey        Pennsylvania              Illinois         Rhode Island

Gadolinium (III) nitrate,                  -               X                              -               X                              -

hexahydrate

Gadolinium nitrate                     -               X                              -               X                              -

U.S. Department of Transportation

Reportable Quantity (RQ):        N

DOT Marine Pollutant           N

DOT Severe Marine Pollutant      N

U.S. Department of Homeland       This product does not contain any DHS chemicals.

Security

Other International Regulations

Mexico - Grade                No information available

Authorisation/Restrictions according to EU REACH         Not applicable

Component              CAS No        REACH (1907/2006) -   REACH (1907/2006) -   REACH Regulation (EC

Annex XIV - Substances Annex XVII - Restrictions  1907/2006) article 59  -

Subject to Authorization  on Certain Dangerous      Candidate List of

Substances      Substances of Very High

Concern (SVHC)

Gadolinium (III) nitrate, hexahydrate       19598-90-4                             -                                    -                                    -

Gadolinium nitrate               10168-81-7                             -                                    -                                    -

Safety, health and environmental regulations/legislation specific for the substance or mixture

Component          CAS No        OECD HPV       Persistent Organic   Ozone Depletion       Restriction of

Pollutant             Potential          Hazardous

Substances (RoHS)

Gadolinium (III) nitrate,         19598-90-4         Not applicable        Not applicable        Not applicable        Not applicable

hexahydrate

Gadolinium nitrate           10168-81-7         Not applicable        Not applicable        Not applicable        Not applicable

Contains component(s) that meet a 'definition' of per & poly fluoroalkyl substance (PFAS)?

Not applicable

Other International Regulations

Component          CAS No       Seveso III Directive  Seveso III Directive      Rotterdam       Basel Convention

(2012/18/EC) -        (2012/18/EC) -     Convention (PIC)   (Hazardous Waste)

Qualifying Quantities Qualifying Quantities

for Major Accident    for Safety Report

Notification        Requirements

Gadolinium (III) nitrate,         19598-90-4         Not applicable        Not applicable        Not applicable        Not applicable

hexahydrate

Gadolinium nitrate           10168-81-7         Not applicable        Not applicable        Not applicable        Not applicable

16. Other Information

Prepared By                       Product stewardship (Regulatory Affairs)

Thermo Fisher Scientific

email - begel.sdsdesk@thermofisher.com

______________________________________________________________________________________________

Page 8 / 9

Page 9

Gadolinium(III) nitrate hexahydrate                                                   Revision Date 19-Dec-2025

______________________________________________________________________________________________

Creation Date                     22-Sep-2009

Revision Date                     19-Dec-2025

Print Date                        19-Dec-2025

Revision Summary              UpdatedtotheU.S.DepartmentofLabor’sOccupationalSafetyandHealthAdministration

(OSHA) which published its Final Rule in the Federal Register revising the Hazard

Communication Standard (HCS/HazCom), 29 CFR 1910.1200 (2024) (HCS §1910.1200,

2024), May 20, 2024, effective July 19, 2024.

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of SDS

______________________________________________________________________________________________

Page 9 / 9