Gadolinium(III) oxide 278513 SDS

Publisher: Sigma-Aldrich

Source: https://resds.com/document/gadolinium-iii-oxide-sds-sigma-en-6b6573eee5

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SIGMA-ALDRICH                                                                                sigma-aldrich.com

SAFETY DATA SHEET

Version 4.9

Revision Date 09/28/2017

Print Date 11/10/2018

1. PRODUCT AND COMPANY IDENTIFICATION

1.1   Product identifiers

Product name                        :  Gadolinium(III) oxide

Product Number                     :  278513

Brand                                     :   Aldrich

CAS-No.                                 :  12064-62-9

1.2   Relevant identified uses of the substance or mixture and uses advised against

Identified uses                        :  Laboratory chemicals, Synthesis of substances

1.3    Details of the supplier of the safety data sheet

Company                               :  Sigma-Aldrich

3050 Spruce Street

SAINT LOUIS MO 63103

USA

Telephone                              :  +1 800-325-5832

Fax                                         :  +1 800-325-5052

1.4   Emergency telephone number

Emergency Phone #              :  +1-703-527-3887 (CHEMTREC)

2. HAZARDS IDENTIFICATION

2.1    Classification of the substance or mixture

GHS Classification in accordance with 29 CFR 1910 (OSHA HCS)

Eye irritation (Category 2A), H319

For the full text of the H-Statements mentioned in this Section, see Section 16.

2.2  GHS Label elements, including precautionary statements

Pictogram

Signal word                 Warning

Hazard statement(s)

H319                     Causes serious eye irritation.

Precautionary statement(s)

P264                   Wash skin thoroughly after handling.

P280                    Wear eye protection/ face protection.

P305 + P351 + P338            IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P337 + P313                               If eye irritation persists: Get medical advice/ attention.

2.3   Hazards not otherwise classified (HNOC) or not covered by GHS - none

3. COMPOSITION/INFORMATION ON INGREDIENTS

3.1   Substances

Formula                                  :  Gd2O3

Aldrich - 278513                                                                                                Page 1 of 7

Page 2

Molecular weight                    :  362.50 g/mol

CAS-No.                                 :  12064-62-9

EC-No.                                   :  235-060-9

Hazardous components

Component                                                Classification                  Concentration

Digadolinium trioxide

Eye Irrit. 2A; H319           90 - 100 %

For the full text of the H-Statements mentioned in this Section, see Section 16.

4. FIRST AID MEASURES

4.1   Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.Move out of dangerous area.

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

4.2   Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

4.3    Indication of any immediate medical attention and special treatment needed

No data available

5. FIREFIGHTING MEASURES

5.1   Extinguishing media

Suitable extinguishing media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

5.2   Special hazards arising from the substance or mixture

No data available

5.3   Advice for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

5.4   Further information

The product itself does not burn.

6. ACCIDENTAL RELEASE MEASURES

6.1   Personal precautions, protective equipment and emergency procedures

Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate

ventilation. Avoid breathing dust.

For personal protection see section 8.

6.2   Environmental precautions

Do not let product enter drains.

6.3   Methods and materials for containment and cleaning up

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for

disposal.

Aldrich - 278513                                                                                                Page 2 of 7

Page 3

6.4   Reference to other sections

For disposal see section 13.

7. HANDLING AND STORAGE

7.1   Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.Further processing of solid materials may result in

the formation of combustible dusts. The potential for combustible dust formation should be taken into consideration

before additional processing occurs.

Provide appropriate exhaust ventilation at places where dust is formed.

For precautions see section 2.2.

7.2   Conditions for safe storage, including any incompatibilities

Keep container tightly closed in a dry and well-ventilated place.

Hygroscopic. Store under inert gas. Keep in a dry place.

7.3    Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1   Control parameters

Components with workplace control parameters

Contains no substances with occupational exposure limit values.

8.2   Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of

workday.

Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved

under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without

touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after

use in accordance with applicable laws and good laboratory practices. Wash and dry hands.

Full contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm

Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

Splash contact

Material: Nitrile rubber

Minimum layer thickness: 0.11 mm

Break through time: 480 min

Material tested:Dermatril® (KCL 740 / Aldrich Z677272, Size M)

data source: KCL GmbH, D-36124 Eichenzell, phone +49 (0)6659 87300, e-mail sales@kcl.de, test method:

EN374

If used in solution, or mixed with other substances, and under conditions which differ from EN 374, contact the

supplier of the CE approved gloves. This recommendation is advisory only and must be evaluated by an

industrial hygienist and safety officer familiar with the specific situation of anticipated use by our customers. It

should not be construed as offering an approval for any specific use scenario.

Body Protection

Impervious clothing, The type of protective equipment must be selected according to the concentration and

amount of the dangerous substance at the specific workplace.

Aldrich - 278513                                                                                                Page 3 of 7

Page 4

Respiratory protection

For nuisance exposures use type P95 (US) or type P1 (EU EN 143) particle respirator.For higher level

protection use type OV/AG/P99 (US) or type ABEK-P2 (EU EN 143) respirator cartridges. Use respirators and

components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Control of environmental exposure

Do not let product enter drains.

9. PHYSICAL AND CHEMICAL PROPERTIES

9.1   Information on basic physical and chemical properties

a)  Appearance            Form: powder

Colour: beige

b)  Odour              No data available

c)  Odour Threshold      No data available

d)  pH                No data available

e)  Melting point/freezing     Melting point/range: 2,330 °C (4,226 °F) - lit.

point

f)    Initial boiling point and   No data available

boiling range

g)  Flash point             Not applicable

h)  Evaporation rate       No data available

i)   Flammability (solid, gas) No data available

j)   Upper/lower          No data available

flammability or

explosive limits

k)  Vapour pressure       No data available

l)   Vapour density        No data available

m)  Relative density         7.407 g/mL at 20 °C (68 °F)

n)  Water solubility        No data available

o)   Partition coefficient: n-   No data available

octanol/water

p)  Auto-ignition          No data available

temperature

q)  Decomposition        No data available

temperature

r)   Viscosity            No data available

s)  Explosive properties    No data available

t)   Oxidizing properties    No data available

9.2   Other safety information

No data available

10. STABILITY AND REACTIVITY

10.1  Reactivity

No data available

10.2  Chemical stability

Stable under recommended storage conditions.

10.3  Possibility of hazardous reactions

No data available

Aldrich - 278513                                                                                                Page 4 of 7

Page 5

10.4  Conditions to avoid

No data available

10.5  Incompatible materials

Strong acids

10.6  Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - gadolinium oxides

Other decomposition products - No data available

In the event of fire: see section 5

11. TOXICOLOGICAL INFORMATION

11.1  Information on toxicological effects

Acute toxicity

LD50 Oral - Rat - > 5,000 mg/kg

Inhalation: No data available

Dermal: No data available

No data available

Skin corrosion/irritation

No data available

Serious eye damage/eye irritation

Eyes - Rabbit

Result: Mild eye irritation

Respiratory or skin sensitisation

No data available

Germ cell mutagenicity

No data available

Carcinogenicity

IARC:    No component of this product present at levels greater than or equal to 0.1% is identified as

probable, possible or confirmed human carcinogen by IARC.

ACGIH:   No component of this product present at levels greater than or equal to 0.1% is identified as a

carcinogen or potential carcinogen by ACGIH.

NTP:     No component of this product present at levels greater than or equal to 0.1% is identified as a

known or anticipated carcinogen by NTP.

OSHA:    No component of this product present at levels greater than or equal to 0.1% is identified as a

carcinogen or potential carcinogen by OSHA.

Reproductive toxicity

No data available

No data available

Specific target organ toxicity - single exposure

No data available

Specific target organ toxicity - repeated exposure

No data available

Aspiration hazard

No data available

Additional Information

RTECS: Not available

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Aldrich - 278513                                                                                                Page 5 of 7

Page 6

12. ECOLOGICAL INFORMATION

12.1  Toxicity

No data available

12.2  Persistence and degradability

No data available

12.3  Bioaccumulative potential

No data available

12.4  Mobility in soil

No data available

12.5  Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

12.6  Other adverse effects

No data available

13. DISPOSAL CONSIDERATIONS

13.1  Waste treatment methods

Product

Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed professional waste

disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

14. TRANSPORT INFORMATION

DOT (US)

Not dangerous goods

IMDG

Not dangerous goods

IATA

Not dangerous goods

15. REGULATORY INFORMATION

SARA 302 Components

No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 313 Components

This material does not contain any chemical components with known CAS numbers that exceed the threshold (De

Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards

Acute Health Hazard

Massachusetts Right To Know Components

No components are subject to the Massachusetts Right to Know Act.

Pennsylvania Right To Know Components

CAS-No.           Revision Date

Digadolinium trioxide                                       12064-62-9

New Jersey Right To Know Components

CAS-No.           Revision Date

Digadolinium trioxide                                       12064-62-9

California Prop. 65 Components

Aldrich - 278513                                                                                                Page 6 of 7

Page 7

This product does not contain any chemicals known to State of California to cause cancer, birth defects, or any other

reproductive harm.

16. OTHER INFORMATION

Full text of H-Statements referred to under sections 2 and 3.

Eye Irrit.          Eye irritation

H319            Causes serious eye irritation.

HMIS Rating

Health hazard:           2

Chronic Health Hazard:

Flammability:            0

Physical Hazard         0

NFPA Rating

Health hazard:           2

Fire Hazard:            0

Reactivity Hazard:        0

Further information

Copyright 2016 Sigma-Aldrich Co. LLC. License granted to make unlimited paper copies for internal use only.

The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a

guide. The information in this document is based on the present state of our knowledge and is applicable to the

product with regard to appropriate safety precautions. It does not represent any guarantee of the properties of the

product. Sigma-Aldrich Corporation and its Affiliates shall not be held liable for any damage resulting from handling

or from contact with the above product. See www.sigma-aldrich.com and/or the reverse side of invoice or packing

slip for additional terms and conditions of sale.

Preparation Information

Sigma-Aldrich Corporation

Product Safety – Americas Region

1-800-521-8956

Version: 4.9                      Revision Date: 09/28/2017          Print Date: 11/10/2018

Aldrich - 278513                                                                                                Page 7 of 7