Gadolinium(III) oxide AC315510000 SDS

Publisher: Thermo Fisher Scientific

Source: https://resds.com/document/gadolinium-iii-oxide-sds-thermo-fisher-en-1cfb7415b1

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Creation Date 26-Sep-2009                  Revision Date 19-Dec-2025                          Revision Number 5

This safety data sheet was created pursuant to the requirements of: US OSHA Hazard Communication Standard 2024 (29

CFR 1910.1200)

1. Identification

Product Name                Gadolinium(III) oxide

Cat No. :                  AC315510000; AC315510100; AC315510250; AC315511000

CAS No                           12064-62-9

Synonyms                           Gadolinium(III)Oxide; Gadolinia; Digadolinium Trioxide

Recommended Use                 Laboratory chemicals.

Uses advised against              Food, drug, pesticide or biocidal product use.

Details of the supplier of the safety data sheet

Company

Fisher Scientific Company                 Acros Organics

One Reagent Lane                   One Reagent Lane

Fair Lawn, NJ 07410                          Fair Lawn, NJ 07410

Tel: (201) 796-7100

Emergency Telephone Number

For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

2. Hazard(s) identification

Classification

This chemical is considered hazardous according to [US] OSHA (29 CFR 1910.1200, 2024)

Serious Eye Damage/Eye Irritation                           Category 2

Label Elements

Signal Word

Warning

Hazard Statements

Causes serious eye irritation

______________________________________________________________________________________________

Page 1 / 8

Page 2

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

Precautionary Statements

Prevention

Wash face, hands and any exposed skin thoroughly after handling

Wear eye/face protection

Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

If eye irritation persists: Get medical advice/attention

Hazards not otherwise classified (HNOC)

None identified

Hazards classified under paragraph (d)(1)(ii) of 1910.1200

No information available

3. Composition/information on Ingredients

Component                      CAS No                        Weight %

Gadolinium oxide (Gd2O3)                       12064-62-9                         >95

4. First-aid measures

Eye Contact                       Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Get

medical attention.

Skin Contact                  Wash off immediately with plenty of water for at least 15 minutes. Get medical attention.

Inhalation                    Remove to fresh air. If not breathing, give artificial respiration. Get medical attention.

Ingestion                    Do NOT induce vomiting. Get medical attention.

Most important symptoms and    No information available.

effects

Notes to Physician                  Treat symptomatically

5. Fire-fighting measures

Suitable Extinguishing Media      Carbon dioxide (CO 2). Dry chemical. Chemical foam.

Unsuitable Extinguishing Media    No information available

Flash Point                No information available

Method -                  No information available

Autoignition Temperature        No information available

Explosion Limits

Upper                    No data available

Lower                    No data available

Sensitivity to Mechanical Impact No information available

Sensitivity to Static Discharge  No information available

______________________________________________________________________________________________

Page 2 / 8

Page 3

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

Specific Hazards Arising from the Chemical

Keep product and empty container away from heat and sources of ignition.

Hazardous Combustion Products

None known.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

NFPA

Health                   Flammability                     Instability                Physical hazards

2                         0                         1                        N/A

6. Accidental release measures

Personal Precautions              Ensure adequate ventilation. Use personal protective equipment as required. Avoid dust

formation.

Environmental Precautions         Should not be released into the environment.

Methods for Containment and Clean Sweep up and shovel into suitable containers for disposal. Avoid dust formation.

Up

7. Handling and Storage

Handling                         Ensure adequate ventilation. Wear personal protective equipment/face protection. Avoid

contact with skin and eyes. Do not breathe dust. Do not ingest. If swallowed then seek

immediate medical assistance. Minimize dust generation and accumulation. Wash hands

before breaks and immediately after handling the product.

Storage.                       Keep in a dry, cool and well-ventilated place. Keep container tightly closed. Keep under

nitrogen. Incompatible Materials. Strong acids. Finely powdered metals.

8. Exposure controls / personal protection

Exposure Guidelines                This product does not contain any hazardous materials with occupational exposure

limitsestablished by the region specific regulatory bodies.

Engineering Measures             Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations

and safety showers are close to the workstation location.

Personal Protective Equipment

Eye/face Protection           Wear appropriate protective eyeglasses or chemical safety goggles as described by

OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard

EN166.

Skin and body protection      Wear appropriate protective gloves and clothing to prevent skin exposure.

Respiratory Protection        No protective equipment is needed under normal use conditions.

Hygiene Measures             Handle in accordance with good industrial hygiene and safety practice.

9. Physical and chemical properties

Appearance

Physical State                   Powder Solid

Color                             White

______________________________________________________________________________________________

Page 3 / 8

Page 4

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

Odor                       No information available

Odor Threshold               No information available

Property                         Values                       Remarks   •Method

Melting Point/Range               1900 °C  / 3452 °F

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Flammability (liquid)               Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Autoignition Temperature        No data available

Decomposition Temperature      No data available

pH                         No information available

Viscosity                         Not applicable                         Solid

Water Solubility                      Insoluble

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Vapor Pressure                No data available

Density  / Specific Gravity        No data available

Bulk Density                  No data available

Vapor Density                     Not applicable                         Solid

Particle characteristics          No data available

Other Information

Molecular Formula              Gd2 O3

Molecular Weight                  362.5

Evaporation Rate                  Not applicable  - Solid

10. Stability and reactivity

Reactive Hazard                None known, based on information available

Stability                            Hygroscopic. Air sensitive.

Conditions to Avoid               Exposure to air. Incompatible products. Exposure to moisture. Avoid dust formation.

Incompatible Materials             Strong acids, Finely powdered metals

Hazardous Decomposition Products None under normal use conditions

Hazardous Polymerization        No information available.

Hazardous Reactions            None under normal processing.

11. Toxicological information

Information on expected route of exposure

Inhalation                         Avoid breathing dust or spray mist. May be harmful if inhaled.

Ingestion                     May be harmful if swallowed.

Eyes                              Avoid contact with eyes. Irritating to eyes.

Skin                               Avoid contact with skin. May cause irritation.

Toxicology data for the components

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Gadolinium oxide (Gd2O3)           LD50 > 5 g/kg  ( Rat )                              -              LC50 > 5.04 mg/L  ( Rat ) 4 h

______________________________________________________________________________________________

Page 4 / 8

Page 5

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

Toxicologically Synergistic       No information available

Products

(b) skin corrosion/irritation;       No data available

(c) serious eye damage/irritation;   Category 2

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;

The table below indicates whether each agency has listed any ingredient as a carcinogen

Component       CAS No          IARC          NTP          ACGIH        OSHA           Mexico

Gadolinium oxide       12064-62-9         Not listed          Not listed          Not listed          Not listed          Not listed

(Gd2O3)

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

Target Organs              No information available.

(j) aspiration hazard;               Not applicable

Solid

Other Adverse Effects            The toxicological properties have not been fully investigated.

Symptoms  / effects,both acute and No information available.

delayed

Other Adverse Effects            The toxicological properties have not been fully investigated.

Endocrine Disrupting Properties    This product does not contain any known or suspected endocrine disruptors.

12. Ecological information

Ecotoxicity

.

Persistence and Degradability       Insoluble in water

Bioaccumulation/ Accumulation   No information available.

Mobility                                   Is not likely mobile in the environment due its low water solubility.

______________________________________________________________________________________________

Page 5 / 8

Page 6

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

13. Disposal considerations

Waste Disposal Methods           Chemical waste generators must determine whether a discarded chemical is classified as a

hazardous waste. Chemical waste generators must also consult local, regional, and

national hazardous waste regulations to ensure complete and accurate classification.

14. Transport information

DOT                             Not regulated

TDG                            Not regulated

IATA                             Not regulated

IMDG/IMO                        Not regulated

15. Regulatory Information

United States of America Inventory

Component           CAS No          TSCA        TSCA Inventory notification -    TSCA - EPA Regulatory

Active-Inactive                   Flags

Gadolinium oxide (Gd2O3)        12064-62-9          X                   ACTIVE                                       -

Legend:

TSCA US EPA (TSCA) - Toxic Substances Control Act, (40 CFR Part 710)

X - Listed

'-' - Not Listed

TSCA - Per 40 CFR 751, Regulation of Certain Chemical     Not applicable

Substances & Mixtures, Under TSCA Section 6(h) (PBT)

TSCA 12(b) - Notices of Export                             Not applicable

International Inventories

Canada (DSL/NDSL), Europe (EINECS/ELINCS/NLP), Philippines (PICCS), Japan (ENCS), Japan (ISHL), Australia (AICS), China (IECSC), Korea

(KECL).

Component          CAS No     DSL   NDSL   EINECS  PICCS  ENCS    ISHL    AICS   IECSC   KECL

Gadolinium oxide (Gd2O3)      12064-62-9     X            -     235-060-9   X      X      X      X      X     KE-10558

KECL - NIER number or KE number (http://ncis.nier.go.kr/en/main.do)

U.S. Federal Regulations

SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product does not contain any

chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372

SARA 311/312 Hazard Categories

Should this product meet EPCRA 311/312 Tier reporting criteria at 40 CFR 370, refer to Section 2 of this SDS for appropriate

classifications.

CWA (Clean Water Act)            Not applicable

Clean Air Act                     Not applicable

OSHA - Occupational Safety and     Not applicable

Health Administration

CERCLA

This material, as supplied, does not contain any substances regulated as hazardous substances under the Comprehensive

______________________________________________________________________________________________

Page 6 / 8

Page 7

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund Amendments and

Reauthorization Act (SARA) (40 CFR 355). There may be specific reporting requirements at the local, regional, or state level

pertaining to releases of this material.

California Proposition 65            This product does not contain any Proposition 65 chemicals.

U.S. State Right-to-Know           Not applicable

Regulations

U.S. Department of Transportation

Reportable Quantity (RQ):        N

DOT Marine Pollutant           N

DOT Severe Marine Pollutant      N

U.S. Department of Homeland       This product does not contain any DHS chemicals.

Security

Other International Regulations

Mexico - Grade                No information available

Authorisation/Restrictions according to EU REACH         Not applicable

Component              CAS No        REACH (1907/2006) -   REACH (1907/2006) -   REACH Regulation (EC

Annex XIV - Substances Annex XVII - Restrictions  1907/2006) article 59  -

Subject to Authorization  on Certain Dangerous      Candidate List of

Substances      Substances of Very High

Concern (SVHC)

Gadolinium oxide (Gd2O3)           12064-62-9                             -                                    -                                    -

Safety, health and environmental regulations/legislation specific for the substance or mixture

Component          CAS No        OECD HPV       Persistent Organic   Ozone Depletion       Restriction of

Pollutant             Potential          Hazardous

Substances (RoHS)

Gadolinium oxide (Gd2O3)       12064-62-9         Not applicable        Not applicable        Not applicable        Not applicable

Contains component(s) that meet a 'definition' of per & poly fluoroalkyl substance (PFAS)?

Not applicable

Other International Regulations

Component          CAS No       Seveso III Directive  Seveso III Directive      Rotterdam       Basel Convention

(2012/18/EC) -        (2012/18/EC) -     Convention (PIC)   (Hazardous Waste)

Qualifying Quantities Qualifying Quantities

for Major Accident    for Safety Report

Notification        Requirements

Gadolinium oxide (Gd2O3)       12064-62-9         Not applicable        Not applicable        Not applicable        Not applicable

16. Other Information

Prepared By                       Product stewardship (Regulatory Affairs)

Thermo Fisher Scientific

email - begel.sdsdesk@thermofisher.com

______________________________________________________________________________________________

Page 7 / 8

Page 8

Gadolinium(III) oxide                                                                 Revision Date 19-Dec-2025

______________________________________________________________________________________________

Creation Date                     26-Sep-2009

Revision Date                     19-Dec-2025

Print Date                        19-Dec-2025

Revision Summary              UpdatedtotheU.S.DepartmentofLabor’sOccupationalSafetyandHealthAdministration

(OSHA) which published its Final Rule in the Federal Register revising the Hazard

Communication Standard (HCS/HazCom), 29 CFR 1910.1200 (2024) (HCS §1910.1200,

2024), May 20, 2024, effective July 19, 2024.

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of SDS

______________________________________________________________________________________________

Page 8 / 8