Gadolinium Nitrate Hexahydrate, 99.5% 070-02682 SDS

Publisher: FUJIFILM Wako Pure Chemical Corporation

Source: https://resds.com/document/gadolinium-nitrate-hexahydrate-99-5-sds-fujifilm-wako-pure-chemical-en-7666a8b345

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

According to JIS Z 7253:2019

Revision date 04-Feb-2023

Revision Number  2.04

Section 1: PRODUCT AND COMPANY IDENTIFICATION

Product Name            Gadolinium Nitrate Hexahydrate, 99.5%

Product Code           070-02682

Manufacturer                   FUJIFILM Wako Pure Chemical Corporation

1-2 Doshomachi 3-Chome

Chuo-ku, Osaka 540-8605, Japan

Phone: +81-6-6203-3741

Fax: +81-6-6203-5964

Supplier                       FUJIFILM Wako Pure Chemical Corporation

1-2 Doshomachi 3-Chome, Chuo-ku, Osaka 540-8605, Japan

Phone: +81-6-6203-3741

Fax: +81-6-6203-2029

Emergency telephone number     +81-6-6203-3741 / +81-3-3270-8571

Recommended uses and          For research use only

restrictions on use

Section 2: HAZARDS IDENTIFICATION

GHS classification

Classification of the substance or mixture

Oxidizing solids                                                              Category 2

Skin corrosion/irritation                                                       Category 2

Serious eye damage/eye irritation                                              Category 2A

Pictograms

Signal word                    Danger

Hazard statements

H272 - May intensify fire; oxidizer

H315 - Causes skin irritation

H319 - Causes serious eye irritation

Precautionary statements-(Prevention)

• Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking

• Keep/Store away from clothing/ combustible materials

• Wear protective gloves/protective clothing/eye protection/face protection

• Wash face, hands and any exposed skin thoroughly after handling

• Take any precaution to avoid mixing with combustibles

Precautionary statements-(Response)

• IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue

rinsing

• If eye irritation persists: Get medical advice/attention

• IF ON SKIN: Wash with plenty of soap and water

• If skin irritation occurs: Get medical advice/attention

__________________________________________________________________________________________

Page 1 / 6

W01W0107-0268 JGHEEN                                         Gadolinium Nitrate Hexahydrate, 99.5%

Page 2

Revision date 04-Feb-2023

__________________________________________________________________________________________

• Take off contaminated clothing and wash before reuse

Precautionary statements-(Storage)

• Not applicable

Precautionary statements-(Disposal)

• Dispose of contents/container to an approved waste disposal plant

Others

Other hazards                    Not available

Section 3: COMPOSITION/INFORMATION ON INGREDIENTS

Single Substance or Mixture      Substance

Formula                      Gd(NO3)3·6H2O

Chemical Name         Weight-%       Molecular weight      ENCS          ISHL No.       CAS RN

Gadolinium nitrate          99.5               451.36            1-700                   *            19598-90-4

hexahydrate              ( subtracting

method )

Note on ISHL No.:                     * in the table means announced chemical substances.

Impurities and/or Additives：     Not applicable

Section 4: FIRST AID MEASURES

Inhalation

Remove to fresh air. If symptoms persist, call a physician.

Skin contact

Wash off immediately with soap and plenty of water. If symptoms persist, call a physician.

Eye contact

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue

rinsing. Immediate medical attention is required.

Ingestion

Rinse mouth. Never give anything by mouth to an unconscious person. Call a physician or poison control center

immediately. Do not induce vomiting without medical advice.

Protection of first-aiders

Use personal protective equipment as required.

Section 5: FIRE FIGHTING MEASURES

Suitable extinguishing media

Flood with water

Unsuitable extinguishing media

Powder, Carbondioxide

Specific hazards arising from the chemical product

Thermal decomposition can lead to release of irritating and toxic gases and vapors.

Special extinguishing method

No information available

Special protective actions for

fire-fighters

Use personal protective equipment as required. Firefighters should wear self-contained breathing apparatus and full

firefighting turnout gear.

Section 6: ACCIDENTAL RELEASE MEASURES

Personal precautions, protective equipment and emergency procedures

For indoor, provide adequate ventilation process until the end of working. Deny unnecessary entry other than the people

__________________________________________________________________________________________

Page 2 / 6

W01W0107-0268 JGHEEN                                         Gadolinium Nitrate Hexahydrate, 99.5%

Page 3

Revision date 04-Feb-2023

__________________________________________________________________________________________

involved by, for example, using a rope. While working, wear appropriate protective equipments to avoid adhering it on skin,

or inhaling the gas. Work from windward, and retract the people downwind.

Environmental precautions

To be careful not discharged to the environment without being properly handled waste water contaminated.

Methods and materials for contaminent and methods and materials for cleaning up

Sweep up and gather scattered particles, and collect it in an empty airtight container.

Recoverly, neutralization

No information available

Secondary disaster prevention measures

Clean contaminated objects and areas thoroughly observing environmental regulations.

Section 7: HANDLING AND STORAGE

Handling

Technical measures

Flammable. Do not give shock. Avoid contact with reducing agents and combustible materials. Avoid contact with organic

substance Use with local exhaust ventilation.

Precautions

Do not rough handling containers, such as upsetting, falling, giving a shock, and dragging Prevent leakage, overflow, and

scattering. Not to generate steam and dust in vain. Seal the container after use. After handling, wash hands and face, and

then gargle In places other than those specified, should not be smoking or eating and drinking Should not be brought

contaminated protective equipment and gloves to rest stops Deny unnecessary entry of non-emergency personnel to the

handling area

Safety handling precautions

Avoid contact with skin, eyes or clothing. Use personal protective equipment as required.

Storage

Safe storage conditions

Storage conditions             Store away from sunlight in a cool (2-10 °C) well-ventilated dry place.

Safe packaging material       Glass

Incompatible substances          Organic substance, Combustible materials, Reducing agent

Section 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

Engineering controls

In case of indoor workplace, seal the source or use a local exhaust system. Provide the safety shower facility, and hand-

and eye-wash facility. And display their position clearly.

Exposure limits                    This product, as supplied, does not contain any hazardous materials with occupational

exposure limits established by the region specific regulatory bodies.

Personal protective equipment

Respiratory protection         Dust mask

Hand protection                 Protection gloves

Eye protection                   protective eyeglasses or chemical safety goggles

Skin and body　protection     Long-sleeved work clothes

General hygiene considerations

Handle in accordance with good industrial hygiene and safety practice.

Section 9: PHYSICAL AND CHEMICAL PROPERTIES

Form

Color                                               White - slightly yellow

Appearance                                                 crystals - crystalline powder or mass

Odor                                              no data available

Melting point/freezing point                           no data available

Boiling point, initial boiling point and boiling range       no data available

Flammability                                        no data available

Evaporation rate:                                    no data available

Flammability (solid, gas):                             no data available

__________________________________________________________________________________________

Page 3 / 6

W01W0107-0268 JGHEEN                                         Gadolinium Nitrate Hexahydrate, 99.5%

Page 4

Revision date 04-Feb-2023

__________________________________________________________________________________________

Upper/lower flammability or

explosive limits

Upper：                                        no data available

Lower：                                        no data available

Flash point                                         no data available

Auto-ignition temperature:                            no data available

Decomposition temperature:                          no data available

pH                                                no data available

Viscosity (coefficient of viscosity)                      no data available

Dynamic viscosity                                   no data available

Solubilities                                               water , Ethanol : soluble .

n-Octanol/water partition coefficient:(log Pow)           no data available

Vapour pressure                                    no data available

Specific Gravity / Relative density                      no data available

Vapour density                                      no data available

Particle characteristics                               no data available

Section 10: STABILITY AND REACTIVITY

Stability

Reactivity                   no data available

Chemical stability              Stable under recommended storage conditions.

Hazardous reactions

None under normal processing

Conditions to avoid

Extremes of temperature and direct sunlight

Incompatible materials

Organic substance, Combustible materials, Reducing agent

Hazardous decomposition products

Nitrogen oxides (NOx), Metal oxides

Section 11: TOXICOLOGICAL INFORMATION

Acute toxicity

Chemical Name                Oral LD50               Dermal LD50               Inhalation LC50

Gadolinium nitrate            >5 gm/kg(Rat)                  N/A                      N/A

hexahydrate

Skin irritation/corrosion                              no data available

Serious eye damage/ irritation                         no data available

Respiratory or skin sensitization                       no data available

Reproductive cell mutagenicity                        no data available

Carcinogenicity

Chemical Name                NTP          IARC          ACGIH    JSOH (Japan)

Gadolinium nitrate hexahydrate                           Group 2A

19598-90-4

Reproductive toxicity                                no data available

STOT-single exposure                               no data available

STOT-repeated exposure                             no data available

Aspiration hazard                                   no data available

Section 12: ECOLOGICAL INFORMATION

Ecotoxicity                  No information available

__________________________________________________________________________________________

Page 4 / 6

W01W0107-0268 JGHEEN                                         Gadolinium Nitrate Hexahydrate, 99.5%

Page 5

Revision date 04-Feb-2023

__________________________________________________________________________________________

Other data                     no data available

Persistence and degradability    No information available

Bioaccumulative potential       No information available

Mobility in soil                No information available

Hazard to the ozone layer       No information available

Section 13: DISPOSAL CONSIDERATIONS

Waste from residues

Disposal should be in accordance with applicable regional, national and local laws and regulations.

Contaminated container and contaminated packaging

Disposal should be in accordance with applicable regional, national and local laws and regulations.

Section 14: TRANSPORT INFORMATION

ADR/RID

UN number                UN1477

Proper shipping name:          Nitrates, inorganic, n.o.s. (Gadolinium nitrate hexahydrate)

UN classfication                5.1

Subsidiary hazard class

Packing group                              II

Marine pollutant              Not applicable

IMDG

UN number                UN1477

Proper shipping name:          Nitrates, inorganic, n.o.s. (Gadolinium nitrate hexahydrate)

UN classfication                5.1

Subsidiary hazard class

Packing group                              II

Marine pollutant (Sea)         Not applicable

Transport in bulk according to No information available

Annex II of MARPOL 73/78 and

the IBC Code

IATA

UN number                UN1477

Proper shipping name:          Nitrates, inorganic, n.o.s. (Gadolinium nitrate hexahydrate)

UN classfication                5.1

Subsidiary hazard class

Packing group                              II

Environmentally Hazardous    Not applicable

Substance

Section 15: REGULATORY INFORMATION

International Inventories

EINECS/ELINCS                      -

TSCA                                     -

Japanese regulations

Fire Service Act               Category I, nitrates, dangerous grade 3

Poisonous and Deleterious    Not applicable

Substances Control Law

Industrial Safety and Health Act Dangerous Substances - Oxidizing Substance (Enforcement Order Attached Table 1

Item 3)

Regulations for the carriage    Oxidizing Agents - Oxidizing Agents (Ordinance Art.3, Ministry of Transportation

and storage of dangerous      Ordinance Regarding Transport by Ship and Storage, Attached Table 1)

goods in ship

Civil Aeronautics Law          Oxidizing Agents - Oxidizing Agents (Ordinance Art.194, MITL Nortification for Air

__________________________________________________________________________________________

Page 5 / 6

W01W0107-0268 JGHEEN                                         Gadolinium Nitrate Hexahydrate, 99.5%

Page 6

Revision date 04-Feb-2023

__________________________________________________________________________________________

Transportation of Explosives etc., Attached Table 1)

Pollutant Release and Transfer Not applicable

Register Law

(~2023.3.31)

Pollutant Release and Transfer     Not applicable

Register Law

(2023/4/1~)

Water Pollution Control Act    Harmful Substances (Law Art.2, Enforcement Order Art.2, Ordinace Designating

Wastewater Standards Art.1)

Export Trade Control Order    Not applicable

Section 16: OTHER INFORMATION

Key literature references and      NITE: National Institute of Technology and Evaluation (JAPAN)

sources for data etc.                http://www.safe.nite.go.jp/japan/db.html

IATA dangerous Goods Regulations

RTECS:Registry of Toxic Effects of Chemical Substances

Japan Industrial Safety and Health Association GHS Model SDS

Dictionary of Synthetic Oraganic Chemistry , SSOCJ, Koudansha Scientific Co.Ltd.

Chemical Dictionary, Kyouritsu Publishing Co., Ltd.

etc

Disclaimer

This SDS is according to JIS Z 7253: 2019. The information provided in this Safety Data Sheet is correct to the best of our

knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe

handling, and is not to be considered a warranty or quality specification. The information relates only to the specific material

designated and may not be valid for such material used in combination with any other materials or in any process, unless

specified in the text.

GHS Classification is according to JIS Z7252(2019). *JIS: Japanese Industrial Standards

End of Safety Data Sheet

__________________________________________________________________________________________

Page 6 / 6

W01W0107-0268 JGHEEN                                         Gadolinium Nitrate Hexahydrate, 99.5%