Gadolinium Oxide SDS

Publisher: Angstrom Sciences

Source: https://resds.com/document/gadolinium-oxide-sds-angstrom-en-3dd97460ff

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

GADOLINIUM OXIDE

SAFETY DATA SHEET

DATE OF LAST REVISION: 07/09/15

Section 1:  Identification

Product Name: Gadolinium Oxide

CAS Number: 12064-62-9 / EC Number:  235-060-9

Company:    Angstrom Sciences, Inc.

40 South Linden Street

Duquesne, PA 15110

For more information call: 412-469-8466

(Monday - Friday 9:00 AM -5:00 PM EST)

Section 2: HAZARD IDENTIFICATION

Signal Word: None

Hazard Statements:          None

Precautionary Statements:    None

HMIS Health Ratings (0-4):

Health:                1

Flammability:            0

Physical:               0

Section 3: Composition/Information on Ingredients

Chemical characterization: Ceramic

Additional Names:     None

CAS# Description: 12064-62-9

Percentage: 0-100 wt%

EC number: 235-060-9

Section 4: FIRST AID MEASURES

General Treatment: Seek medical attention if symptoms persist.

Special Treatment: None

Important Symptoms: None

Eye Contact: Flush eyes with water, blinking often for several minutes. Remove contact lenses if

present and easy to do. Continue rinsing.

Skin Contact: Wash affected area with mild soap and water. Remove any contaminated clothing.

Inhalation:Remove to fresh air.   If required, give artificial respiration.  Seek medical advice.

Ingestion: Seek medical attention

Page 2

GADOLINIUM OXIDE

SAFETY DATA SHEET

Section 5: FIRE FIGHTING MEASURES

Suitable extinguishing Media: Do not use water for metal fires - special extinguishing powder.

Flammability: Non-Flammable

Special Fire Fighting Procedure: Use full-face, self contained breathing apparatus with full protective

clothing to prevent contact with skin and eyes. See section 10 for decomposition products.

Section 6: ACCIDENTAL RELEASE MEASURES

Personal Precautions: Wear appropriate respiratory and protective equipment specified in special

protection information.  Isolate spill area and provide ventilation. Vacuum up spill using a high

efficiency particulate absolute (HEPA) air filter and place in a closed container for disposal. Take

care not to raise dust.

Environmental Precautions:  Isolate runoff to prevent environmental pollution.

Section 7: HANDLING AND STORAGE

Handling Precautions for safe handling:  Handle under dry protective gas. Wash thoroughly after

handling.

Conditions for safe storage, including any incompatibilities Store in a cool dry place in a tightly sealed

container. Store apart from materials and conditions listed in Section 10.

Work/Hygienic Maintenance: Do not use tobacco or food in work area. Wash thoroughly before eating

and smoking. Do not blow dust off clothing or skin with compressed air.

Ventilation: Provide sufficient ventilation to maintain concentration at or below threshold limit.

Section 8: EXPOSURE CONTROL/PERSONAL PROTECTION

Permissible Exposure Limits:   N/A

Threshold Limit Value:       N/A

Special Equipment:           None

Respiratory Protection:        Dust respirator

Protective Gloves:            Rubber Gloves

Eye Protection:                  Safety glasses or goggles, Full face protection

Body Protection:                Protective work clothing. Wear close-toed shoes and long sleeves/pants.

Section 9: PHYSICAL AND CHEMICAL PROPERTIES

Appearance:

Form:                   Powder, Granules, Pellets, Sputtering target, Custom parts

Color:                   White

Odor:                     Odorless

Water Solubility:            Insoluble

Page 3

GADOLINIUM OXIDE

SAFETY DATA SHEET

Boiling Point:          N/A

Melting Point:            2330C

Flash Point:           N/A

Autoignition Temperature: N/A

Density:                7.407 g/cc

Molecular Weight:       362.5 g/mol

Section 10: STABILITY AND REACTIVITY

Reactivity:                                           Oxidizing agents

Chemical Stability:                               Stable under recommended storage conditions

Incompatible Conditions:                   None

Hazardous Decomposition Products:           Metal oxide fume

Section 11: TOXICOLOGICAL INFORMATION

Potential Health Effects:

Eyes:  May cause irritation

Skin:  May cause irritation

Ingestion: May cause irritation

Inhalation: May cause irritation

Chronic: N/A

Signs & Symptoms:              N/A

Aggravated Medical Conditions:      N/A

Median Lethal Dose:             N/A

Carcinogen:                   N/A

Section 12: ECOLOGICAL INFORMATION

Aquatic Toxicity: Low

Persistence Bioaccumulation Toxicity: No

Very Persistent, Very Bioaccumulative: No

Notes: N/A

Section 13: DISPOSAL CONSIDERATIONS

Dispose of in accordance with local, state, national, and international regulations

Page 4

GADOLINIUM OXIDE

SAFETY DATA SHEET

Section 14: TRANSPORT INFORMATION

Hazardous:         Not hazardous for transportation

Hazard Class:      N/A

Packing Group:     N/A

UN Number:      N/A

Proper Shipping Name: N/A

Section 15: REGULATORY INFORMATION

Sec 302 Extremely Hazardous:       No

Sec 304 Reportable Quantities:      N/A

Sec 313 Toxic Chemicals:           No

Section 16: OTHER INFORMATION

This safety data sheet should be used in conjunction with technical sheets.  It does not replace them. The

information given is based on our knowledge of this product, at the time of publication.  It is given in good

faith. The attention of the user is drawn to the possible risks incurred by using the product for any other

purpose other than that for which it was intended. This does not in any way excuse the user from knowing

and applying all the regulations governing his activity.  It is the sole responsibility of the user to take all

precautions required in handling the product. The aim of the mandatory regulations mentioned is to help the

user to fulfill his obligations regarding the use of hazardous products.