Holmium oxide in perchloric acid solution NIST SRM 2034 SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/holmium-oxide-solution-wavelength-standard-240-nm-to-650-nm-nist-srm-2034-en-b3e4b62882

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

12 April 2016

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   2034

SRM Name:    Holmium Oxide Solution Wavelength Standard (240 nm to 650 nm)

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is a certified transfer standard intended for the verification and

calibration of the wavelength scale of ultraviolet and visible absorption spectrophotometers having nominal

spectral bandwidths not exceeding 3 nm. A unit of SRM 2034 consists of an aqueous solution containing 4 %

(mass fraction) holmium oxide (Ho2O3) in 10 % (volume fraction) perchloric acid (HClO4).  The solution is

contained in a flame-sealed, nonfluorescent, fused-silica cuvette of optical quality (parallel to ≈ 0.9 mrad and flat

to ≈ 1 µm).

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                       +1-703-527-3887    (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Classification

Physical Hazard:    Not classified.

Health Hazard:      Skin Corrosion/Irritation            Category 1B

Serious Eye Damage/Irritation       Category 1

Label Elements

Symbol

Signal Word

DANGER

Hazard Statement(s)

H314               Causes severe skin burns and eye damage.

Precautionary Statement(s)

P260            Do not breathe fumes, mists, vapors, spray.

P264             Wash hands thoroughly after handling.

P270            Do not eat, drink or smoke when using this product.

P280              Wear protective gloves, protective clothing, eye protection.

P301 + P330 + P331   If swallowed: Rinse mouth. Do NOT induce vomiting.

P303 + P361 + P353   If on skin (or hair): Remove immediately all contaminated clothing.  Rinse skin with

water.

P304 + P340             If inhaled: Remove person to fresh air and keep comfortable for breathing.

P305 + P351 + P338   If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if

present and easy to do. Continue rinsing.

SRM 2034                                                                             Page 1 of 6

Page 2

P310                Immediately call a doctor.

P363             Wash contaminated clothing before reuse.

P405                  Store locked up.

P501                Dispose of contents and container according to local regulations.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Perchloric acid/holmium oxide solution

Other Designations:

Perchloric acid (dioxonium perchlorate; perchloric acid solution; perchloric acid; HClO4)

Holmium oxide (Ho2O3; diholmium trioxide; holmium (3+) oxide; holmium sesquioxide; holmium trioxide)

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the Certificate of

Analysis.

Hazardous Component(s)       CAS Number    EC Number     Nominal Mass Concentration

(EINECS)               (%)

Perchloric acid                        7601-90-3         231-512-4                  7

Holmium oxide                      12055-62-8         234-015-3                  4

Non-Hazardous Component(s)

Water                               7732-18-5         231-791-2                 89

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:  If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and

shoes. Get immediate medical attention. Thoroughly clean and dry contaminated clothing before reuse. Destroy

contaminated shoes.

Eye Contact: Immediately flush eyes, including under the eyelids with copious amounts of water for at least

15 minutes. Seek immediate medical attention.

Ingestion: Contact a poison control center immediately for instructions. Do not induce vomiting. Give water to

rinse out mouth. Never give liquids to a person with reduced awareness or becoming unconscious.  If vomiting

occurs, keep head lower than hips to prevent aspiration.  If not breathing, give artificial respiration by qualified

personnel. Seek immediate medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Acid burns to skin, eyes, and lungs.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek immediate medical attention.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing media appropriate to the surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: Thermal decomposition will form hydrogen chloride and oxides of

chlorinated compounds.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 3            Fire = 0          Reactivity = 0

SRM 2034                                                                             Page 2 of 6

Page 3

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures:  Immediately contact emergency

personnel. Keep unnecessary personnel away. Use suitable protective equipment; see Section 8, “Exposure Controls

and Personal Protection”.

Methods and Materials for Containment and Clean up: Do not touch spilled material. Notify safety personnel

of spills. Absorb with sand or other non-combustible material. Collect spilled material in appropriate container for

disposal. Isolate hazard area and deny entry.

7. HANDLING AND STORAGE

Safe Handling Precautions: See Section 8, “Exposure Controls and Personal Protection”. Handle glass ampoules

with care.

Storage:   Store and handle in accordance with  all current regulations and standards.  Keep separated from

incompatible substances (See Section 10, “Stability and Reactivity”).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits: No occupational exposure limits established for the main components (perchloric acid; holmium

oxide) or the solution.

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:  If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eyewash station should be

readily available near areas of use.

Skin and Body Protection:  Personal protective equipment for the body should be selected based on the task

being performed and the risks involved and should be approved by a specialist before handling this product.

Chemical-resistant gloves should be worn at all times when handling chemicals.

9. PHYSICAL AND CHEMICAL PROPERTIES

NOTE: The physical and chemical data provided are for the pure components. No physical or chemical data are

available for this solution. The actual behavior of the solution may differ from the individual components.

Descriptive Properties:                           Perchloric Acid          Holmium Oxide

(7 % mass fraction)          (4 % mass fraction)

Appearance (physical state, color, etc.):           clear, colorless solution        off-white to orange solid;

hygroscopic

Molecular Formula:                       HClO4                   Ho2O3

Molar Mass (g/mol):                          100.46                     377.86

Odor:                                            odorless                      odorless

Odor threshold:                                 not available                  not available

pH:                                                acidic                        not available

Evaporation rate:                               not available                  not available

Melting point/freezing point:                 –4 °C (24.8 °F)             2360 °C (4280 °F)

Relative Density (g/L) as specific gravity         1.764 at 22 °C                8.4

(water = 1):

Vapor Pressure (mmHg):                        not available                  not available

Vapor Density (air = 1):                        3.46                          not available

Viscosity (cP):                                   not available                  not available

Solubility(ies):                                    soluble in water               insoluble in water

Partition coefficient (n-octanol/water):          not available                  not available

SRM 2034                                                                             Page 3 of 6

Page 4

Thermal Stability Properties:                       Perchloric Acid          Holmium Oxide

(7 % mass fraction)          (4 % mass fraction)

Autoignition Temperature (ºC):                 not available                  not available

Thermal Decomposition (ºC):                    not available                  not available

Initial boiling point and boiling range:         39 °C at 56 mmHg            not available

Explosive Limits, LEL (Volume %):             not available                  not available

Explosive Limits, UEL (Volume %):             not available                  not available

Flash Point (ºC):                                not available                  not available

Flammability (solid, gas):                        not applicable                not available

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:   X       Stable              Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Contact with combustible or incompatible materials.

Incompatible Materials: Incompatible with acids, combustible materials, halo carbons, bases, oxidizing materials,

metals, halogens, metal salts, metal oxides.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition:  Thermal decomposition will form hydrogen chloride and oxides of chlorinated

compounds.

Hazardous Polymerization:          Will Occur    X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Burning pain and severe skin

corrosion and eye damage.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation: Inhalation of perchloric acid may damage the mucous membranes and upper respiratory tract. Short

term exposure may cause irritation, headache, dizziness, and nausea. Long term exposure to acid fumes may

cause damage  to  teeth,  bronchial  irritation,  chronic  cough,  bronchial pneumonia, and  gastrointestinal

disturbances.

Skin Contact:  Perchloric acid may cause skin burns and irritation.  Severity of the damage depends on the

concentration and duration of exposure. Effects of acid burns may be delayed.

Eye Contact: Perchloric acid may cause eye irritation or corneal burns. Severity of the damage depends on the

concentration and duration of exposure.

Ingestion: Ingestion of this material is unlikely under normal conditions of use.  If ingested, perchloric acid can

cause burns and damage to the gastrointestinal tract.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified.

Perchloric acid, Rat, Oral LD50: 1100 mg/kg;

Skin Corrosion/Irritation: This SRM contains >1 % of perchloric acid and it is classified as Category 1B.

Serious Eye Damage/Irritation: This SRM contains >1 % of perchloric acid and it is classified as Category 1.

Respiratory Sensitization: No data available; not classified.

Skin Sensitization: No data available; not classified.

Germ Cell Mutagenicity: No data available; not classified.

SRM 2034                                                                             Page 4 of 6

Page 5

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen               Yes       X   No

Perchloric acid and holmium oxide are not classified as either a carcinogen or potential carcinogen by OSHA,

IARC, or NTP.

Reproductive Toxicity: No data available; not classified.

Specific Target Organ Toxicity, Single Exposure: No data available; not classified.

Specific Target Organ Toxicity, Repeated Exposure: No data available; not classified.

Aspiration Hazard: No data available; not classified.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations. Perchloric

acid is subject to disposal regulations: U.S. EPA 40 CFR 262: D001 and D003.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: UN1760, Corrosive liquid, n.o.s. (contains perchloric acid), Hazard Class 8, Packing Group

II, Excepted Quantities E2.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4): Not regulated.

SARA Title III Section 302 (40 CFR 355.30): Not regulated.

SARA Title III Section 304 (40 CFR 355.40): Not regulated.

SARA Title III Section 313 (40 CFR 372.65): Not regulated.

OSHA Process Safety (29 CFR 1910.119): Perchloric acid, 5000 lbs TQ (≥60 % by weight)

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:       Yes

CHRONIC HEALTH:    No

FIRE:               No

REACTIVE:           No

PRESSURE:           No

State Regulations:

California Proposition 65: Not listed.

U.S. TSCA Inventory: Perchloric acid and holmium oxide are listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations: WHMIS Information is not provided for this material.

SRM 2034                                                                             Page 5 of 6

Page 6

16. OTHER INFORMATION

Issue Date: 12 April 2016

Sources:   ChemAdvisor, Inc., SDS Perchloric Acid, 09 December 2015.

ChemAdvisor, Inc., SDS Holmium Oxide, 09 December 2015.

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NRC          Nuclear Regulatory Commission

Hygienists

ALI          Annual Limit on Intake                     NTP           National Toxicology Program

CAS         Chemical Abstracts Service                OSHA         Occupational Safety and Health Administration

CERCLA     Comprehensive Environmental Response,         PEL           Permissible Exposure Limit

Compensation, and Liability Act

CFR         Code of Federal Regulations                RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                REL         Recommended Exposure Limit

EC50          Effective Concentration, 50%             RM           Reference Material

EINECS      European Inventory of Existing Commercial      RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transport Agency            SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

LC50          Lethal Concentration, 50 %                   STEL          Short Term Exposure Limit

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NFPA         National Fire Protection Association          TWA        Time Weighted Average

NIOSH        National Institute for Occupational Safety and Health  UEL         Upper Explosive Limit

NIST          National Institute of Standards and Technology     WHMIS      Workplace Hazardous Materials Information System

n.o.s.         Not Otherwise Specified

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 2034                                                                             Page 6 of 6