Holmium, Powder 1966 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/holmium-powder-prochem-en-1db1c65ce6

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Holmium, Powder

PRODUCT NUMBER:  1966

CAS NUMBER:        7440-60-0

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228                  Flammable solid.

H260                        In contact with water released flammable gases, which may ignite spontaneously.

Precautionary Phrases

P210                 Keep away from heat/sparks/open flames/hot surfaces. No smoking.

P231+P232             Handle under inert gas. Protect from moisture.

P280                 Wear protective gloves/protective clothing/eye protection/face protection.

P370+P378                 In case of fire: Use for extinction: Special powder for metal fires.

P402+P404               Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 1              Fire: 3                  Reactivity Hazard: 2

NFPA RATING:

Health: 1            Flammability: 3         Reactivity Hazard: 2

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: Inhalation of any fine dust may be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: No specific information is available on the physiological effects of ingestion.

HOLMIUM POWDER                                                                    PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Ho

Molecular Weight: 164.93

CHEMICAL NAME                                           CAS#                               %

Holmium Powder                                                        7440-60-0                                               100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with plenty of water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Special powder for metal fires. Do not use water.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Holmium oxide

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation. Avoid breathing dust. Keep away from sources of ignition.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Avoid contact

with skin and eyes.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from water, air, moisture, oxidizing

agents, and acid chlorides. Store under inert gas. Protect from humidity and water.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: This product does not contain any quantities of materials with critical values that have to be monitored.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

HOLMIUM POWDER                                                                    PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Grey to silver powder

ODOR: None

MOLECULAR WEIGHT: 164.93

BOILING POINT: 2695° C

MELTING POINT: 1474° C

SPECIFIC GRAVITY: 8.795 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with water releases flammable gas.

INCOMPATIBILITY: Strong oxidizers, water, alcohols, oxygen, halogens, halocarbons, and mineral acids.

DECOMPOSITION PRODUCTS: Acids, air, oxidizing agents, acid chlorides, water, and moisture.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: Not classified as a human carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:   UN3089                                              IATA:  UN3089

Metal powders, flammable, n.o.s.                                    Metal powder, flammable, n.o.s.

(Holmium powder)                                             (Holmium powder)

CLASS 4.1                                          CLASS 4.1

PG III                                         PG III

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

HOLMIUM POWDER                                                                    PAGE 3

Page 4

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 04/15

SDS DEPT,

HOLMIUM POWDER                                                                    PAGE 4