Lanthanum acetate, hydrate 2104 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/lanthanum-acetate-hydrate-prochem-en-7bbb1b363d

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1  PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Lanthanum acetate, hydrate

PRODUCT NO:       2104

CAS NO:             100587-90-4

SYNONYMS:         Lanthanum (III) acetate, hydrate, Lanthanum triacetate

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2  HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not a dangerous substance according to the GHS

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: Causes irritation of the eyes.

SKIN CONTACT: May cause irritation and redness.

INHALATION: May cause irritation to nose, throat, and respiratory tract.

INGESTION: No Data

SECTION 3   COMPOSITION / INFORMATION ON INGREDIENTS

Formula:   La(COOH3)3  X H2O

Molecular Weight: 316.05 (anhy.)

CHEMICAL NAME                   CAS#              %                       ACGIH (TWA)            OSHA (PEL)

Lanthanum acetate, hydrate                100587-90-4                   100                          Not determined                 Not determined

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water (lukewarm if possible) for several minutes. Assure

flushing under eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

LANTHANUM ACETATE, HYDRATE                                                                 PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES (Cont.)

SKIN EXPOSURE: Wash affected area with soap and copious amounts of water. Remove contaminated clothes if necessary. Seek

medical assistance if irritation, burns, or redness persist.

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in

breathing, coughing, wheezing, or pain. In such cases seek immediate medical assistance. Give oxygen if deemed necessary.

If not breathing administer CPR. Get emergency care immediately.

INGESTION: No specific data available. Seek medical assistance immediately. Keep the victim calm.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Water spray or measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: No special fire-fighting procedures required. Use normal conditions which include

wearing NIOSH/MSHA approved self-contained breathing apparatus, flam and chemical resistant clothing, hats, boots and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Carbon monoxide, carbon dioxide, toxic metal oxide fume.

UNUSUAL FIRE OR EXPLOSION HAZARDS: None known.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Avoid dust formation. Avoid breathing vapors, mist, or gas.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Wear appropriate respiratory and protective equipment to

avoid exposure with material, sweep up avoiding raising dust and place in a well-marked container for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Provide appropriate exhaust ventilation at places where dust is formed. Normal measures for

preventive fire protection.

CONDITIONS FOR SAFE STORAGE: Store in a tightly sealed container. Store in cool, dry conditions. Store away from oxidizing

agents.  Irritant. Do not breathe dust. Hygroscopic. Protect from humidity and keep away from water. Avoid contact with skin and

eyes.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, or safety goggles, face shield when

handling a chemicals substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION:  If possible, handle the material in an efficient fume hood.

RESPIRATOR:   If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9   PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White solid

MELTING POINT: No data

BOILING POINT: No data

MOLECULAR WEIGHT: 316.05 (anhy.)

ODOR: Odorless

SPECIFIC GRAVITY: No data

VAPOR PRESSURE: Not applicable

SOLUBILITY IN WATER: Soluble

LANTAHNUM ACETATE, HYDRATE                                                                 PAGE 2

Page 3

SECTION 10   STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Hydroscopic. Protect from humidity and moisture.

INCOMPATIBILITY: Oxidizing agents

DECOMPOSITION PRODUCTS: Carbon monoxide, carbon dioxide, and metal oxide fumes

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as probable,

possible or confirmed human carcinogen.

MUTAGENIC EFFECTS: No data

TETRATOGENIC EFFECTS: No data

CHRONIC TOXICITY: Lanthanons can cause delayed blood clotting leading to hemorrhages. Exposure may also lead to

sensitivity to heat, itching, increased awareness of odor and taste, and liver damage.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

Do not allow material to be released to the environment without proper governmental permits. Do not allow undiluted product or large

quantities of it to reach ground water, water course, or sewage system.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA Inventory

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA (TITLE 313): Not listed

MASSACHUSETTS RIGHT TO KNOW COMPONENETS: No components are subject to the Massachusetts Right to Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Lanthanum acetate

NEW JERSEY RIGHT TO KNOW COMPONENTS: Lanthanum acetate

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 03/14

SDS DEPT.

LANTHANUM ACETATE, HYDRATE                                                                 PAGE 3