Lanthanum, Chips 2098 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/lanthanum-chips-sds-prochem-en-be548a9d11

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Lanthanum, Chips

PRODUCT NUMBER:  2098

CAS NUMBER:        7439-91-0

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

H260                    In contact with water released flammable gases, which may ignite spontaneously.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces. No smoking.

P223              Keep away from any possible contact with water, because of violent reaction and possible flash fire.

P231+P232          Handle under inert gas. Protect from moisture.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P335+P334          Brush off loose particles from skin. Immerse in cool water/wrap in wet bandages.

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction

P402+P404           Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 0              Fire: 4                  Reactivity Hazard: 2

NFPA RATING:

Health: 0            Flammability: 4         Reactivity Hazard: 2       Specific Hazard: W

EYE CONTACT: Oil, and dust, if present, may cause slight to mild irritation of the eyes.

SKIN CONTACT: Dust, if present, may cause slight to mild irritation of the skin.

INHALATION: Inhalation of any oil free fine dust may be irritation to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: No information is available on the physiological effects of ingestion. Mineral oil may be harmful.

LANTHANUM CHIPS                                                                    PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: La

Molecular Weight: 138.91

CHEMICAL NAME                                           CAS#                               %

Lanthanum                                                              7439-91-0                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Dry chemical or class D metal fire agents.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Lanthanum oxides

UNUSUAL FIRE OR EXPLOSION HAZARDS: Flammable solid. Burns with intense heat. Any dust may form explosive

mixtures with air. Evolves hydrogen with hot water.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust. Keep away from ignition sources.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Avoid formation of dusts and aerosols. Handle under dry protective gas.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store under mineral oil and dry inert gas. Store

away from oxidizing agents, acidic materials, air, water, and moisture. Air sensitive.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

LANTHANUM CHIPS                                                                    PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Gray solid in mineral oil

ODOR: None

MOLECULAR WEIGHT: 138.91

BOILING POINT: 3454° C

MELTING POINT: 920° C

SPECIFIC GRAVITY: 6.17 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Reacts with water

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Air and moisture sensitive

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with hot water and any oil free fine dust with air.

INCOMPATIBILITY: Acids, oxidizing agents, air, acid chlorines, halogens, water, moisture, halogens, phosphorous, sulfur

compounds, and alcohols.

DECOMPOSITION PRODUCTS: Lanthanum oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN3208

Metallic substance, water-reactive, n.o.s.

(Lanthanum)

CLASS 4.3

PG I

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Reactivity Hazard

LANTHANUM CHIPS                                                                    PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Lanthanum

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Lanthanum

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 01/15

SDS DEPT.

LANTHANUM CHIPS                                                                    PAGE 4