Lanthanum (III) chloride hydrate 2116 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/lanthanum-iii-chloride-hydrate-prochem-en-0434ce00ba

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Lanthanum (III) chloride hydrate

PRODUCT NUMBER:  2116

CAS NUMBER:        20211-76-1

SYNONYMS:         Lanthanum trichloride hydrate

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H303             May be harmful if swallowed.

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapours/spray.

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 2            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 2              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: Causes mild to severe irritation to the eyes.

SKIN CONTACT: Causes slight to mild irritation of the skin.

INHALATION: Inhalation of powder can lead to irritation of the respiratory tract.

INGESTION: May be harmful if swallowed.

LANTHANUM (III) CHLORIDE HYDRATE                                                             PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: LaCl3  XH2O

Molecular Weight: 245.27 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Lanthanum (III) chloride hydrate                                           20211-76-1                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in

breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable or combustible.

AUTO IGNITION TEMPERATURE: None

EXPLOSION LIMITS: None

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: If involved in a fire, this material may emit toxic chlorine

and hydrochloric acid fumes.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear approved personal protective equipment. Avoid dust formation. Avoid breathing dust, vapors, mist

or gas. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Avoid contact with skin and eyes. Avoid formation of dust. Ensure adequate ventilation.

CONDITIONS FOR SAFE STORAGE: Store material in a tightly sealed container and keep away from heat.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White crystal

MOLECULAR WEIGHT: 245.27 (anhy.)

BOILING POINT: No data available

MELTING POINT: 92-93° C (dec.)

LANTHANUM (III) CHLORIDE HYDRATE                                                             PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

SPECIFIC GRAVITY: No data available

VAPOR PRESSURE: Not applicable

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Air stable, hygroscopic

HAZARDOUS POLYMERIZATION: None

CONDITIONS TO AVOID: Exposure to moisture

INCOMPATIBILITY: Active metals such as sodium or potassium, strong oxidizing agents, and strong acids

DECOMPOSITION PRODUCTS: Hydrogen chloride gas and Lanthanum oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: For lanthanum chloride (anhydrous): Oral (rat); LD50: 4184 mg/kg. Interperitoneal (rat); LD50: 106 mg/kg.

Intravenous (rat); LDLo: 4 mg/kg. Intraperitoneal (mouse); LD50: 121 mg/kg. Subcutaneous (mouse); LD50: 2424 mg/kg. Intravenous

(mouse); LD50: 18 mg/kg. Intravenous (rabbit); LD50: 148 mg/kg. Intraperitoneal (guinea pig); LD50: 129 mg/kg. Intraperitoneal (frog)

LD50: 694 mg/kg. Intraperitoneal (mouse); TDLo: 112 mg/kg. Intratesticular (domestic mammal); TDLo: 5 mg/kg. Morphological

transformation (mouse cells); 30 umol/L. Sperm Morphology (Intratesticular – domestic mammal); 5 mg/kg. Effects include somnolenc

and general depression.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified as a confirmed

carcinogen by IARC, ACGIH, NTP, and OSHA.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory as LaCl3 (CAS# 10099-58-8)

SARA 313 COMPONENTS: This material does not contain any chemical components with know CAS numbers that exceed the

threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

OSHA: Irritant

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 311/312 HAZARDS: Acute Health Hazard

LANTHANUM (III) CHLORIDE HYDRATE                                                             PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right To Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Lanthanum (III) chloride hydrate

NEW JERSEY RIGHT TO KNOW COMPONENTS: Lanthanum (III) chloride hydrate

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/14

LANTHANUM (III) CHLORIDE HYDRATE                                                             PAGE 4