Lanthanum (III) fluoride, anhydrous SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/lanthanum-iii-fluoride-anhydrous-sds-prochem-en-90b8b03941

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1  PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Lanthanum (III) fluoride, anhydrous

PRODUCT NO:       2128

CAS NO:             13709-38-1

SYNONYMS:         Lanthanum trifluoride

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2  HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315               Causes skin irritation

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapours/spray

P302 + P352           IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305 + P351 + P338   IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present

P405                 Store locked up

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: Causes severe irritation of the eyes.

SKIN CONTACT: Causes irritation, with redness and pain. Soluble fluoride may be corrosive. Effects may not appear immediately.

INHALATION:  Irritating to skin, eyes, and respiratory tract.

LANTHANUM (III) FLOURIDE                                                                     PAGE 1

Page 2

SECTION 2  HAZARDS IDENTIFICATION (Cont.)

INGESTION: May cause salivation, nausea, vomiting, diarrhea, and abdominal pain.

CHRONIC HEALTH EFFECTS: Prolonged exposure to water soluble fluorine compounds can cause deterioration of bone and tooth

structure.

ACUTE HEALTH EFFECTS: Irritating to skin, eyes and respiratory tract. Fluoride poisoning may cause nausea, vomiting, diarrhea,

weakness, coma, and respiratory paralysis.

SECTION 3   COMPOSITION / INFORMATION ON INGREDIENTS

Formula:   LaF3

Molecular Weight: 195.9

CHEMICAL NAME                         CAS#        %                ACGIH (TWA)            OSHA (PEL)

Lanthanum (III) fluoride                              13709-38-1         100                   2.5 mg/m 3 (as F)                 2.5 mg/m 3 (as F)

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical attention.

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in breathing,

coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: None

EXPLOSION LIMITS: None

EXTINGUISHING MEDIUM: None. Material if not flammable.

SPECIAL FIRE FIGHTING PROCEDURES:   If involved in a fire, fire fighters should be equipped with a NIOSH approved positive

pressure self-contained breathing apparatus and full protective clothing.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen fluoride, Lanthanum oxides

UNUSUAL FIRE OR EXPLOSION HAZARDS: No unusual fire or explosion hazards.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUATIONS: Avoid dust formation. Avoid breathing vapors, mist or gas.

ENVIRONMENTAL PRECAUTIONS: Do not let products enter drains.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with powdered sodium

bicarbonate, lime, or calcium carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Provide appropriate exhaust ventilation. Normal measures for preventive fire protection.

CONDITIONS FOR SAFE STORAGE: Store in a tightly sealed container away from moisture. Handle under a dry atmosphere of air or

nitrogen. Prolonged exposure to the atmosphere may degrade the product.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, or safety goggles, face shield when

handling a chemicals substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

LANTHANUM (III) FLOURIDE                                                                     PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

VENTILATION:  If possible, handle the material in an efficient fume hood.

RESPIRATOR:   If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9   PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White powder

MELTING POINT: 1493° C

BOILING POINT: 2330° C

MOLECULAR WEIGHT: 195.9

ODOR: None

SPECIFIC GRAVITY: 5.936 gm/ml

VAPOR PRESSURE: No data available

VAPOR DENSITY (AIR = 1): No data available

SOLUBILITY IN WATER: Slightly soluble

SECTION 10   STABILITY AND REACTIVITY

STABILITY: Stable under recommended storage conditions

HAZARDOUS POLYMERIZATION: No hazardous

CONDITIONS TO AVOID: Contact with moisture

INCOMPATIBILITY: Active metals

DECOMPOSITION PRODUCTS: Hydrogen fluoride, Lanthanum oxides

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as probable,

possible or confirmed human carcinogen.

MUTAGENIC EFFECTS: May cause adverse reproductive effects (fetotoxicity) based on animal data.

TETRATOGENIC EFFECTS: No data

TOXICITY DATA: Hazardous in case of skin contact (irritant), of ingestion, of inhalation.

RTECS DATA: No information available in the RTECS files.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

Do not allow material to be released to the environment without proper governmental permits. Do allow undiluted product or large

quantities of it to reach ground water, water course, or sewage system.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA Inventory

LANTHANUM (III) FLOURIDE                                                                     PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

OSHA HAZARDS: No known OSHA hazards

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA (TITLE 313): This material does not contain any chemical components with known CAS numbers that exceed the threshold (De

Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 HAZARD: No SARA hazards

MASSACHUSETTS RIGHT TO KNOW COMPONENETS: No components are subject to the Massachusetts Right to Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Lanthanum fluoride

NEW JERSEY RIGHT TO KNOW COMPONENTS: Lanthanum fluoride

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 03/14

SDS DEPT.

LANTHANUM (III) FLOURIDE                                                                     PAGE 2