Lanthanum (La) Standard Solution SRM 3127a SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/lanthanum-la-standard-solution-nist-srm-3127a-en-30f2064582

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

21 March 2014

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   3127a

SRM Name:     Lanthanum (La) Standard Solution

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM)  is intended for use as a primary calibration standard for the

quantitative determination of lanthanum. A unit of SRM 3127a consists of five 10 mL sealed borosilicate glass

ampoules of an acidified aqueous solution prepared gravimetrically to contain a known mass fraction of

lanthanum.  The solution contains nitric acid at a volume fraction of approximately 10 %, equivalent to an

amount of substance concentration (molarity) of approximately 1.6 mol/L.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300   (North America)

E-mail: SRMMSDS@nist.gov                        +1-703-527-3887   (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Classification

Physical Hazard:    Not classified.

Health Hazard:      Skin Corrosion/Irritation            Category 1B

Serious Eye Damage/Eye Irritation   Category 1

Label Elements

Symbol

Signal Word

DANGER

Hazard Statement(s)

H314                  Causes severe skin burns and eye damage.

Precautionary Statement(s)

P260              Do not breathe fume, mists, vapors or spray.

P264               Wash hands thoroughly after handling.

P280                Wear protective gloves, protective clothing and eye protection.

P301 + P330 + P331       If swallowed: Rinse mouth. Do NOT induce vomiting.

P303 + P361 + P353       If on skin (or hair): Take off immediately all contaminated clothing. Rinse skin with

water.

P304 + P340                 If inhaled: Remove person to fresh air and keep comfortable for breathing.

P305 + P351 + P338       If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses,

if present and easy to do. Continue rinsing.

P310                   Immediately call a doctor.

P363               Wash contaminated clothing before reuse.

SRM 3127a                                                                            Page 1 of 6

Page 2

P405                     Store locked up.

P501                   Dispose of contents according to local regulations.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Lanthanum in nitric acid solution

Other Designations:

Nitric acid (Aqua fortis; hydrogen nitrate; azotic acid; engraver’s acid)

Lanthanum nitrate [Lanthanum (III) trinitrate; lanthanum nitrate; nitric acid, lanthanum (3+) nitrate]

NOTE: Lanthanum in nitric acid solution forms a solvated lanthanum nitrate salt. The health and physical hazard

information provided in this SDS are for nitric acid and lanthanum nitrate. No physical or chemical data are listed

for this solution. The actual effects of the solution may differ from the individual components.

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the Certificate of

Analysis.

Hazardous Component(s)       CAS Number       EC Number       Nominal Mass Concentration

(EINECS)                 (%)

Nitric acid                        7697-37-2             231-714-2                   10

Lanthanum nitrate                10099-59-9             233-238-0                     2.34

Non-Hazardous Component(s)

Water                            7732-18-5             231-791-2                  >87

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:   If adverse effects occur, remove to uncontaminated area.   If not breathing, give  artificial

respiration or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing

and shoes. Get immediate medical attention.  Thoroughly clean and dry contaminated clothing before reuse.

Destroy contaminated shoes.

Eye Contact: Immediately flush eyes, including under the eyelids with copious amounts of water for at least

15 minutes. Seek immediate medical attention.

Ingestion: Contact a poison control center immediately for instructions. Do not induce vomiting. Give water

to rinse out mouth.  Never give liquids to a person with reduced awareness or becoming unconscious.   If

vomiting occurs, keep head lower than hips to prevent aspiration.  If not breathing, give artificial respiration by

qualified personnel. Seek immediate medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Acid burns to skin, eyes, and lungs.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek immediate medical attention.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing media appropriate to the surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: Thermal decomposition will form oxides of nitrogen.

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

SRM 3127a                                                                            Page 2 of 6

Page 3

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 3            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures:  Immediately contact emergency

personnel.  Keep unnecessary personnel away.  Use suitable protective equipment; see Section 8, “Exposure

Controls and Personal Protection”.

Methods and Materials for Containment and Clean up: Do not touch spilled material. Notify safety personnel

of spills. Absorb with sand or other non-combustible material. Collect spilled material in appropriate container for

disposal. Isolate hazard area and deny entry.

7. HANDLING AND STORAGE

Safe Handling Precautions: See Section 8, “Exposure Controls and Personal Protection”. Handle glass ampoules

with care.

Storage:  Store and handling in accordance with all current regulations and standards.  Keep separated from

incompatible substances (acids, combustible materials, halo carbons, amines, bases, oxidizing materials, metals,

halogens, metal salts, metal oxides, reducing agents, peroxides, metal carbide, cyanides).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits:

Component: Nitric acid

NIOSH (REL):     5 mg/m3 (2 ppm; TWA)

10 mg/m3 (4 ppm; STEL)

65 mg/m3 (25 ppm; IDLH)

ACGIH (TLV):    5 mg/m3 (2 ppm; TWA)

10 mg/m3 (4 ppm; STEL)

OSHA (PEL):      5 mg/m3

Component: Lanthanum nitrate: No occupational limits established.

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:   If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection:  Wear splash resistant safety goggles with a face shield. An eyewash station should be

readily available near areas of use.

Skin and Body Protection:  Personal protective equipment for the body should be selected based on the task

being performed and the risks involved and should be approved by a specialist before handling this product.

Chemical-resistant gloves should be worn at all times when handling chemicals.

9. PHYSICAL AND CHEMICAL PROPERTIES

NOTE: The physical and chemical data provided are for the pure components. No physical or chemical data are

available for this solution of lanthanum nitrate and nitric acid. The actual behavior of the solution may differ from

the individual components.

Descriptive Properties:                                Nitric acid                Lanthanum nitrate

(10 % of this SRM)             (2.34 % of this SRM)

Appearance                                          colorless to yellow               solid

(physical state, color, etc.):                          liquid

Molecular Formula:                     HNO3                       La(NO3)3

Molar Mass (g/mol):                            63.01                         324.92

Odor:                                                      irritating odor                Not available

Odor threshold:                                  not available                 Not available

SRM 3127a                                                                            Page 3 of 6

Page 4

Descriptive Properties:                                Nitric acid                Lanthanum nitrate

(10 % of this SRM)             (2.34 % of this SRM)

pH:                                        1 (1 M)                        not applicable

Evaporation rate:                                 not available                    not applicable

Melting point/freezing point (ºC):               –42 (–43 ºF)                    not available

Relative Density (g/L):                            not available                    not available

Vapor Pressure (mmHg):                        47.9 at 20 ºC                   not applicable

Vapor Density (air = 1):                           3.2                             not applicable

Viscosity (cP):                                    not available                    not available

Solubility(ies):                                    miscible with water and         not available

ether

Partition coefficient (n-octanol/water):            not available                    not available

Particle Size (if relevant)                          not applicable                  not available

Thermal Stability Properties:

Autoignition Temperature (ºC):                   not applicable                  not applicable

Thermal Decomposition (ºC):                     not applicable                  not available

Initial boiling point and boiling range (ºC):       83 (181 ºF)                     not applicable

Explosive Limits, LEL (Volume %):              not applicable                  not applicable

Explosive Limits, UEL (Volume %):              not applicable                  not applicable

Flash Point (ºC)                                   not applicable                  not applicable

Flammability (solid, gas):                         not applicable                  not applicable

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:     X    Stable               Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Contact with combustible or incompatible materials.

Incompatible Materials:  Acids, combustible materials, halo carbons, amines, bases, oxidizing materials, metals,

halogens, metal salts, metal oxides, reducing agents, peroxides, metal carbide, cyanides.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will produce oxides of nitrogen.

Hazardous Polymerization:         Will Occur    X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:   X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Burning pain and severe skin

corrosion, and eye damage.

Potential Health Effects (Acute, Chronic and Delayed):

Inhalation:  Inhalation of nitric acid can damage the mucous membranes and upper respiratory tract.  Short

term exposure may cause irritation and inflammation of the upper respiratory tract, coughing, choking, sore

throat, shortness of breath, headache, dizziness, and nausea.  Long term exposure to acid fumes may cause

damage to teeth, bronchial irritation, chronic cough, bronchial pneumonia, and gastrointestinal disturbances.

Lanthanum nitrate exposure may cause irritation; chronic exposure may result in itching, sensitivity to heat,

lung damage.

Skin Contact:  Nitric acid can cause severe skin burns.  Severity of the damage depends on the concentration

and duration of exposure.  Effects of acid burns may be delayed. Lanthanum nitrate exposure may result in

irritation and hair loss.

SRM 3127a                                                                            Page 4 of 6

Page 5

Eye Contact:  Nitric acid can cause severe eye irritation, corneal burns, permanent eye damage, or blindness.

Severity of the damage depends on the concentration and duration of exposure. Lanthanum nitrate exposure

may cause irritation.

Ingestion:  If ingested, nitric acid can cause severe burns and damage to the gastrointestinal tract.  Intestinal

absorption is usually poor for rare earth salts, lanthanum nitrate.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified.

Nitric acid, Rat, Inhalation LC50: 130 mg/m3 (4 h)

Lanthanum nitrate, Rat, Oral LD50: 4500 mg/kg

Skin Corrosion/Irritation: This SRM contains >1 % of nitric acid and it is classified as Category 1B.

Serious Eye damage/Eye irritation: This SRM contains >1 % nitric acid and it is classified as Category 1.

Respiratory Sensitization: No data available.

Skin Sensitization: No data available.

Germ Cell Mutagenicity: No data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen              Yes       X   No

Nitric acid is not listed by NTP, IARC or OSHA as a carcinogen.

Lanthanum nitrate is not listed by NTP, IARC or OSHA as a carcinogen.

Reproductive Toxicity: Not classified.

Nitric acid, Rat, Oral TDLo: 21 150 mg/kg (pregnant 1 d to 21 d)

Nitric acid, Rat, Oral TDLo: 2345 mg/kg (pregnant 18 d)

Lanthanum nitrate, Rat, Subcutaneous TDLo: 25 995 µg/kg (1 d)

Specific Target Organ Toxicity, Single Exposure: Not classified.

Specific Target Organ Toxicity, Repeated Exposure: Not classified.

Aspiration Hazard: No data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data:

Nitric acid: Starfish (Asterias rubens) LC50: 100-300 mg/L (48 h, renewal/aerated water)

Lanthanum nitrate: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal:  Dispose of waste in accordance with all applicable federal, state, and local regulations.  Nitric

acid subject to disposal regulations: U.S. EPA 40 CFR 262, Hazardous Waste Numbers: D001, D002.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: UN1760, Corrosive liquid, n.o.s. (contains nitric acid), Hazard Class 8, Packing Group II,

Excepted Quantity E2.

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4):  Nitric acid, 1000 lbs (454 kg) RQ

SARA Title III Section 302 (40 CFR 355.30):  Nitric acid, 1000 lbs (454 kg) TPQ

SARA Title III Section 304 (40 CFR 355.40):  Nitric acid, 1000 lbs (454 kg) EPCRA RQ

SARA Title III Section 313 (40 CFR 372.65):  Nitric acid, 1 % de minimis concentration.

SRM 3127a                                                                            Page 5 of 6

Page 6

OSHA Process Safety (29 CFR 1910.119):     Regulated   for    nitric   acid    at   higher   concentrations

500 lbs TQ (≥94.5 % by weight).

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:        Yes.

CHRONIC HEALTH:     No.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations:

California Proposition 65: Not listed.

U.S. TSCA Inventory: Nitric acid and lanthanum nitrate are listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations:

WHMIS Information: Not provided for this material.

16. OTHER INFORMATION

Issue Date: 21 March 2014

Sources:   ChemAdvisor, Inc., MSDS Nitric Acid, 07 February 2014.

ChemAdvisor, Inc., MSDS Lanthanum Nitrate, 23 December 2013.

CDC; NIOSH; NIOSH Pocket Guide to Chemical Hazards; Department of Health and Human

Services (DHHS), Centers for Disease Control and Prevention (CDC), National Institute for Safety and

Health;  Nitric Acid, 18 November 2010; available  at http://www.cdc.gov/niosh/npg/npgd0447.html

(accessed Mar 2014).

Hazardous Substances Data Bank (HSDB), National Library of Medicine's TOXNET system, Nitric

Acid CAS No. 7697-37-2; available at http://toxnet.nlm.nih.gov (accessed Mar 2014).

Key of Acronyms:

ACGIH       American Conference  of Governmental  Industrial NRC          Nuclear Regulatory Commission

Hygienists

ALI          Annual Limit on Intake                     NTP           National Toxicology Program

CAS         Chemical Abstracts Service                 OSHA         Occupational Safety and Health Administration

CERCLA     Comprehensive      Environmental      Response, PEL           Permissible Exposure Limit

Compensation, and Liability Act

CFR         Code of Federal Regulations                RCRA        Resource Conservation and Recovery Act

DOT         Department of Transportation                 REL         Recommended Exposure Limit

EC50          Effective Concentration, 50%             RM           Reference Material

EINECS      European   Inventory   of   Existing  Commercial RQ            Reportable Quantity

Chemical Substances

EPCRA      Emergency Planning and Community Right-to-Know RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC          International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA          International Air Transportation Agency          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

LC50          Lethal Concentration, 50 %                   STEL          Short Term Exposure Limit

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NFPA         National Fire Protection Association          TWA        Time Weighted Average

NIOSH        National Institute for Occupational Safety and Health  UEL         Upper Explosive Limit

NIST          National Institute of Standards and Technology     WHMIS      Workplace Hazardous Materials Information System

n.o.s.         Not Otherwise Specified

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does

not certify the data in the SDS.  The certified values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current.  This can be accomplished by

contacting the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the

Internet at http://www.nist.gov/srm.

SRM 3127a                                                                            Page 6 of 6