Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate SDS

Publisher: Neonest AB

Source: https://resds.com/document/lithium-7-isotope-sds-li-7-7li-lithium-hydroxide-monohydrate-enriched-lithium-hydroxide-monohydrate-31c5cfb65d

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Lithium Hydroxide Monohydrate, Enriched Lithium.

Molecular weight:   41.96

Chemical formula:   LiOH H 2 O

Synonyms:   Lithium hydroxide.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Acute Toxicity, Oral (Category 4)   H302

Skin Corrosive (Category 1B)   H314

Eye Damage (Category 1)   H318

Label elements:

Hazard pictograms:

Signal Words:   Danger.

Page 2

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 2/9

Hazard Statements:

H302 Harmful if swallowed.

H314 Causes severe skin burns and eye damage.

H318 Causes serious eye damage.

Precautionary Statements:

P270 Do not eat, drink or smoke when using this product.

P280 Wear protective gloves/protective clothing/eye protection/face protection.

P301 + P330 + P331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P303 + P361 + P353 IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water [or

shower].

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and

easy to do. Continue rinsing.

P310 Immediately call a POISON CENTER/doctor.

P405 Store locked up.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Concentration

% by weight

Classification

EC1272/2008 EC - No.

Lithium hydroxide

1310-66-3

>99.9%

Acute Tox. 4   H302

Skin Corr. 1B   H314

Eye Dam. 1   H318 603-454-3

For explanation of abbreviations see Section 16.

Section 4:   First - Aid Measures

Description of first aid measures:

Eyes:   Immediately flush with water for at least 15 minutes, lifting upper and lower lids intermittently. See a doctor

immediately.

Skin:   Immediately flush with plenty of water while removing contaminated clothing and/or shoes and thoroughly

wash with soap and water. Obtain medical attention. Contact a doctor if necessary.

Ingestion:   Rinse mouth with water. Dilute by giving 1 or 2 glasses of water. Do not induce vomiting. Do not give

anything by mouth to an unconscious person. See a doctor immediately.

Inhalation:   Remove to fresh air. If breathing discomfort occurs and persists, see a doctor. If breathing has stopped,

give artificial respiration and see a doctor immediately.

Notes to doctor:   This product is corrosive to the skin, eyes and mucous membranes of the respiratory and

gastrointestinal tracts. Consideration should be given to gastric lavage, with endotracheal tube in place. Treatment is

controlled removal of exposure with symptomatic and supportive care.

Most important symptoms and effects, both acute and delayed:

Page 3

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 3/9

Inhalation:   Corrosive. Extremely destructive to tissues of the mucous membranes and upper respiratory tract.

Symptoms may include burning sensation, coughing, wheezing, laryngitis, shortness of breath, headache, nausea and

vomiting. Inhalation may be fatal as a result of spasm inflammation and edema of the larynx and bronchi, chemical

pneumonitis and pulmonary edema.

Ingestion:   Corrosive. Swallowing can cause severe burns of the mouth, throat and stomach, leading to death. Can

cause sore throat, vomiting, diarrhoea. In severe cases, lithium can cause apathy, sluggishness, drowsiness, slurred

speech, blurred vision, irregular eye movements, weakness, incoordination, lethargy, heart effects, brain effects,

ringing in the ears, tremors and muscle twitching, central nervous system damage, kidney effects, thyroid changes,

coma, pulmonary edema and renal failure.

Skin Contact:   Dermal contact with alkaline corrosives may produce pain, redness, severe irritation or full thickness

burns.

Eye Contact:   Corrosive. Contact can cause blurred vision, redness, pain and severe tissue burns.

Chronic Exposure:   Prolonged skin contact causes dermatitis, deep burns and scarring. Chronic exposure may damage

the liver or kidneys and may cause central nervous system depression.

Aggravation of Pre-existing Conditions:   Persons with pre-existing skin disorders or eye problems or impaired

respiratory function may be more susceptible to the effects of the substance.

Section 5: Fire - Fighting Measures

Extinguisher media:

Flammable Limits:   Upper: Not available. Lower: Not available.

General Hazards:   None.

Extinguishing Media:   Dry chemical, CO 2 , water or regular foam.

Hazardous Combustion Products:   Corrosive lithium hydroxide dust.

Fire-Fighting Procedure:   Wear full protective clothing and Self-Contained Breathing Apparatus (SCBA) approved for

firefighting. This is necessary to protect against the hazards of heat, products of combustion and oxygen deficiency.

Do not breathe smoke, gases or vapours generated.

Flash Point:   Not applicable.

Autoignition Temperature:   Not applicable.

Properties Contributing to Flammability:   None.

Sensitivity to Static Discharge:   Not applicable.

Sensitivity to Impact:   Not applicable.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Ventilate area of leak or spill. Keep unnecessary and unprotected people away from area of spill. Wear appropriate

personal protective equipment as specified in Section 8. Avoid dust formation. Avoid breathing vapours, mist or gas.

Environmental precautions:   Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Discharge into the environment must be avoided.

Methods and materials for containment and cleaning up:

Sweep up and collect in a suitable transport container.

Page 4

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 4/9

Section 7: Handling and Storage

Precautions for safe handling:

Handling:   Do not get in eyes, on skin or clothing. Avoid breathing dust. Wash thoroughly after handling.

Conditions for safe storage, including any incompatibilities:

Keep container closed. Store away from acids and water.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:   STEL (EH40): 1mg/m 3

Exposure controls:

Appropriate engineering controls:   Use local exhaust ventilation to keep airborne concentrations below exposure

limits.

Individual protection measures, such as personal protective equipment:

Respiratory:   Wear a CEN Class P respirator approved for protection against inorganic dusts, when adequate

ventilation is not available.

Eyes:   Safety glasses or goggles.

Gloves:   Nitrile (typical permeation breakthrough time >480 minutes).

These glove recommendations should not be used as the absolute basis for glove selection. Actual in-use conditions

may vary glove performance from the controlled conditions of laboratory tests. Factors such as concentration and

temperature, glove thickness and glove reuse, may affect performance. Other glove requirements, such as length,

dexterity, cut, abrasion, puncture and snag resistance, or glove grip need to be considered in making your final

selection.

Others:   Quick-drench eyewash and safety shower.

Section 9: Physical and Chemic al Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (crystalline).

Colour:   White.

Odour:   Odourless.

Odour threshold:   No data available.

pH (concentration):   No data available.

Melting point/range (° C):   462   °C

Boiling point/range (° C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   Non-flammable.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Page 5

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 5/9

Vapour pressure (20   ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   No data available.

Water solubility (g/L ) at 20 ° C:   216 g/L

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Stability:   Stable under normal storage and temperature conditions.

Conditions to avoid:   None.

Hazardous polymerization:   Will not occur.

Incompatible materials:   Acids, aluminium, zinc.

Hazardous decomposition products:   None.

Section 11: Toxicological Information

Information on toxicological effects:

Harmful if swallowed.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Causes severe skin burns and eye damage.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Page 6

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 6/9

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   No data available.

Other adverse effects:   No data available.

Section 13: Disposal Conside rations

Product:   Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved

waste disposal facility. Processing, use or contamination of this product may change the waste management options.

State and local disposal regulations may differ from federal disposal regulations. Dispose of unused contents in

accordance with federal, state and local requirements.

Contaminated Packaging : Dispose of container in accordance with local requirements.

Section 14: Transport Information

UN number:   2680

UN proper shipping name:   LITHIUM HYDROXIDE MONOHYDRATE

Transport hazard class:   Class 8

Transport hazard labels:

Packing group:   II

Marine pollutant:   No.

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Page 7

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 7/9

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Additional Information:

SARA 302 Components:   No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components:   This material does not contain any chemical components with known CAS numbers that

exceed the threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards:   Acute Health Hazard, Chronic Health Hazard.

Massachusetts Right to Know Components:   No components are subject to the Massachusetts Right to Know Act.

Pennsylvania Right to Know Components:   Lithium hydroxide monohydrate / CAS No. 1310-66-3

New Jersey Right to Know Components:   Lithium hydroxide monohydrate / CAS No. 1310-66-3

California Prop. 65 Components:   This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Page 8

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 8/9

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

Page 9

Safety Data Sheet for Lithium Hydroxide

Monohydrate, Enriched Lithium

Hydroxide Monohydrate

According to ISO 11014:2010

First Print Date: 05-Mar-2020

Revision Date: 13-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Lithium Hydroxide Monohydrate, Enriched Lithium Hydroxide Monohydrate   Page 9/9

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H302 Harmful if swallowed.

H314 Causes severe skin burns and eye damage.

H318 Causes serious eye damage.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.