Lutetium (III) carbonate ALFAA14569 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/lutetium-iii-carbonate-sds-avocado-research-chemicals-ltd-part-of-thermo-fisher-en-3fd5ace48a

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 7             SAFETY DATA SHEET

Revision Date 31-Jul-2026

Version 5

ALFAA14569

Lutetium (III) carbonate

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:                    碳酸镥(III),碳酸镥(III),碳酸镥(III),碳酸镥(III),REacton®REacton®REacton®REacton®

Product Description:              Lutetium (III) carbonate

Cat No. :                        14569

CAS No                           5895-53-4

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Solid Crystalline                            White                                Odorless

Emergency Overview

The product contains no substances which at their given concentration are considered to be hazardous to health.

Classification of the substance or mixture

Based on available data, the classification criteria are not met

Label Elements

None required

Physical and Chemical Hazards

None identified.

Health Hazards

The product contains no substances which at their given concentration are considered to be hazardous to health.

Environmental hazards

Contains no substances known to be hazardous to the environment or not degradable in waste water treatment plants. Is not likely

mobile in the environment due its low water solubility. Spillage unlikely to penetrate soil.

This product does not contain any known or suspected endocrine disruptors.

______________________________________________________________________________________________

Page 1

Page 2

ALFAA14569               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 2 / 7

Lutetium (III) carbonate

______________________________________________________________________________________________

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Component                              CAS No                Weight %

Carbonic acid, lutetium(3+) salt (3:2)                               5895-53-4                <=100

SECTION 4. FIRST AID MEASURES

Eye Contact

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Get medical attention.

Skin Contact

Wash off immediately with plenty of water for at least 15 minutes. Get medical attention immediately if symptoms occur.

Inhalation

Remove to fresh air. Get medical attention immediately if symptoms occur.

Ingestion

Clean mouth with water and drink afterwards plenty of water. Get medical attention if symptoms occur.

Most important symptoms and effects

None reasonably foreseeable.

Self-Protection of the First Aider

No special precautions required.

Notes to Physician

Treat symptomatically.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

Extinguishing media which must not be used for safety reasons

No information available.

Specific Hazards Arising from the Chemical

Thermal decomposition can lead to release of irritating gases and vapors.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Ensure adequate ventilation. Use personal protective equipment as required. Avoid dust formation.

Environmental Precautions

Should not be released into the environment. See Section 12 for additional Ecological Information.

Methods for Containment and Clean Up

Sweep up and shovel into suitable containers for disposal. Avoid dust formation.

______________________________________________________________________________________________

Page 2

Page 3

ALFAA14569               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 3 / 7

Lutetium (III) carbonate

______________________________________________________________________________________________

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

Wear personal protective equipment/face protection. Ensure adequate ventilation. Avoid contact with skin, eyes or clothing. Avoid

ingestion and inhalation. Avoid dust formation.

Storage

Keep container tightly closed in a dry and well-ventilated place.

Specific Use(s)

Use in laboratories

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Monitoring methods

BS EN 14042:2003 Title Identifier: Workplace atmospheres. Guide for the application and use of procedures for the assessment of

exposure to chemical and biological agents. MDHS14/3 General methods for sampling and gravimetric analysis of respirable and

inhalable dust

Exposure Controls

Engineering Measures

None under normal use conditions.  .

Personal protective equipment

Eye Protection              Wear safety glasses with side shields (or goggles) (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Natural rubber     See manufacturers               -          EN 374             (minimum requirement)

Nitrile rubber       recommendations

Neoprene

PVC

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection        No protective equipment is needed under normal use conditions.

Large scale/emergency use    Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: Particle filter

Small scale/Laboratory use      Maintain adequate ventilation

______________________________________________________________________________________________

Page 3

Page 4

ALFAA14569               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 4 / 7

Lutetium (III) carbonate

______________________________________________________________________________________________

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls  No information available.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance                       White

Physical State                        Solid Crystalline

Odor                              Odorless

Odor Threshold               No data available

pH                         No information available

Melting Point/Range              > 500 °C  / 932 °F

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Evaporation Rate                  Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Vapor Pressure                No data available

Vapor Density                     Not applicable                         Solid

Specific Gravity  / Density        No data available

Bulk Density                  No data available

Water Solubility                      Insoluble in water

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Autoignition Temperature        No data available

Decomposition Temperature      No data available

Viscosity                         Not applicable                         Solid

Explosive Properties            No information available

Oxidizing Properties            No information available

SECTION 10. STABILITY AND REACTIVITY

Stability                             Stable under normal conditions.

Hazardous Reactions            None under normal processing.

Hazardous Polymerization        No information available.

Conditions to Avoid             None known.

Materials to avoid              No information available.

Hazardous Decomposition Products None under normal use conditions.

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

(a) acute toxicity;

(b) skin corrosion/irritation;       No data available

(c) serious eye damage/irritation;   No data available

______________________________________________________________________________________________

Page 4

Page 5

ALFAA14569               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 5 / 7

Lutetium (III) carbonate

______________________________________________________________________________________________

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;             No data available

There are no known carcinogenic chemicals in this product

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

Target Organs              No information available.

(j) aspiration hazard;               Not applicable

Solid

Symptoms  / effects,both acute and No information available

delayed

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects                 Contains no substances known to be hazardous to the environment or that are not

degradable in waste water treatment plants.

Persistence and Degradability

Persistence                       Insoluble in water.

Degradability                  Not relevant for inorganic substances.

Bioaccumulative Potential        May have some potential to bioaccumulate

Mobility in soil                        Spillage unlikely to penetrate soil Is not likely mobile in the environment due its low water

solubility

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Chemical waste generators must determine whether a discarded chemical is classified as a

Products                         hazardous waste. Consult local, regional, and national hazardous waste regulations to

ensure complete and accurate classification.

Contaminated Packaging          Empty remaining contents. Dispose of in accordance with local regulations. Do not re-use

empty containers.

______________________________________________________________________________________________

Page 5

Page 6

ALFAA14569               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 6 / 7

Lutetium (III) carbonate

______________________________________________________________________________________________

Other Information                Waste codes should be assigned by the user based on the application for which the product

was used.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport           Not Regulated

IMDG/IMO                        Not regulated

IATA                             Not regulated

Special Precautions for User      No special precautions required

SECTION 15. REGULATORY INFORMATION

International Inventories

X = listed, China (IECSC), Europe (EINECS/ELINCS/NLP), U.S.A. (TSCA), Canada (DSL/NDSL), Philippines (PICCS), Japan (ENCS), Japan

(ISHL), Australia (AICS), Korea (KECL).

Component        The        List of    IECSC   EINECS   TSCA   DSL   PICCS  ENCS    ISHL    AICS    KECL

Inventory of dangerous

Hazardous  goods GB

Chemicals   12268 -

(2015      2012

Edition)

Carbonic acid,              -                 -                -      227-586-2    X            -             -      X      X            -     KE-11008

lutetium(3+) salt (3:2)

National Regulations

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

Revision Date                      31-Jul-2026

Revision Summary                Not applicable.

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

______________________________________________________________________________________________

Page 6

Page 7

ALFAA14569               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 7 / 7

Lutetium (III) carbonate

______________________________________________________________________________________________

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of Safety Data Sheet

______________________________________________________________________________________________

Page 7