Lutetium (III) Fluoride 2411 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/lutetium-iii-fluoride-prochem-en-6f65615335

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Lutetium (III) Fluoride

PRODUCT NUMBER:  2411

CAS NUMBER:        13760-81-1

SYNONYMS:          Lutetium trifluoride

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H302+H312          Harmful if swallowed or in contact with skin.

H331                 Toxic if inhaled.

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

305+P351+P338       IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P304+P340            IF INHALED: Remove person to fresh air and keep comfortable for breathing.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

EYE CONTACT: Causes serious eye irritation.

SKIN CONTACT: Causes skin irritation.

LUTETIUM (III) FLUORIDE                                                                PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

INHALATION: Toxic if inhaled.

INGESTION: Harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: LuF3

Molecular Weight: 231.96

CHEMICAL NAME                                           CAS#                               %

Lutetium (III) Fluoride                                                    13760-81-1                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with soap and plenty of water. Remove contaminated clothes if necessary. Rub

skin with calcium gluconate. Seek medical assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen fluoride and lutetium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation and breathing dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in tightly sealed container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Avoid contact

with skin and eyes. Avoid formation of dust.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Protect from carbon dioxide.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS:

Component                 Exposure Limits              Basis                           Entity

Lutetium (III) Fluoride             2.5 mg/m3                 PEL                   OSHA

2.5 mg/m3                  TLV                     ACGIH

TLV: Threshold Limit Value over 8 hours of work.                   PEL: Permissible Exposure Limit

LUTETIUM (III) FLUORIDE                                                                PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White powder

ODOR: None

MOLECULAR WEIGHT: 231.96

BOILING POINT: 2200° C

MELTING POINT: 1182° C

SPECIFIC GRAVITY: 8.332 gm/ml

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Moisture

INCOMPATIBILITY: Strong reducing agents.

DECOMPOSITION PRODUCTS: Hydrogen fluoride and lutetium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product are considered to be a human carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: OK8837500

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN3288

Toxic solid, inorganic, n.o.s.

(Lutetium (III) Fluoride)

CLASS 6.1

PG III

Marine Pollutant: No

LUTETIUM (III) FLUORIDE                                                                PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

SARA 302/304: Not Listed

SARA 311/312: Acute Health Hazard

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

SECTION 15  REGULATORY INFORMATION (Cont.)

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Lutetium (III) Fluoride

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Lutetium (III) Fluoride.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 03/15

SDS DEPT.

LUTETIUM (III) FLUORIDE                                                                PAGE 4