Lutetium Pieces 2383 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/lutetium-pieces-prochem-en-381e4137fb

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Lutetium Pieces

PRODUCT NUMBER:  2383

CAS NUMBER:        7439-94-3

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.

P240                Ground/bond container and receiving equipment.

P241              Use explosion-proof electrical/ventilating/lighting equipment.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction.

HMIS CLASSIFICATION:

Health: 0              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 0            Flammability: 2         Reactivity Hazard: 0

EYE CONTACT: May cause irritation to the eyes.

SKIN CONTACT: May cause irritation of the skin.

INHALATION: May cause irritation to the respiratory tract.

INGESTION: May cause gastrointestinal irritation.

LUTETIUM METAL PIECES                                                                PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Lu

Molecular Weight: 174.97

CHEMICAL NAME                                           CAS#                               %

Lutetium                                                                 7439-94-3                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately wash with plenty of water for at least 15 minutes. Assure flushing under the eyelids. Seek medical

attention.

SKIN EXPOSURE: Wash of with soap and plenty of water while removing all contaminated clothing and shoes. Seek

medical attention.

INHALATION: Move to fresh air.  If breathing is difficult, give oxygen.  If not breathing, give artificial respiration. Seek

medical attention.

INGESTION: Clean out mouth with water. Seek medical attention.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: No information available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use dry chemical, approved class D extinguishers, clay, or sodium carbonate.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Lutetium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust. Remove all sources of ignition.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Avoid formation of dusts and aerosols. Keep away from sources of ignition – No smoking. Take measures to

prevent the build up of electrostatic charge. Handle under argon.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Use only non-sparking tools. Keep away from heat

and sources of ignition.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

LUTETIUM METAL PIECES                                                                PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Silver metallic pieces

ODOR: None

MOLECULAR WEIGHT: 175.97

BOILING POINT: 3402° C

MELTING POINT: 1663° C

SPECIFIC GRAVITY: 9.835 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Heat, flames, and sparks. Extremes of temperatures and direct sunlight. Exposure to air, Exposure to

moisture.

INCOMPATIBILITY: Strong oxidizing agents, strong acids, and halogens.

DECOMPOSITION PRODUCTS: Lutetium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: First Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Lutetium

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Lutetium

LUTETIUM METAL PIECES                                                                PAGE 3

Page 4

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/15

SDS DEPT.

LUTETIUM METAL PIECES                                                                PAGE 4