Monazite SDS

Publisher: RZ Resources Ltd

Source: https://resds.com/document/monazite-en-21bd9fa19a

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Monazite

1.  IDENTIFICATION MATERIAL SUPPLIER

Product Names:       Monazite

Other Names:          Mineral Sands Concentrate

Uses:             Raw material to produce rare earth compounds

Company:         RZ Resources Ltd

ABN 23 160 863 892

Address:              Level 7, 10 Eagle Street,

Brisbane, Qld 4000

Australia

Telephone:          +61 1300 735 386

Email:                product@rzresources.com

Web:                   https://rzresources.com/

Emergency Contract:   +61 408 907 889 (24 hours)

2.  HAZARDS IDENTIFICATION

Classified as hazardous according to SafeWork Australia and the Global Harmonised System of classification and

labelling of chemicals (GHS).

GHS Classifications:

Acute Toxicity: Oral: Category 5

Carcinogenicity: Category 1B

Hazard Statements:

H302             Harmful if swallowed

H332             Harmful if inhaled

Precautionary Statements:

P261             Avoid breathing dust/fume/gas/mist/vapors/spray.

P264          Wash thoroughly after handling.

P270          Do not eat, drink or smoke when using this product.

P271           Use only outdoors or in a well-ventilated area.

Other Hazards:    Radioactive

Date of Issue: October 4, 2022                                                                                Page 1 of 5

Page 2

SAFETY DATA SHEET

Monazite

3.  COMPOSITION / INFORMATION ON INGREDIENTS

Ingredients (typical)               CAS Number           EC Number         Concentration

Monazite                              1306-41-8                                 65-70%

Cerium Phosphate                      13454-71-2                236-637-8          18-22%

Yttrium Phosphate                      13990-54-0                237-790-3          10-14%

Lanthanum Phosphate                  14913-14-5                237-419-5           8-12%

Neodymium Phosphate                  14298-32-9                238-232-1           8-12%

Thorium Phosphate                     14485-31-5                               -               4-6%

Praseodymium Phosphate               14298-31-8                238-231-6           1-3%

Samarium Phosphate                   13465-57-1                236-698-0           1-3%

Dysprosium Phosphate                  13863-49-5                237-605-6           1-3%

Gadolinium Phosphate                  13628-51-8                237-112-6           1-3%

Ytterbium Phosphate                    13759-80-3                237-346-9           0.5-1.5%

Uranium Phosphate                     18433-48-2                242-306-9            0.3 -0.4%

Non monazite Components

Zircon                                 14940-68-2                239-019-6          22-26%

Rutile                                  1317-80-2                 215-282-2           2-4%

Iron Oxide                              1309-37-1                 215-168-2           1-2%

Quartz                                14808-60-7                238-878-4           1-2%

4.  FIRST AID MEASURES

Swallowed:               First aid is unlikely to be required but, if necessary, wash mouth out with water ensuring the

mouthwash is not swallowed. Give one or two glasses of water to drink. Seek medical

attention if a large quantity has been swallowed.

Inhaled:             Blow nose to remove particulates from nose. Move to area with fresh air. Seek medical

attention if adverse reaction develops.

Skin:              Remove contaminated clothing gently to avoid creating dust. Wash skin. If skin becomes

irritated, seek medical attention. Launder affected clothing before re-use

Eye:                 Hold eyelid open and flush with clean water. Continue until grit is removed. Seek medical

attention if irritation or soreness persists.

Acute

Swallowed:             Non-toxic. No known detrimental effect from accident ingestion as may occur during normal

handling. Ingestion of large amounts may cause irritation to the gastro-intestinal system due

to abrasiveness.

Date of Issue: October 4, 2022                                                                                Page 2 of 5

Page 3

SAFETY DATA SHEET

Monazite

Inhaled:               Mainly regarded as nuisance dust but may be irritating if inhaled at high concentrations. May

cause coughing and/or sneezing.

Skin:              Low hazard.

Eye:                    Solid and dust can be moderately irritating due to abrasiveness.

Chronic

Radiation:            Monazite contains naturally occurring radioactive elements of the uranium and thorium

series. The Monazite produced by RZ Resources Ltd contains concentrations of these

impurities with a typical specific activity of approximately 200 Bq/g of combined uranium and

thorium.

•  170 Bq/g (thorium-232) and

•  30 Bq/g (uranium-238).

Daughter  products are  present,  usually  at  equilibrium  concentrations  but  the main

radiological hazard is internal dust. As a guide, continuous worker exposure to respirable dust

levels above 3.5mg/m3 could give rise to annual internal exposures above 1 mSv.

External exposure is from gamma radiation. Continuous exposure (2000 hours per year)

within 2 meters of bulk Monazite could give rise to an annual external dose above 1 mSv.

Silica:                Monazite contains amounts of free quartz and precautions should be taken to avoid inhaling

the dust.

First Aid Facilities:     Eye Wash Station

Doctor Treatment:      Treat symptomatically

5. FIRE FIGHTING MEASURES

Non-flammable, non-combustible. Use suitable firefighting measures for the surrounding fire.

6. ACCIDENTAL RELEASE MEASURES

Emergency Procedures:       Not relevant

Containment and Clean-up:   Wear safety equipment for normal handling. Avoid generating dust. Vacuum up, if

possible, otherwise sweep up and recycle. If the spilled product is not suitable for

re-use, dispose of as radioactive waste.

7. HANDLING AND STORAGE

Handling:                     Dust generation should be minimised when handling. Wash thoroughly after handling.

Storage:                     Storage areas should be ventilated.

Date of Issue: October 4, 2022                                                                                Page 3 of 5

Page 4

SAFETY DATA SHEET

Monazite

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

National Exposure Standards (Source: Safe Work Australia):

Ingredient                                          TWA (mg/m3)    STEL (mg/m3)

Titanium dioxide                                                        10                        -

Iron Oxide                                                             5                        -

Respirable silica (quartz)                                                       0.05                      -

Zirconium compounds (as Zr)                                              5              10

Uranium                                                                          0.2                0.6

Biological Limit Values:       No information

Engineering Controls:           Ventilation requirements will depend on handling methods and the amounts in use,

but should be sufficient to maintain dust levels below exposure limits.

Personal Protective Equipment:  Safety glasses or goggles. If there is a risk of inhaling dust, wear an approved P2, or

better, respirator.

9. PHYSICAL CHEMICAL PROPERTIES

Appearance:                 Brownish free running sand

Odour:                      Odourless

pH:                       5 - 7

Vapour Pressure:             Not applicable

Boiling Point/Range:          Not applicable

Melting Point:                 Monazite 1900°C -2100°C, Rutile 1840°C, Quartz 1700°C, Zirconia 2200°C

Solubility:                       Insoluble

Bulk Density:                2900 kg/m3

Flash Point:                  Not applicable

Flammability Limits:           Not applicable

10. STABILITY AND REACTIVITY

Reactivity:                         Inert

Chemical Stability:             Stable

Incompatible Materials:       None in normal or expected use

Decomposition Products:       Decomposition will not occur

11. TOXICOLOGICAL INFORMATION

Acute Toxicity:      Harmful if swallowed or inhaled

Date of Issue: October 4, 2022                                                                                Page 4 of 5

Page 5

SAFETY DATA SHEET

Monazite

12. ECOLOGICAL INFORMATION

No information available.

13. DISPOSAL CONSIDERATION

If not reusable, dispose in accordance with Commonwealth, State and local government and regulations for

radioactive waste.

14. TRANSPORT INFORMATION

UN Code 2912 - Classified as a Class 7 Dangerous Goods according to the Australian Code for the Transport of

Dangerous Goods by road and rail (ADG) and international codes, IATA and IMDG.

LAND TRANSPORT        SEA TRANSPORT          AIR TRANSPORT

(CNDG)                 (IMDG / IMO)               (IATA / ICAO)

UN Number                 2912                      2912                     2912

Proper Shipping    RADIOACTIVE MATERIAL,    RADIOACTIVE MATERIAL,   RADIOACTIVE MATERIAL,

Name          LOW SPECIFIC ACTIVITY   LOW SPECIFIC ACTIVITY   LOW SPECIFIC ACTIVITY

(LSA-I), non                     (LSA-I), non                    (LSA-I), non

fissile or fissile-excepted          fissile or fissile-excepted         fissile or fissile-excepted

Transport hazard              7                         7                        7

class

Packing Group         None allocated.             None allocated.            None allocated.

Transport of Class 7 by road or rail in Australia is subject to separate state and territory legislation and the ARPANZA

Code of Practice for the Safe Transport of Radioactive Substances (2008).

Transport may be regulated in some countries.

Material should be transported in sealed containers to prevent spillage and leakage. All spillages should be cleaned

up immediately.

15. REGULATORY INFORMATON

Poisons Schedule:    None allocated

16. OTHER INFORMATION

Date of Issue:                October 4, 2022

Replaces Issue:              August 8, 2022

Review Date:                 October 4, 2027

End of Safety Data Sheet

Date of Issue: October 4, 2022                                                                                Page 5 of 5