Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol ALFAA36573 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/neodymium-iii-isopropoxide-2-5-w-v-in-toluene-isopropanol-sds-avocado-research-chemicals-ltd-part-of-thermo-en-a3fc4672dd

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 10             SAFETY DATA SHEET

Revision Date 05-Aug-2026

Version 6

ALFAA36573

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:                  异丙醇钕(III)异丙醇钕(III)异丙醇钕(III)异丙醇钕(III)

Product Description:               Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

Cat No. :                        36573

Molecular Formula             C9 H21 NdO3

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Liquid                                 Blue                    No information available

Emergency Overview

Highly flammable liquid and vapor. May be fatal if swallowed and enters airways. Causes skin irritation. Causes serious eye

irritation. Suspected of damaging fertility or the unborn child. May cause drowsiness and dizziness. Toxic to aquatic life. Harmful to

aquatic life with long lasting effects. May cause damage to organs through prolonged or repeated exposure. Moisture sensitive.

Classification of the substance or mixture

Flammable liquids.                                                                Category 2

Aspiration Toxicity                                                                Category 1

Skin Corrosion/Irritation                                                           Category 2

Serious Eye Damage/Eye Irritation                                                  Category 2

Reproductive Toxicity                                                             Category 2

Specific target organ toxicity - (single exposure)                                       Category 3

Specific target organ toxicity - (repeated exposure)                                    Category 2

Acute aquatic toxicity                                                              Category 2

Chronic aquatic toxicity                                                            Category 3

Label Elements

______________________________________________________________________________________________

Page 1

Page 2

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 2 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

Signal Word                    Danger

Hazard Statements

H225 - Highly flammable liquid and vapor

H304 - May be fatal if swallowed and enters airways

H315 - Causes skin irritation

H319 - Causes serious eye irritation

H336 - May cause drowsiness or dizziness

H401 - Toxic to aquatic life

H412 - Harmful to aquatic life with long lasting effects

H373 - May cause damage to organs through prolonged or repeated exposure

H361 - Suspected of damaging fertility or the unborn child

Precautionary Statements

Prevention

P201 - Obtain special instructions before use

P202 - Do not handle until all safety precautions have been read and understood

P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking

P240 - Ground and bond container and receiving equipment

P242 - Use non-sparking tools

P243 - Take action to prevent static discharges

P260 - Do not breathe dust/fume/gas/mist/vapors/spray

P264 - Wash face, hands and any exposed skin thoroughly after handling

P271 - Use only outdoors or in a well-ventilated area

P280 - Wear eye protection/ face protection

Response

P301 + P310 - IF SWALLOWED: Immediately call a POISON CENTER or doctor

P303 + P361 + P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

P308 + P313 - IF exposed or concerned: Get medical advice/attention

P331 - Do NOT induce vomiting

P332 + P313 - If skin irritation occurs: Get medical advice/attention

P370 + P378 - In case of fire: Use dry sand, dry chemical or alcohol-resistant foam to extinguish

P362 + P364 - Take off contaminated clothing and wash it before reuse

Storage

P403 + P233 - Store in a well-ventilated place. Keep container tightly closed

Disposal

P501 - Dispose of contents/ container to an approved waste disposal plant

Physical and Chemical Hazards

Vapors may cause flash fire or explosion. Highly flammable.

Health Hazards

Aspiration hazard if swallowed - can enter lungs and cause damage. Causes skin irritation. Causes serious eye irritation.

Suspected of damaging fertility or the unborn child. May cause drowsiness or dizziness. May cause damage to organs through

prolonged or repeated exposure.

Environmental hazards

Toxic to aquatic life. Harmful to aquatic life with long lasting effects. Is not likely mobile in the environment due its low water

solubility. Spillage unlikely to penetrate soil.

Toxic to terrestrial vertebrates. This product does not contain any known or suspected endocrine disruptors.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

______________________________________________________________________________________________

Page 2

Page 3

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 3 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

Component                              CAS No                Weight %

Toluene                                             108-88-3                    48.75

Isopropyl alcohol                                          67-63-0                    48.75

Neodymium(III) isopropoxide                                  19236-15-8                    2.50

SECTION 4. FIRST AID MEASURES

General Advice

If symptoms persist, call a physician.

Eye Contact

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Get medical attention.

Skin Contact

Wash off immediately with plenty of water for at least 15 minutes. If skin irritation persists, call a physician.

Inhalation

Remove to fresh air. If not breathing, give artificial respiration. Get medical attention if symptoms occur. Risk of serious damage to

the lungs (by aspiration).

Ingestion

Clean mouth with water and drink afterwards plenty of water. Do NOT induce vomiting. Call a physician or poison control center

immediately. If vomiting occurs naturally, have victim lean forward.

Most important symptoms and effects

Difficulty in breathing. Inhalation of high vapor concentrations may cause symptoms like headache, dizziness, tiredness, nausea

and vomiting

Self-Protection of the First Aider

Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and prevent spread of

contamination.

Notes to Physician

Treat symptomatically. Symptoms may be delayed.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Water mist may be used to cool closed containers.

Extinguishing media which must not be used for safety reasons

No information available.

Specific Hazards Arising from the Chemical

Flammable. Containers may explode when heated. Vapors may form explosive mixtures with air. Vapors may travel to source of

ignition and flash back.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Ensure adequate ventilation. Use personal protective equipment as required. Remove all sources of ignition. Take precautionary

measures against static discharges.

______________________________________________________________________________________________

Page 3

Page 4

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 4 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

Environmental Precautions

Do not flush into surface water or sanitary sewer system. Should not be released into the environment. Do not allow material to

contaminate ground water system.

Methods for Containment and Clean Up

Soak up with inert absorbent material. Keep in suitable, closed containers for disposal. Remove all sources of ignition. Use

spark-proof tools and explosion-proof equipment.

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

Wear personal protective equipment/face protection. Ensure adequate ventilation. Do not get in eyes, on skin, or on clothing. Avoid

ingestion and inhalation. Keep away from open flames, hot surfaces and sources of ignition. Use only non-sparking tools. To avoid

ignition of vapors by static electricity discharge, all metal parts of the equipment must be grounded. Take precautionary measures

against static discharges.

Storage

Keep container tightly closed in a dry and well-ventilated place. Keep away from heat, sparks and flame.

Specific Use(s)

Use in laboratories

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Component                China              ACGIH TLV           OSHA PEL              NIOSH

Toluene            TWA: 50 mg/m3          TWA: 20 ppm         (Vacated) TWA: 100 ppm        IDLH: 500 ppm

STEL: 100 mg/m3                                 (Vacated) TWA: 375 mg/m3   REL = 100 ppm (TWA)

Skin                                                   Ceiling: 300 ppm      REL = 375 mg/m3 (TWA)

(Vacated) STEL: 150 ppm       STEL: 150 ppm

(Vacated) STEL: 560 mg/m3     STEL: 560 mg/m3

TWA: 200 ppm

Isopropyl alcohol         TWA: 350 mg/m3          TWA: 200 ppm        (Vacated) TWA: 400 ppm       IDLH: 2000 ppm

STEL: 700 mg/m3          STEL: 400 ppm       (Vacated) TWA: 980 mg/m3   REL = 400 ppm (TWA)

(Vacated) STEL: 500 ppm   REL = 980 mg/m3 (TWA)

(Vacated) STEL: 1225        STEL: 500 ppm

mg/m3             STEL: 1225 mg/m3

TWA: 400 ppm

TWA: 980 mg/m3

Component            Hong Kong                Thailand            European Union       The United Kingdom

Toluene             TWA: 50 ppm               Ceiling: 300 ppm        TWA: 50 ppm (8hr)       STEL: 100 ppm 15 min

TWA: 188 mg/m3          STEL: 500 ppm       TWA: 192 mg/m3 (8hr)     STEL: 384 mg/m3 15 min

TWA: 200 ppm        STEL: 100 ppm (15min)      TWA: 50 ppm 8 hr

STEL: 384 mg/m3 (15min)    TWA: 191 mg/m3 8 hr

Skin                       Skin

Isopropyl alcohol          TWA: 400 ppm          TWA: 400 ppm                                STEL: 500 ppm 15 min

TWA: 983 mg/m3                                                     STEL: 1250 mg/m3 15 min

STEL: 500 ppm                                                  TWA: 400 ppm 8 hr

STEL: 1230 mg/m3                                                TWA: 999 mg/m3 8 hr

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH: NIOSH - National Institute for Occupational Safety and Health

Monitoring methods

BS EN 14042:2003 Title Identifier: Workplace atmospheres. Guide for the application and use of procedures for the assessment of

______________________________________________________________________________________________

Page 4

Page 5

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 5 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

exposure to chemical and biological agents. MDHS70 General methods for sampling airborne gases and vapours MDHS 88

Volatile organic compounds in air. Laboratory method using diffusive samplers, solvent desorption and gas chromatography MDHS

96 Volatile organic compounds in air - Laboratory method using pumped solid sorbent tubes, solvent desorption and gas

chromatography MDHS 91 Metals and metalloids in workplace air by X-ray fluorescence spectrometry MDHS 99 Metals in air by

ICP-AES

Exposure Controls

Engineering Measures

Ensure that eyewash stations and safety showers are close to the workstation location. Ensure adequate ventilation, especially in

confined areas. Use explosion-proof electrical/ventilating/lighting equipment. Wherever possible, engineering control measures

such as the isolation or enclosure of the process, the introduction of process or equipment changes to minimise release or contact,

and the use of properly designed ventilation systems, should be adopted to control hazardous materials at source.

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Nitrile rubber      See manufacturers               -          EN 374             (minimum requirement)

Viton (R)        recommendations

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use    Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: Organic gases and vapours filter low boiling organic solvent

Type AX Brown conforming to EN371 or Type A Brown conforming to EN14387

Small scale/Laboratory use     Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

limits are exceeded or if irritation or other symptoms are experienced.

Recommended half mask:- Valve filtering: EN405; or; Half mask: EN140; plus filter, EN

141

When RPE is used a face piece Fit Test should be conducted

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls   Prevent product from entering drains. Do not allow material to contaminate ground water

system.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance                       Blue

Physical State                        Liquid

Odor                       No information available

Odor Threshold               No data available

pH                         No information available

______________________________________________________________________________________________

Page 5

Page 6

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 6 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

Melting Point/Range            No data available

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Evaporation Rate              No data available

Flammability (solid,gas)            Not applicable                         Liquid

Explosion Limits               No data available

Vapor Pressure                  23 hPa @ 20 °C

Vapor Density                 No data available                         (Air = 1.0)

Specific Gravity  / Density        No data available

Bulk Density                      Not applicable                         Liquid

Water Solubility                     Immiscible

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Component                        log Pow

Toluene                            2.73

Isopropyl alcohol                    0.05

Autoignition Temperature        No data available

Decomposition Temperature      No data available

Viscosity                    No data available

Explosive Properties                                               Vapors may form explosive mixtures with air

Oxidizing Properties            No information available

Molecular Formula             C9 H21 NdO3

Molecular Weight                  321.30

SECTION 10. STABILITY AND REACTIVITY

Stability                            Moisture sensitive.

Hazardous Reactions            None under normal processing.

Hazardous Polymerization        No information available.

Conditions to Avoid              Keep away from open flames, hot surfaces and sources of ignition.

Materials to avoid              No information available.

Hazardous Decomposition Products Carbon monoxide (CO). Carbon dioxide (CO2). Neodymium oxide.

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

(a) acute toxicity;

Toxicology data for the components

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Toluene                  > 5000 mg/kg  ( Rat )      LD50 = 12000 mg/kg  ( Rabbit )     26700 ppm  ( Rat ) 1 h

Isopropyl alcohol                 5045 mg/kg  ( Rat )           12800 mg/kg  ( Rat )            72.6 mg/L  ( Rat ) 4 h

3600 mg/kg  ( Mouse )

(b) skin corrosion/irritation;        Category 2

(c) serious eye damage/irritation;   Category 2

______________________________________________________________________________________________

Page 6

Page 7

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 7 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;             No data available

There are no known carcinogenic chemicals in this product

(g) reproductive toxicity;           Category 2

(h) STOT-single exposure;          Category 3

Results / Target organs          Central nervous system (CNS)

(i) STOT-repeated exposure;        Category 2

Target Organs                  Neuropsychological effects, Eyes, Ears.

(j) aspiration hazard;               Category 1

Symptoms  / effects,both acute and Inhalation of high vapor concentrations may cause symptoms like headache, dizziness,

delayed                                tiredness, nausea and vomiting

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects               The product contains following substances which are hazardous for the environment.

Contains a substance which is:. Toxic to aquatic organisms. May cause long-term adverse

effects in the environment. Do not allow material to contaminate ground water system.

Component                Freshwater Fish         Water Flea        Freshwater Algae         Microtox

Toluene                 50-70 mg/L LC50 96 h EC50: = 11.5 mg/L, 48h EC50: = 12.5 mg/L, 72h  EC50 = 19.7 mg/L 30

5-7 mg/L LC50 96 h      (Daphnia magna)                static                 min

15-19 mg/L LC50 96 h EC50: 5.46 - 9.83 mg/L,  (Pseudokirchneriella

28 mg/L LC50 96 h    48h Static (Daphnia         subcapitata)

12 mg/L LC50 96 h         magna)        EC50: > 433 mg/L, 96h

(Pseudokirchneriella

subcapitata)

Isopropyl alcohol            LC50: = 9640 mg/L, 96h 13299 mg/L EC50 = 48 EC50: > 1000 mg/L, 72h  = 35390 mg/L EC50

flow-through              h             (Desmodesmus        Photobacterium

(Pimephales promelas) 9714 mg/L EC50 = 24 h      subspicatus)       phosphoreum 5 min

LC50: > 1400000 µg/L,                     EC50: > 1000 mg/L, 96h

96h (Lepomis                           (Desmodesmus

macrochirus)                                   subspicatus)

LC50: = 11130 mg/L,

96h static (Pimephales

promelas)

LC50: = 10000000 µg/L,

96h (Daphnia)

Persistence and Degradability      Product contains heavy metals. Discharge into the environment must be avoided. Special

pre-treatment is necessary

Persistence                     Immiscible with water, May persist.

Component                                                       Degradability

Toluene                                           86% (20d)

108-88-3 ( 48.75 )

______________________________________________________________________________________________

Page 7

Page 8

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 8 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

Degradation in sewage          Contains substances known to be hazardous to the environment or not degradable in waste

treatment plant                 water treatment plants.

Bioaccumulative Potential        May have some potential to bioaccumulate; Product has a high potential to bioconcentrate

Component                                 log Pow                           Bioconcentration factor (BCF)

Toluene                                      2.73                                      90

Isopropyl alcohol                                  0.05                            No data available

Mobility in soil                        Spillage unlikely to penetrate soil Is not likely mobile in the environment due its low water

solubility

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Waste is classified as hazardous. Dispose of in accordance with the European Directives

Products                        on waste and hazardous waste. Dispose of in accordance with local regulations.

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point. Empty containers

retain product residue, (liquid and/or vapor), and can be dangerous. Keep product and

empty container away from heat and sources of ignition.

Other Information              Do not flush to sewer. Waste codes should be assigned by the user based on the

application for which the product was used. Can be landfilled or incinerated, when in

compliance with local regulations.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport

UN-No                       UN1993

Proper Shipping Name            Flammable liquid, n.o.s.

Technical Shipping Name      (TOLUENE, ISOPROPANOL (ISOPROPYL ALCOHOL))

Hazard Class                    3

Packing Group                                     II

IMDG/IMO

UN-No                       UN1993

Proper Shipping Name            Flammable liquid, n.o.s.

Technical Shipping Name      (TOLUENE, ISOPROPANOL (ISOPROPYL ALCOHOL))

Hazard Class                    3

Packing Group                                     II

IATA

UN-No                       UN1993

Proper Shipping Name            Flammable liquid, n.o.s.

Technical Shipping Name      (TOLUENE, ISOPROPANOL (ISOPROPYL ALCOHOL))

Hazard Class                    3

Packing Group                                     II

Special Precautions for User      No special precautions required

______________________________________________________________________________________________

Page 8

Page 9

ALFAA36573               SAFETY DATA SHEET                    Revision Date 05-Aug-2026Page 9 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

SECTION 15. REGULATORY INFORMATION

International Inventories

X = listed, China (IECSC), Europe (EINECS/ELINCS/NLP), U.S.A. (TSCA), Canada (DSL/NDSL), Philippines (PICCS), Japan (ENCS), Japan

(ISHL), Australia (AICS), Korea (KECL).

Component        The        List of    IECSC   EINECS   TSCA   DSL   PICCS  ENCS    ISHL    AICS    KECL

Inventory of dangerous

Hazardous  goods GB

Chemicals   12268 -

(2015      2012

Edition)

Toluene         X        X       X      203-625-9    X      X      X      X      X      X     KE-33936

Isopropyl alcohol      X        X       X      200-661-7    X      X      X      X      X      X     KE-29363

National Regulations

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

Revision Date                     05-Aug-2026

Revision Summary                Not applicable.

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Use of personal protective equipment, covering appropriate selection, compatibility, breakthrough thresholds, care, maintenance, fit

and standards.

First aid for chemical exposure, including the use of eye wash and safety showers.

Fire prevention and fighting, identifying hazards and risks, static electricity, explosive atmospheres posed by vapours and dusts.

Chemical incident response training.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

______________________________________________________________________________________________

Page 9

Page 10

ALFAA36573               SAFETY DATA SHEET                    Revision Date Page05-Aug-202610 / 10

Neodymium(III) isopropoxide 2.5% w/v in toluene/isopropanol

______________________________________________________________________________________________

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Physical hazards              On basis of test data

Health Hazards                       Calculation method

Environmental hazards              Calculation method

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of Safety Data Sheet

______________________________________________________________________________________________

Page 10