Neodymium (III) Nitrate Hydrate 2646 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/neodymium-iii-nitrate-hydrate-sds-prochem-en-498d5747bb

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Neodymium (III) Nitrate Hydrate

PRODUCT NUMBER:  2646

CAS NUMBER:        13746-96-8

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H272             May intensify fire; oxidizer.

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P210              Keep away from heat.

P220                 Keep/Store away from clothing/combustible materials.

P221               Take any precaution to avoid mixing with combustibles.

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction.

HMIS CLASSIFICATION:

Health: 0            Chronic: *               Fire: 0                     Reactivity Hazard: 1

NFPA RATING:

Health: 0            Flammability: 0         Reactivity Hazard: 1       Special: OX

NEODYMIUM (III) NITRATE HYDRATE                                                         PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes severe irritation of the eyes.

SKIN CONTACT: Causes skin irritation.

INHALATION: May cause respiratory irritation.

INGESTION: No data available.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Nd(NO3) XH2O

Molecular Weight: 330.25 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Neodymium (III) Nitrate Hydrate                                           13746-96-8                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with soap and plenty of water. Remove contaminated clothes if necessary. Seek

medical assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Contact with combustible material may cause fire.

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistant foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots. This substance is an oxidizer and its heat of reaction with reducing agents or combustibles

may cause ignition.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Neodymium oxides and nitrogen oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation. Avoid breathing dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Absorb spill with inert materials. Sweep up and place in

suitable container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Avoid contact

with skin and eyes.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from flammable substances, reducing

agents, and metal powders.

NEODYMIUM (III) NITRATE HYDRATE                                                         PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Purple solid

ODOR: Nitric acid

MOLECULAR WEIGHT: 330.25 (anhy.)

BOILING POINT: No data available

MELTING POINT: 40° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Excessive temperatures

INCOMPATIBILITY: Reducing agents, bases, flammable substances, organic materials, and metal powders.

DECOMPOSITION PRODUCTS: Neodymium oxides and nitrogen oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat – 2072 mg/kg

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is not considered a human

carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:   UN1465                                                Marine Pollutant: No

Didymium Nitrate

CLASS 5.1

PG III

NEODYMIUM (III) NITRATE HYDRATE                                                         PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

SARA 302/304: Not Listed

SARA 311/312: Not Listed

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Neodymium (III) Nitrate Hydrate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Neodymium (III) Nitrate Hydrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 03/15

SDS DEPT.

NEODYMIUM (III) NITRATE HYDRATE                                                         PAGE 4